| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | RS_Config-CMD message format (Section 16.6.2.14 of the IEEE 802.16m/D4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation | E-mail: alexander.maltsev@intel.com |
| Re: | Category: LB31 / Area: Chapter 16.6.2.14 / Topic: Relay |  |
| Abstract | This contribution proposes the format of the RS_Config-CMD message used to configure the PHY layer operational parameters for relay stations support |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

RS_Config-CMD message format 
(Section 16.6.2.14 of the IEEE 802.16m/D4)

Alexander Maltsev, Alexey Khoryaev, 
Andrey Chervyakov, Mikhail Shilov

Intel Corporation

- Introduction

This contribution proposes the format of the RS_Config-CMD message which is used to configure the PHY layer operational parameters for relay stations support. The proposed text is developed so that it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

Current amendment text [1] does not specify the format of the RS_Config-CMD message which shall be used by the ABS to configure the ARS PHY layer operational parameters. In this contribution the format of this message is proposed. The following general parameters are included in this message:

- The information on the duration of the 16m DL and UL Relay zones,

- The information on whether the ABS allocates resources to the AMSs in the 16m DL and UL Relay zones;

- The information on whether the ARS shall send the enhanced MIMO feedback for CL SU-MIMO and MU-MIMO modes;

- The duration of R-TTI and R-RTI transition intervals for both FDD and TDD modes;

- Permutation parameters required to support cell-specific resource mapping in the 16m DL and UL Relay zones in case when the ABS does not allocate transmissions to the AMSs in these zones.

- References

- IEEE P802.16m/D4, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the P802.16m/D5

Instruction to editor:

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

============================= Start of the text #1==============================

[Note to Editor – Insert subsection 16.6.2.14.3 RS_Config-CMD message format into section 16.6.2.14 ARS Configuration in page 727 as follows]

16.6.2.14.3 RS_Config-CMD message format

Table xx– RS_Config-CMD message format

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | 16m_DL_Relay_zone_duration |  | 3 | The duration of the 16m DL Relay zone in units of AAI subframes. It has 3 bits to represent the values {1, 2, 3, 4, 5. 6, 7, 8} | N/A |
| M | 16m_UL_Relay_zone_duration |  | 3 | The duration of the 16m UL Relay zone in units of AAI subframes. It has 3 bits to represent the values {1, 2, 3, 4, 5. 6, 7, 8} | N/A |
| M | 16m_Relay_zone_AMS_allocation_indicator |  | 1 | The indicator which signalizes if the ABS may allocate resources to the AMS in the 16m DL Relay zone and 16m UL Relay zone. It has 1 bit to represent the values {0, 1}. If it is set to 1 then the AMSs are allocated in Relay zones | N/A |
| M | 16m_Relay_Enhanced_MIMO |  | 1 | The indicator which signalizes if the ARS shall provide enhanced MIMO feedback to the ABS. It has 1 bit to represent the values {0, 1}. If it is set to one then ARS shall report Enhanced MIMO Feedback | N/A |
| O | 16m_Relay_FDD_TTI_Parameters | FDD_DL_R_TTI | 1 | The duration of the R-TTI in the 16m DL Relay zone of FDD ARS frame in units of OFDMA symbols. It has1 bit to represent the values {0, 1}. If it is set to 1 then R-TTI has a duration equal to one OFDMA symbol. In opposite case the R-TTI duration is set to zero. | Present if system operates in FDD mode |
| O |  | FDD_DL_R_RTI | 1 | The duration of the R-RTI in the 16m DL Relay zone of FDD ARS frame in units of OFDMA symbols. It has1 bit to represent the values {0, 1}, If it is set to 1 then R-RTI has a duration equal to one OFDMA symbol. In opposite case the R-RTI duration is set to zero. | Present if system operates in FDD mode |
| O |  | FDD_UL_R_TTI | 1 | The duration of the R-TTI in the 16m UL Relay zone of FDD ARS frame in units of OFDMA symbols. It has1 bit to represent the values {0, 1} If it is set to 1 then R-TTI has a duration equal to one OFDMA symbol. In opposite case the R-TTI duration is set to zero. | Present if system operates in FDD mode |
| O |  | FDD_UL_R_RTI | 1 | The duration of the R-RTI in the 16m UL Relay zone of FDD ARS frame in units of OFDMA symbols. It has1 bit to represent the values {0, 1}. If it is set to 1 then R-RTI has a duration equal to one OFDMA symbol. In opposite case the R-RTI duration is set to zero. | Present if system operates in FDD mode |
| O | 16m_Relay_TDD_TTI_Parameters | TDD_DL_R_TTI | 1 | The duration of the R-TTI in the 16m DL Relay zone of TDD ARS frame in units of OFDMA symbols. It has1 bit to represent the values {0, 1}. If it is set to 1 then R-TTI has a duration equal to one OFDMA symbol. In opposite case the R-TTI duration is set to zero. | Present if system operates in TDD mode |
| O |  | TDD_UL_R_RTI | 1 | The duration of the R-RTI in the 16m UL Relay zone of TDD ARS frame in units of OFDMA symbols. It has1 bits to represent the values {0, 1}. If it is set to 1 then R-TTI has a duration equal to one OFDMA symbol. In opposite case the R-TTI duration is set to zero. | Present if system operates in TDD mode |
| O | 16m_DL_Relay_zone_Cell_Specific_Parameters | R_DCASSB0 | 5/4/3 | See 16.6.3.3.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O |  | R_DCASMB0 | 5/4/3 | See 16.6.3.3.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O |  | R_DCASi | 3/2/1 | See 16.6.3.3.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O | 16m_UL_Relay_zone_Cell_Specific_Parameters | R_UCASSB0 | 5/4/3 | See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O |  | R_UCASMB0 | 5/4/3 | See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O |  | R_UCASi | 3/2/1 | See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |

============================= End of the text #1==============================