| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of resource allocation using Basic Assignment A-MAP IE (16.3.6.5.2.4.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yaniv Weizman Alvarion | yaniv.weizman@alvarion.com |
| Re: |  |  |
| Abstract | Clarifying the LRU allocation of different CRU/DRU using Basic Assignment A-MAP IE |  |
| Purpose | To be discussed and adopted by TGm into P802.16m/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of resource allocation using Basic Assignment A-MAP IE (16.3.6.5.2.4.1)

Yaniv Weizman

Alvarion

Problem Statement

The current text is not clear about allocating different LRU types (DLRU/NLRU/SLRU) within same allocation using Basic Assignment A-MAP IE. In addition, it is not clear if resource allocation of continuous LRUs can span over multiple frequency partitions within subframe.

Suggested Solution

Clarifying that Basic Assignment A-MAP IE can allocate continuous LRUs mapped to same LRU type (DLRU/NLRU/SLRU).

Clarify that assigned continuous LRUs allocation cannot span over multiple frequency partitions within subframe.

Suggested Standard Modifications

Modify text in page 464, line 59 in section 16.3.6.5.2.4.1, as follows:

The Resource Index in the DL/UL Basic Assignment A-MAP IE identifies a single allocation of a resource comprising a set of contiguous LRUs in a DL/UL AAI subframe. The index determines the size (S) of the allocated resource in number of contiguous LRUs in frequency within the subframe and the index (L) of the LRU from where the scheduled allocation begins. The contiguous LRUs shall be constructed from same LRU type (i.e DLRU, s or NLRUs or SLRUs).  Continuous LRUs resources shall be allocated from same frequency partition and cannot span over multiple frequency partitions. Multiple non-contiguous subbands are indexed using the DL/UL Subband Assignment A-MAP IEs.