| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for multiple PA and/or GRA allocations per AMS (16.3.3.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yaniv Weizman Alvarion Amir Rubin Intel Corporation | yaniv.weizman@alvarion.com Amir.rubin@intel.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | We propose to define the concurrent number of PA and/or GRA allocations according to AMS capabilities. |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for multiple PA and/or GRA allocations per AMS (16.3.3.1)

Yaniv Weizman
Alvarion

Problem Statement

Currently there is a limitation of AMS to have up to 2 concurrent PA and/or GRA allocations processes. We propose to extend the limitation to be per AMS capabilities in order to manage (gateway like) AMSs, having multiple active flows (for example as VoIP gateway) via PA and/or GRA.

Suggested Solution

Define the number of concurrent allocations via PA and/or GRA to be defined by AMS capabilities.

Suggested Standard Modifications

Modify text in page 348, line 1 in section 16.3.3.1, as follows:

The total number of concurrent DL persistent allocations and/or DL group resource allocations processes supported by AMS is determined by MAX_DL_GRA_PA_PROCESSES parameter within AAI_REG-REQ and AAI_REG-RSP. The ABS shall not allocate to an AMS more than its supported capabilities. DL Persistent allocation process is defined as active DL persistent allocation of AMS flow. DL Group Resource Allocation is defined as inclusion of AMS flow within a group. The default number of concurrent DL processes is 2.  2 concurrent DL allocations that are either group resource allocations or persistent allocations AMS capabilities. In other words the ABS may allocate either 2 DL group resource allocations, 2 DL persistent allocations, 1 DL group resource allocation and 1 DL persistent allocation, or 1 DL allocation of these types to any specific AMS.

The total number of concurrent UL persistent allocations and/or UL group resource allocations processes supported by AMS is determined by MAX_UL_GRA_PA_PROCESSES parameter within AAI_REG-REQ and AAI_REG-RSP. The ABS shall not allocate to an AMS processes more than its supported capabilities. UL Persistent allocation process is defined as active UL persistent allocation of AMS flow. UL Group Resource Allocation is defined as inclusion of AMS flow within a group. The default number of concurrent UL processes is 2.   2 concurrent UL allocations that are either group resource allocations or persistent allocations. In other words the ABS may allocate either 2 UL group resource allocations, 2 UL persistent allocations, 1 UL group resource allocation and 1 UL persistent allocation, or 1 UL allocation of these types to any specific AMS.

Add the following rows to table 682 in section 16.2.3.7, page 75 after line 14:

| Name |  | Value |
| --- | --- | --- |
| AMS capability negotiation parameters | MAX_DL_GRA_PA_PROCESSES | Maximum concurrent DL GRA & PA processes |
|  | MAX_UL_GRA_PA_PROCESSES | Maximum concurrent DL GRA & PA processes |

Add the following rows to table 683 in section 16.2.3.8, page 78 after line 64:

| Name |  | Value |
| --- | --- | --- |
| AMS capability negotiation parameters | MAX_DL_GRA_PA_PROCESSES | Maximum concurrent DL GRA & PA processes |
|  | MAX_UL_GRA_PA_PROCESSES | Maximum concurrent DL GRA & PA processes |