| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Sub-band group resource allocation mechanism (16.2.9) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yaniv Weizman, Ariel Doubchak, Erez Biton Alvarion. | Email : erez.biton@alvaion.com |
| Re: | Comments on P802.16m/D3 |  |
| Abstract | The contribution proposes the addition of Subband group resource allocation mechanism to allow reduced maps overhead. |  |
| Purpose | To be discussed and adopted into P802.16m/D5 by WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Sub-band group resource allocation mechanism (16.2.9)

Yaniv Weizman, Ariel Doubchak, Erez Biton

Alvarion.

# Problem Statement

Group allocation is known as efficient way to allocate resource to users with similar traffic characteristics. However, currently, group resource allocation does not support sub-band allocations. Currently, multiple subband allocations for AMSs within the same subframe enforce the use of multiple subband resource allocation IE, which lead to significant maps overhead.

Suggested Solution

In the following, based on the methods defined for GRA and subband allocations, we propose a method for sub-band group resource allocation.

Suggested Standard Modifications

Modify text in page 247, line 34 in section 16.2.9, as follows:

The User and MIMO bitmaps are defined exactly as for GRA.

The third bitmap is the Resource Allocation bitmap which uses 2 bits per AMS to signal HARQ burst size as within GRA and 11 bits per AMS to signal sub-bands allocated the for the scheduled AMS in the AAI sub-frame or extended AAI sub-frame that are scheduled in the frame. The sub-band bitmap interpretation should be similar to that defined for the Sub-band Resource Allocation IE for the Resource Index field (BW and number of sub-band defined within sub-frame).

Figure 1 illustrates the proposed bitmaps.

Figure 1: sub-band group resource allocation – option 1

Modify text in page 511, line 15 in section 16.3.6.5.2.4.10, as follows:

The following definition for Subband Group Resource Allocation IEs (DL/UL):

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL/UL Group Resource Allocation IE(){ | - |  |
| A-MAP IE Type | 4 | DL/UL Subband Group Resource Allocation A-MAP IE |
| HFA offset | 6 | Indicates the start of the HARQ feedback channel index used for scheduled allocations. |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size signaled to each AMS in the Group configuration AMC control message. 0b0: AMS not allocated in this AAI subframe 0b1: AMS allocated in this AAI subframe |
| If (Group MIMO mode set ==0b01){ |  |  |
| MIMO bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. 0b0: Mode 0 0b1: Mode 1 |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate subband allocations for each scheduled AMS |
| } | - | - |