| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of FFR Configuration Selection for the Reuse-2 Case (Section 16.2.20.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Bill Hillery, Fred Vook, Mark Cudak Motorola Inc. | bill.hillery@motorola.com |
| Re: | Category: P802.16m/D4 comments for LB31 Area: Sections 16.2.20.1 (DL FFR) |  |
| Abstract | The equation used to determine the DL FFR configuration for the reuse-2 case allows different SA-Preamble and MIMO Midamble subcarrier sets to be used within a single FFR partition when the Segment ID is 1. This contributions proposes a method for choosing the FFR configuration which fixes this problem. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of FFR Configuration Selection for the Reuse-2 Case (Section 16.2.20.1)

Bill Hillery, Fred Vook, Mark Cudak, Eugene Visotsky, Anup Talukdar

Motorola, Inc.

- Introduction

The current equation used to set the FFR configuration (k) for the case when FPCT=3 and FPS3=0 or when FPCT=2 is:

k = floor(IDcell/384) + 1

With this equation, ABS’s with an IDcell from segment 1 (256 <= IDcell < 512) may be assigned to either k=1 or k=2.  Therefore, some ABS’s in segment 1 will use FFR configuration 1 and some will use FFR configuration 2 in Figure 459.  If a CINR measurement is made using either the SA-Preamble or MIMO Midamble for the boosted FFR partition, it may be inaccurate because some base stations which will not interfere on the FFR partition do interfere in the SA-Preamble or MIMO Midamble.  This problem can be avoided by restricting the range of used IDcell values in segment 1.  However, a better solution is to modify the equation which determines the FFR configuration to avoid the problem altogether.

This contribution proposes the use of the same equation used when FPCT=4 or FPCT=3 with FP3>0 with the restriction that only segments 0 and 1 are used when FPCT=3 and FPS3=0 or when FPCT=2.

- Proposed Text Changes

[Modify the text beginning on line 32 of page 329 as follows:]

When FPCT=3 and FPS3=0, for example, each ABS chooses one of the two FFR patterns (cell 1 and cell 2 pattern) as shown in Figure 459  (when FPCT=2, the same FFR patterns exists only excluding FP0).  The index of FFR power pattern is set by a particular ABS with its cellsegment ID, which is restricted in these cases to be either 0 or 1.  That is, each ABS adopts the FFR power pattern corresponding to the Cell k in Figure 454 and k is decided by the following equation:

k = floor(IDcell/384) + 1 SegmentID + 1