| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Relay Frame Structure Changes and Text Cleanup (Section 16.6.3 of the IEEE 802.16m/D4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation | E-mail: alexander.maltsev@intel.com |
| Re: | Category: LB31 / Area: Chapter 16.6.3 / Topic: Relay |  |
| Abstract | Proposed Text on Relay Frame Structure Changes and Text Cleanup |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Relay Frame Structure Changes and Text Cleanup
(Section 16.6.3 of the IEEE 802.16m/D4)

Alexander Maltsev, Alexey Khoryaev, 
Andrey Chervyakov, Mikhail Shilov

Intel Corporation

- Introduction

This contribution proposes technical and editorial changes and a number of clarifications to the current amendment text describing frame structure supporting ARS operation and to be included in the 802.16m amendment draft [1]. The proposed text is developed so that it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

The technical changes described in this contribution reflect the change in MIMO-Midamble position that was moved from the second last subframe to the first symbol of the second subframe of the frame. The shift of MIMO-Midamble position requires slight modification of ARS frame structure to enable proper ARS operation in the majority of the existing frame configurations and preserve the flexibility in the configuration of the 16m DL Relay zone duration. To achieve these goals the following changes were made in this contribution:

- R-TTI gap is moved from 16m DL Access zone and placed inside the 16m DL Relay zone in order to increase the number of frame configurations that support Relay operation and decrease the impact on AMS stations for the case when Relays are deployed

- The restrictions on the durations of the 16m Access and Relay zones are introduced and the frame configurations that shall not be used with ARSs are described.

- The figures are updated to reflect the change in R-TTI position.

- The special DL data mapping on relay links is defined for the case when the duration of transition intervals are equal to the OFDMA symbol duration. In this case, the ARS is not able to receive the first/last symbol of the AAI subframe that is transmitted from ABS to ARS in the first/last subframe of the 16m Relay zone. In this case in order to avoid degradation of the Relay link throughput it is proposed to define the specific mapping of data subcarriers.

- In addition it is specified that the duration of Relay zones are broadcasted to all AMSs in AAI_System Configuration Descriptor message

- References

- IEEE P802.16m/D4, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the P802.16m/D5

Instruction to editor:

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

============================= Start of the text #1==============================

[Note to Editor – Modify the text in section 16.6.3.1 Basic frame structure supporting ARS in page 731 as follows]

16.6.3.1 Basic frame structure supporting ARS

The advanced air interface supports two hop data transmission between an ABS and an AMS using an intermediate ARS. Figure 602 shows an example of the basic frame structure for system supporting ARSs. When an ARS is deployed it shall use the same OFDMA signal parameters (defined in Table 775) as its serving ABS. The ABS and ARS superframes shall be time aligned and shall consist of the same number of frames and AAI subframes. Every ARS superframe shall contain a superframe header (SFH). The SFH transmitted by the ARS shall have the same location and format as the SFH transmitted by the ABS. The ARS preambles (SA-Preamble and PA-Preamble) shall be transmitted synchronously with superordinate ABS preambles.

When ARSs are supported the ABS frame is divided on 16m Access zone and 16m Relay zone. The 16m Access zone position precedes the 16m Relay zone position inside the frame. The duration of the 16m Access zone and 16m Relay zone may be different in DL and UL directions. The zone configuration of 16m Access zone and 16m Relay zone is informed to the ARS by the ABS in RS_config-CMD message.

The ABS frame 16m Access zone shall consist of 16m DL Access zone and 16m UL Access zone, and 16m Relay zone shall consist of 16m DL Relay zone and 16m UL Relay zone. The ABS frame 16m Access zone shall be used for communication with AMSs only. The ABS frame 16m Relay zone shall be used for communication with ARSs and may be used for communication with AMSs. In the 16m DL Relay zone the ABS shall transmit to its subordinate ARS and in the 16m UL Relay zone the ABS shall receive transmissions from its subordinate ARS.

The ARS frame 16m Access zone shall consist of 16m DL Access zone and 16m UL Access zone and 16m Relay zone shall consist of 16m DL Relay zone and 16m UL Relay zone. The ARS frame 16m Access Zone shall be used for communication with AMSs only. In the 16m DL Relay zone the ARS shall receive transmissions from its superordinate ABS and in the 16m UL Relay zone the ARS shall transmit to its superordinate ABS.

The position durations of the 16m DL and UL Rrelay zones is indicated for AMS using AAI_System Configuration Descriptor message when 16m_Relay_zone_AMS_allocation_indicator is equal to 0.

During the configuration of the 16m DL/UL Relay zones the ABS shall apply the following restrictions on the frame configurations for both TDD and FDD modes:

- In order to enable MIMO Midamble transmissions from ABSs and ARSs to the attached AMSs, the minimum duration of the 16m DL Access zone shall be limited to 2 AAI subframes. The minimum duration of the 16m DL Relay zone shall be limited to 1 AAI subframe. The frame configurations defined in 16.3.3.7 which have less than 3 DL AAI subframes shall not be used in cells with deployed ARSs.

- In order to enable MIMO Midamble transmissions from ABSs to the attached ARSs, the 16m DL Relay zone shall be configured in order to have at least 1 non type-3 AAI subframe.

- The minimum duration of the 16m UL Relay zone shall be 1 AAI subframe. The frame configurations defined in 16.3.3.7 that have one UL AAI subframe shall not be used for the ARS operation.

In each ARS frame, the relay transmit to receive transition interval (R-TTI) may be inserted in order to make allowances for ARSTTG and RTD between the ARSs and their its superordinate station.

In each ARS frame, the relay transmit to receive transition interval (R-RTI) may be inserted in order to make allowances for ARSRTG and RTD between the ARSs and their its superordinate station.

The duration and the positions of R-TTI and R-RTI are defined in FDD (see 16.6.3.2.1) and TDD (see 16.6.3.2.2), respectively. The each R-TTI and R-RTI shall be equal or less than one OFDMA symbol.

When multiple ARSs are deployed in the same sector of the ABS, the same values of R-TTI and R-RTI shall be applied for all ARSs. The ABS shall select the maximum R-TTI and R-RTI values among all attached ARSs and shall apply these values for frame configuration. The ABS shall report R-TTI and R-RTI values to the attached ARSs using the RS_config-CMD message.

============================= End of the text #1==============================

============================= Start of the text #2==============================

[Note to Editor – Modify the text in section 16.6.3.2.1 FDD frame structure in page 733 as follows]

16.6.3.2.1 FDD frame structure

The FDD frame shall be constructed on the basis of the basic frame structure defined in 156.3.3.

An ARS supporting FDD mode shall communicate with the ABS using full duplex FDD mode and it shall be able to simultaneously support half duplex and full duplex AMSs operating on the same RF carrier. An ARS in FDD systems shall use DL carrier frequency (FDL) for receiving from the ABS in the 16m DL Relay zone and shall use UL carrier frequency (FUL) for transmission to the ABS in the 16m UL Relay zone.

The following additional restrictions apply for FDD frame configurations in case of Relays support:

- In order to enable MIMO Midamble transmissions from ABSs to the attached ARSs, the minimum duration of the 16m DL Relay zones shall be 2 AAI subframes.

In ARS radio frame, the ARS idle state time interval (R_IdleTime) shall be inserted between two ARS frames. The duration of the R_IdleTime is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor. In ARS DL frame, R_IdleTime shall be same as IdleTime of ABS. In ARS UL frame, the duration of R_IdleTime shall be less or equal to the duration of IdleTime of ABS. The duration of the R_IdleTime for the ARS UL frame is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor. The ARS UL frame may be time-advanced for a Tadv Tadv interval referring to the start of the ABS UL frame. The duration of Tadv Tadv is calculated according to the following equation:

Tadv= IdleTime−R_IdleTime 									(315)

When an ARS switches transceiver states from transmit to receive or from receive to transmit, a DL ARS radio frame in FDD system may have a R-TTI between 16m Access and 16m Relay zone and a R-RTI between 16m Relay zone and 16m Access zone in the next DL ARS frame. The location of the R-TTI is the last OFDM symbol of the last AAI subframe of 16m Access zone the first OFDM symbol of the first AAI subframe of the 16m Relay zone and the location of the R-RTI is the last OFDM symbol of the last AAI subframe of the 16m Relay zone. The durations of R-TTI and R-RTI in DL for each ARS shall be calculated by following equations:

R-TTI =0 if RTD2≥ARSTTGTs if RTD2<ARSTTG								(316)

R-RTI = 0 if IdleTime−RTD2≥ARSRTGTs if IdleTime−RTD2<ARSRTG							(317)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table 762>>.

If the R-TTI is present (i.e. its duration is equal to Ts), then the ARS DL Access zone AAI subframe with the R-TTI is formed by type-3 or type-1 AAI subframe when the corresponding ABS DL Access zone AAI subframe is type-1 or type-2 AAI subframe, respectively. If there is no R-TTI (i.e. its duration is equal to zero), the AAI subframes in the DL Access zone at ARS are the same as those at ABS.

When an ARS switches transceiver states from receive to transmit or from transmit to receive, an UL ARS radio frame in FDD system may have a R-RTI between 16m Access and 16m Relay zone and a R-TTI between 16m Relay zone and 16m Access zone in the next UL ARS frame. The location of the R-RTI is the first OFDM symbol of the first AAI subframe of 16m Relay zone and the location of R-TTI is the last OFDM symbol of the last AAI subframe of 16m Relay zone. The duration of R-RTI and R-TTI in UL for each ARS shall be calculated by following equations:

R−RTI=0 if Tadv≥ARSRTG+RTD2Ts if Tadv<ARSRTG+RTD2							(318)

R−TTI=0 if R_IdleTime+RTD2≥ARSTTGTs if R_IdleTime+RTD2<ARSTTG						(319)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table 762>>.

Figure 603— illustrates an example frame structure with ARS support for FDD mode, which is applicable to the nominal channel bandwidth of 5, 10, 20 MHz with G = 1/8. All transition intervals R-TTI and R-RTI in the figure example are equal to the duration of one OFDMA symbol.

============================= End of the text #2==============================

============================= Start of the text #3==============================

[Note to Editor – Replace the Figure 603 in page 735 with the figure provided below]

============================= End of the text #3==============================

============================= Start of the text #4==============================

[Note to Editor – Modify the text in section 16.6.3.2.2 TDD frame structure in page 735 as follows]

16.6.3.2.2 TDD frame structure

The TDD frame shall be constructed on the basis of the basic frame structure defined in 156.3.3.

In ARS radio frame, the ARS idle state time interval (R_IdleTime) shall be inserted before the switching point from DL to UL. The duration of the R_IdleTime is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor. The ARS UL frame may be advanced (Tadv) referring to the start of ABS UL frame. The duration of Tadv Tadv is calculated according to the following equation:

Tadv=TTG−R_IdleTime									(320)

The duration of R_IdleTime shall be less or equal to the TTG. In each ARS radio frame, the RTG interval shall be inserted before the switching point from UL to DL.

An ARS radio frame in TDD system shall have an R-TTI transition interval in DL between the 16m Access zone and the 16m Relay zones. The location of the R-TTI is the first OFDM symbol of the first AAI subframe of the 16m DL Relay zone. The duration of the R-TTI in DL for each ARS shall be calculated by following equation:

R-TTI =0 if RTD2≥ARSTTGTs if RTD2<ARSTTG								(321)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table 762>>.

If the R-TTI is present (i.e. its duration is equal to Ts), then ARS DL Access zone AAI subframe with the R-TTI is formed by type-3 or type-1 AAI subframe when the corresponding ABS DL Access zone AAI subframe is type-1 or type-2 AAI subframe, respectively. If there is no R-TTI (i.e. its duration is equal to zero), the AAI subframes in the DL Access zone at ARS are the same as those at ABS.

An ARS radio frame in TDD system shall have an R-RTI transition interval in UL between 16m Access and 16m Relay zones. The location of the R-TTI is the first OFDM symbol of the first AAI subframe of the 16m UL Relay zone. The duration of the R-RTI in UL for each ARS shall be calculated by the following equation:

R-RTI = 0 if Tadv−RTD2≥ARSRTGTs if Tadv−RTD2<ARSRTG							(322)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table 762>>.

Figure 604— illustrates an example frame structure with ARS support for TDD mode D:U = 5:3, which is applicable to the nominal channel bandwidth of 5, 10, 20 MHz with G = 1/8. The number of AAI subframes allocated to the Relay zone in DL direction is two and in UL direction is one. The duration of R-TTI and R-RTI is equal to the duration of one OFDMA symbol.

============================= End of the text #4==============================

============================= Start of the text #5==============================

[Note to Editor – Replace the Figure 604 in page 736 with the figure provided below]

============================= End of the text #5==============================

============================= Start of the text #6==============================

[Note to Editor – Add new subsection 16.6.3.3.2 Downlink mapping of data subcarriers to the section 16.6.3.3 Relay Downlink PHY structure in page 736]

16.6.3.3.2 Downlink mapping of data subcarriers

The procedure of mapping of data subcarriers described in 16.3.5 shall be applied in the 16m DL Relay zone, in all cases except the following ones when ARS specific mapping of data subcarriers shall be used:

- In the first AAI subframe of the 16m DL Relay zone of TDD and FDD frame structures if the R-TTI duration is equal to one OFDMA symbol duration (Ts);

- In the last AAI subframe of the 16m DL Relay zone of FDD frame structure if the R-RTI duration is equal to one OFDMA symbol duration (Ts);

If the MIMO-Midamble signal on relay link is present in the AAI subframe, then the type of the AAI subframe shall be changed first as described in 16.6.3.3.3 and then the ARS specific mapping of data subcarriers shall be applied in accordance with the description below.

ARS specific mapping of data subcarriers when both AMS and ARS are allocated in 16m DL Relay zone

The ARS specific mapping of data subcarriers shall be applied for data transmission in 16m DL Relay zone when the 16m_Relay_zone_AMS_allocation_indicator is set to 1 (i.e. ABS is able to transmit to AMSs and ARSs in 16m DL Relay zone) and when R-TTI or R-RTI values are equal to Ts. In this case the insertion of R-TTI or R-RTI shall not impact the data transmission from ABS to attached AMSs and the ARS specific mapping of data subcarriers shall be used for data transmission to ARSs.

When R-TTI is equal to Ts in FDD and TDD frame structures, the ARS is not able to receive the first OFDMA symbol of the first subframe of the 16m DL Relay zone. In this case the ABS shall perform mapping the modulated data symbols on subcarriers starting from the second OFDMA symbol of the first subframe of 16m DL Relay zone. The cell specific resource allocation permutation defined in section 16.3.5.3 shall be applied taking into account that data are mapped to LRUs starting from the second OFDMA symbol and that there is a reduced number of tones for data transmission in these LRUs.

When R-RTI is equal to Ts in FDD frame structure, the ARS is not able to receive the last symbol of the last subframe of 16m Relay zone. In this case the ABS shall not transmit data to ARS at the last OFDMA symbol of the last subframe of 16m DL Relay zone. The cell specific resource allocation permutation defined in section 16.3.5.3 shall be applied assuming that data are not mapped to LRU subcarriers that belong to the last symbol of this subframe and that there is a reduced number of tones for data transmission in LRUs allocated for data transmission to ARS.

In both cases the 16m DL Relay zone is shared between the AMS and ARS so the pilot patterns shall not be affected by the ARS specific mapping of data subcarriers and ABS shall allocate pilots in all symbols of the subframe. An example that shows the data transmission from ABS to ARS on relay link using CRU with the one data stream is shown in Figure x1 for the case of TDD and FDD frame structures when R-TTI == Ts (on the left side) and for FDD frame structure when R-RTI == Ts (on the right side).

Figure x1 – Example of DL ARS specific mapping of data subcarriers on relay link
a) TDD/FDD frame structures when R-TTI=Ts, b) FDD frame structure when R-RTI=Ts

ARS specific mapping of data subcarriers when only ARSs are allocated in 16m DL Relay zone

In this case the 16m_Relay_zone_AMS_allocation_indicator shall be set to 0. ARS specific mapping of data subcarriers shall be applied only in the case when MIMO-Midamble and R-TTI are allocated in the same type-1 AAI subframe for TDD or FDD frame strucutures. In this case the type-1 AAI subframe LRUs shall be converted to type-3 AAI subframe LRUs and the ARS specific mapping of data subcarriers shall be applied. The physical structure of pilot patterns shall be the same as in type-3 AAI subframe.

When R-RTI is equal to Ts in FDD frame structure then the ARS specific mapping of data subcarriers shall be applied only if the last subframe of 16m DL Relay zone is type-3 AAI subframe. In opposite case the conversion of subframe types shall be used, i.e. the type-1 AAI subframe shall be transformed to type-3 AAI subframe and the type 2 AAI subframe shall be transformed to type-1 AAI subframe and the last symbol shall be used for R-RTI allocation.

============================= End of the text #6==============================

============================= Start of the text #7==============================

[Note to Editor – Modify the text in section 16.6.3.4 Downlink Control Structure in page 737 as follows]

16.6.3.4 Downlink Control Structure

In the 16m DL Access zone of the ABS frame and ARS frame, all the DL control channels described in 16.3.6 are reused.

In the 16m DL Relay zone, only the A-MAP control channel described in 16.3.6 is reused for communication between the ABS and the ARS.

In TDD and FDD frame configurations for both possible R-TTI values (0 and duration of 1 OFDMA symbol) the ABS shall perform mapping of DL control channels in the first AAI subframe of the 16m DL Relay zone as it is defined in section 16.3.6.3. When the R-TTI is equal to one OFDMA symbol the ARS shall process DL control signaling assuming that ABS has allocated DL control information on all OFDMA symbols of the subframe.

In FDD frame configurations for both possible R-RTI values (0 and duration of 1 OFDMA symbol) the ABS shall perform mapping of DL control channels in the last AAI subframe of the 16m DL Relay zone as it is defined in section 16.3.6.3. When the R-TTI is equal to one OFDMA symbol the ARS shall process DL control signaling assuming that ABS has allocated DL control information on all OFDMA symbols of the subframe.

============================= End of the text #7==============================