| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Control Message catogorization |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Joey Chou Sassan Ahmadi Muthaiah Venkatachalam Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text of MAC Control Message catogorization |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- MAC Control Message Catogorization

- Joey Chou, Muthaiah Venkatachalam

- Intel

- Introduction

This contribution adds catogorization to the MAC control message table.

- Proposed text

1. 	Proposed Text I

16.2.3 MAC Control messages

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Replace Table 675 as trhe following]

Table 675 – MAC Control Messages

| No. | Functional Areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| 1 | System Information | AAI_SCD | System configuration descriptor | N/A | Broadcast |
| 2 |  | AAI_SII-ADV | Service Identity Information Advertisement | N/A | Broadcast |
| 3 |  | AAI_ULPC_NI | UL Noise and Interference Level Broadcast | N/A | Broadcast |
| 4 |  | AAI-LBS-ADV | LBS Advertizement | NA | Broadcast |
| 5 | Network Entry / Re-entry | AAI_RNG-REQ | Ranging Request | Null: during ranging procedure when there is no SA already established or pre-updated. CMAC: all other cases | Initial Ranging or Unicast |
| 6 |  | AAI_RNG-RSP | Ranging Response | Null: during ranging procedure when there is no primary SA already established or pre-updated. Encrypted/ICV: all other cases in response to the AAI_RNG-REQ message | Initial Ranging or Unicast |
| 7 |  | AAI_RNG-ACK | Aggregated CDMA Ranging Acknowledge | N/A:in broadcast null :in unicast when primary SA is not established Encrypted/ICV :in unicast when primary SA is established | Broadcast/ Unicast |
| 8 |  | AAI_RNG-CFM | Ranging Confirmation | Encrypted/ICV | Unicast |
| 9 |  | AAI_REG-REQ | Registration Request | Encrypted/ICV | Unicast |
| 10 |  | AAI_REG-RSP | Registration Response | Encrypted/ICV | Unicast |
| 11 |  | AAI_SBC-REQ | Basic Capability Request | null: during capability negotiation when there is no primary SA already established or pre-updated. Encrypted/ICV: all other cases | Unicast |
| 12 |  | AAI_SBC-RSP | Basic Capability Response | null: during capability negotiation when there is no primary SA already established or pre-updated. Encrypted/ICV: all other cases | Unicast |
| 13 | Network Exit | AAI_DREG-REQ | Deregistration Request | Encrypted/ICV | Unicast |
| 14 |  | AAI_DREG-RSP | Deregistration Response | Encrypted/ICV | Unicast |
| 15 | Connection Management | AAI_DSA-REQ | Dynamic Service Addition Request | Encrypted/ICV | Unicast |
| 16 |  | AAI_DSA-RSP | Dynamic Service Addition Response | Encrypted/ICV | Unicast |
| 17 |  | AAI_DSA-ACK | Dynamic Service Addition Acknowledge | Encrypted/ICV | Unicast |
| 18 |  | AAI_DSC-REQ | Dynamic Service Change Request | Encrypted/ICV | Unicast |
| 19 |  | AAI_DSC-RSP | Dynamic Service Change Response | Encrypted/ICV | Unicast |
| 20 |  | AAI_DSC-ACK | Dynamic Service Change Acknowledge | Encrypted/ICV | Unicast |
| 21 |  | AAI_DSD-REQ | Dynamic Service Deletion Request | Encrypted/ICV | Unicast |
| 22 |  | AAI_DSD-RSP | Dynamic Service Deletion Response | Encrypted/ICV | Unicast |
|  |  | AAI_DSX-RVD | DSx Received | Encrypted/ICV | Unicast |
| 23 |  | AAI_GRP-CFG | Group Configuration | Encrypted/ICV | Unicast |
| 24 | Security | AAI_PKM-REQ | Privacy Key Management Request | before AK is derived at network entry: NULL after AK is derived at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived at network entry and the other message is enclosed: CMAC | Unicast |
| 25 |  | AAI_PKM-RSP | Privacy Key Management Response | before AK is derived at network entry: NULL at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived after AK is derived at network entry and the other message is enclosed: CMAC | Unicast |
| 26 | ARQ | AAI_ARQ-Feedback | Stand-alone ARQ Feedback | null Encrypted/ICV | Unicast |
| 27 |  | AAI_ARQ-Discard | ARQ Discard | Encrypted/ICV | Unicast |
| 28 |  | AAI_ARQ-Reset | ARQ Reset | Encrypted/ICV | Unicast |
| 29 | Sleep Mode | AAI_SLP-REQ | Sleep Request | Encrypted/ICV | Unicast |
| 30 |  | AAI_SLP-RSP | Sleep Response | Encrypted/ICV | Unicast |
| 31 |  | AAI_TRF-IND | Traffic Indication | N/A | Broadcast |
| 32 |  | AAI_TRF_IND-REQ | Traffic indication request | Encrypted/ICV | Unicast |
| 33 |  | AAI_TRF_IND-RSP | Traffic indication response | Encrypted/ICV | Unicast |
| 34 | Handover | AAI_HO-REQ | AMS Handover Request | Encrypted/ICV | Unicast |
| 35 |  | AAI_HO-CMD | ABS Handover Command | Encrypted/ICV | Unicast |
| 36 |  | AAI_HO-IND | AMS Handover Indication | Encrypted/ICV | Unicast |
| 37 |  | AAI_NBR-ADV | Neighbor Advertisement | Null: in unicast N/A: in broadcast | Unicast or broadcast |
| 38 |  | AAI_NBR-REQ | Request Neighbor List | NA | Unicast |
| 39 |  | AAI_SCN-REQ | Scanning Interval Allocation Request | Encrypted/ICV | Unicast |
| 40 |  | AAI_SCN-RSP | Scanning Interval Allocation Response | Encrypted/ICV | Unicast |
| 41 |  | AAI_SCN-REP | Scanning Result Report | Encrypted/ICV | Unicast |
|  |  | AAI_HO-REQ | AMS Handover Request | Encrypted/ICV | Unicast |
|  |  | AAI_HO-CMD | ABS Handover Command | Encrypted/ICV | Unicast |
|  |  | AAI_HO-IND | AMS Handover Indication | Encrypted/ICV | Unicast |
|  |  | AAI_TRF_IND-REQ | Traffic indication request | Encrypted/ICV | Unicast |
|  |  | AAI_TRF_IND-RSP | Traffic indication response | Encrypted/ICV | Unicast |
|  |  | AAI_L2-XFER | AAI L2 Transfer | Encrypted/ICV | Unicast |
| 42 | Idle Mode | AAI_PAG-ADV | BS Paging Advertisement | N/A | Broadcast |
|  |  | PGID_INFO | Paging Group Advertisement | N/A | Broadcast |
|  |  | AAI_ULPC_NI | UL Noise and Interference Level Broadcast | N/A | Broadcast |
| 43 | Multicarrier | AAI-MC-ADV | multicarrier Advertisement | N/A | Broadcast |
| 44 |  | AAI_MC-REQ | multicarrier Request | Null Encrypted/ICV | Unicast |
| 45 |  | AAI_MC-RSP | multicarrier Response | Null Encrypted/ICV | Unicast |
| 46 |  | AAI_CM-CMD | Carrier Management Command | Encrypted/ICV | Unicast |
| 47 |  | AAI_CM-IND | Carrier Management Indication | Encrypted/ICV | Unicast |
| 48 |  | AAI_Global-Config | Global Carrier Configuration | NA | Unicast |
| 49 | Power Control | AAI_UL_POWER_ADJUST | Uplink TX power adjustment | Null Encrypted/ICV | Unicast |
| 50 |  | AAI_UL_PSR_Config | Uplink Power Status Reporting Configuration | Null Encrypted/ICV | Unicast |
| 51 |  | AAI_UL_PSR | Uplink Power Status Report | Null Encrypted/ICV | Unicast |
| 52 | Collocated Coexistence | AAI_CLC-REQ | Co-located coexistence request | Encrypted/ICV | Unicast |
| 53 |  | AAI_CLC-RSP | Co-located coexistence response | Encrypted/ICV | Unicast |
|  |  | AAI_RES-CMD | Reset command | Before authentication :Null After authentication : Encrypted/ICV | Unicast |
| 55 |  | AAI_SingleBS_MIMO_FBK | Single-BS MIMO feedback | <TBD> Null | Unicast |
| 56 |  | AAI_MultiBS_MIMO_FBK | Multi-BS MIMO feedback | <TBD> Null | Unicast |
| 57 |  | AAI_DL_IM | Downlink interference mitigation parameter | N/A | Broadcast |
| 58 |  | AAI_MULTI_BS_MIMO-REQ | Multi-BS MIMO Request | <TBD> Null | Unicast |
| 59 |  | AAI_MULTI_BS_MIMO-RSP | Multi-BS MIMO Response | <TBD> Null | Unicast |
|  |  | PGID_Info | Paging Group Advertisement | N/A | Broadcast |
| 60 |  | AAI_MultiBS_PMI_COM | Multi-BS PMI Combination | Null | Unicast |
|  |  | AAI_GRP-CFG | Group Configuration | Encrypted/ICV | Unicast |
| 61 | FFR | AAI_FFR-CMD | FFR measurement report command | Null Encrypted/ICV | Unicast |
| 62 |  | AAI_FFR-REP | FFR measurement report | Null Encrypted/ICV | Unicast |
|  |  | AAI_UL_POWER_ADJUST | Uplink TX power adjustment | Encrypted/ICV | Unicast |
|  |  | AAI_UL_PSR_Config | Uplink Power Status Reporting Configuration | Encrypted/ICV | Unicast |
|  |  | AAI_UL_PSR | Uplink Power Status Report | Encrypted/ICV | Unicast |
| 63 | SON | AAI-SON-ADV | SON Advertizement | NA | Broadcast |
| 64 | MISC | AAI_L2-XFER | AAI L2 Transfer | Encrypted/ICV | Unicast |
|  |  | AAI_CLC-REQ | Co-located coexistence request | Encrypted/ICV | Unicast |
|  |  | AAI_CLC-RSP | Co-located coexistence response | Encrypted/ICV | Unicast |
| 65 |  | AAI_MSG-ACK | MAC message acknowledgement | NA | Unicast |
| 66 |  | AAI_E-MBS-CFG | E-MBS Configuration | Null | Broadcast |
| 67 |  | AAI_RES-CMD | Reset command | Before authentication :Null After authentication : Encrypted/ICV | Unicast |
|  |  | AAI_LBS-ADV | LBS Advertisement | Null | Broadcast |

------------------------------------------------- End of proposed text IX --------------------------------------------------