| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Legacy Support Modes in D4 (16.3.3.7) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Mark Cudak, Fred Vook, Bill Hillery, Eugene Visotsky, Anup Talukdar Motorola Jaeweon Cho, Hokyu Choi Samsung Electronics Co., Ltd. Ran Yaniv Alvarion Masoud Olfat Clearwire | Mark.Cudak@motorola.com Jaeweon.Cho@samsung.com Ran.Yaniv@alvarion.com Masoud.Olfat@clearwire.com |
| Re: | Category: P802.16m/D4 comments for LB31 Area: Chapter 16.3.3.7 |  |
| Abstract | The current backward compatibility modes in D4 do not meet the requirements for legacy support in the SRD. Specifically, the performance does not scale with the fraction of deployed 16m mobiles. The current draft requires that a minimum of 2 subframes be allocated to 16m even when there are no 16m mobiles with data queued. As a result, 40% of the downlink capacity will be lost to 16e mobiles as soon as the 16m mode is enabled. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Legacy Support Modes in D4 (16.3.3.7)

Fred Vook, Mark Cudak, Bill Hillery,

Eugene Visotsky, Anup Talukdar

Motorola Inc

Jaeweon Cho, Hokyu Choi

Samsung Electronics Co., Ltd.

Ran Yaniv

Alvarion

Masoud Olfat

Clearwire

- Introduction

The current backward compatibility modes in D4 do not meet the requirements for legacy support in the SRD.  Specifically, the performance does not scale with the fraction of deployed 16m mobiles.  The current draft requires that a minimum of 2 subframes be allocated to 16m even when there are no 16m mobiles with data queued.  As a result, 40% of the downlink capacity will be lost to 16e mobiles as soon as the 16m mode is enabled.

The remedy below proposes the following:

1) Reduce minimum number of 16m subframes to one subframe instead of two.

2) Introduce a new set of frame configurations with a DL length of 1 for a 5:3 TDD split

3) Introduce backward compatible frame configurations for 6:2 TDD split.

4) Utilize a Type 2 (7 symbol) subframe when the Frame Configuration Index provisions a DL length of 1 so that both a 16m pre-amble and MIMO mid-midamble can be included within the same sub-frame.

- Proposed Text Changes in P802.16m/D4

[Remedy 1: Insert the following entries into Table 780 on the page 369.  Replace “*” with next available “No” and “Frame configuration index” as appropriate]

Table 780 – Frame Configuration and Indexing (5/10/20MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 5:3 | - 2 | - 1 | - 3 | - DL Type2 | - Not used | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 5:3 | - 1 | - 1 | - 3 | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 1 | - 1 | - 2 | - DL Type2 | - Not used | - Not used | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 1 | - 2 | - 2 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 1 | - 3 | - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 1 | - 4 | - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 1 | - 5 | - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 2 | - 1 | - 2 | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 2 | - 2 | - 2 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 2 | - 3 | - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - * | - 5/10/20 | - 1/8 | - * | - TDD | - 6:2 | - 2 | - 4 | - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |

[Remedy 2: Insert the following entries into Table 781 on the page 370.  Replace “*” with next available “No” and “Frame configuration index” as appropriate]

Table 781 – Frame Configuration and Indexing (8.75 MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - * | - 8.75 | - 1/8 | - * | - TDD | - 5:2 | - 2 | - 1 | - 2 | - DL Type2 | - Not used | - Not used | - UL Type1 | - UL Type4 | - Not used | - Not used |  | - 87.2/74.4 |

Remedy 3: Insert the following entries into Table 782 on the page 371.  Replace “*” with next available “No” and “Frame configuration index” as appropriate]

Table 782 – Frame Configuration and Indexing (7 MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - * | - 7 | - 1/8 | - * | - TDD | - 3:2 | - 1 | - 1 | - 2 | - DL Type2 | - Not used | - UL Type1 | - UL Type1 | - Not used |  |  |  | - 180/60 |

[Remedy 4: Edit the text in section 16.3.5.4.2 MIMO mid-amble on page 400 as follows: ]

MIMO midamble is used for PMI selection in closed loop MIMO. For OL MIMO, midamble can be used to calculate CQI. MIMO midamble shall be transmitted every frame on the second DL AAI subframe. The midamble signal occupies the first OFDMA symbol in a DL type-1 or type-2 AAI subframe. For the type-1 AAI subframe case, the remaining 5 consecutive symbols form a type-3 AAI subframe. For the type-2 AAI subframe case, the remaining 6 consecutive symbols form a type-1 AAI subframe.

For the special case where the frame structure supports Wirelessman-OFDMA MSs and Frame Configuraton Index provisions that the DL Length is equal to 1 subframe, the MIMO midamble will be transmitted every frame on the last symbol of the first DL AAI subframe.  In these cases, the first DL AAI subframe will always be a type-2 AAI subframe.   The first symbol of this type 2 AAI subframe will be occupied by a preamble symbol and the last symbol will be occupied by the MIMO mid-amble.  The remaining 5 consecutive symbols form a type-3 subframe.