| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on security context maintenance during zone switching (16.2.5) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Xiangying Yang Avishay Shraga Chang Hong Shan Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.5” |  |
| Abstract | This contribution proposes text for security context maintenance during ZS preparation |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on security context maintenance during zone switching (16.2.5)

Xiangying Yang, Avishay Shraga, Chang Hong Shan

Intel

- Introduction

The text change propose to SA mapping rules during Zone Switch functions.

- Proposed Test

[insert new subsection into Section 16.2.5.2.2, P172 L24 in D4]

16.2.5.2.2.2 Mapping of flows to SAs during Zone Switching

During LZone to MZone zone switch, LZone CIDs are mapped to MZone FIDs as defined in 16.2.6.4.1.3 and their mapping to MZone SA is shown in Table aaa.

Table aaa LZone CID to MZone SA mapping during 16e->16m zone switch

| CIDs in LZone | SA in MZone |
| --- | --- |
| Management connections | Null SA (SAID = 0x00), if only security suites with “No data encryption, no data authentication” is supported Primary SA (SAID = 0x01), otherwise |
| Unicast transport CID with SA using AES-CCM | Primary SA (SAID = 0x01) |
| Unicast transport CID with Null SA | Null SA (SAID = 0x00) |
| Broadcast or multicast transport CID | Null SA (SAID = 0x00) |

During MZone to LZone zone switch, MZone FIDs are mapped to LZone CIDs as defined in 16.2.6.4.2.3 and their mapping to LZone SA is shown in Table bbb.

Table bbb MZone FID to LZone SA mapping during MZone to LZone zone switch

| FIDs in MZone | SA in LZone |
| --- | --- |
| Transport FID associated with Primary SA (SAID = 0x01) | Primary SA using AES-CCM |
| FID associated with SAID = 0x02 | Primary SA using AES-CCM |
| FID associated with Null SA (SAID = 0x00) | Null SA using “No data encryption; no data authentication” |
| Broadcast or multicast transport FID | Null SA using “No data encryption; no data authentication” |

--------------------------------text end-----------------------------------------