| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on Direct HO (16.2.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Xiangying Yang Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.6.4.1.2.2” |  |
| Abstract | This contribution proposes text for Direct HO |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on Direct HO (16.2.6)

Xiangying Yang

Intel

- Introduction

The text change propose to 16.2.6.4.1.2.2 Direct HO from YBS to ABS.

- Proposed Test

--------------------------------text start-----------------------------------------

[Replace Section 16.2.6.4.1.2.2 as follow]

16.2.6.4.1.2.2 Direct Handover Procedure

An AMS served by a WirelessMAN-OFDMA BS may discover an AAI only ABS and decide to directly HO to this ABS. In this case, the AMS performs WirelessMAN-OFDMA Reference System HO procedure to the serving WirelessMAN-OFDMA BS per Section 6.3.21.2, and then performs WirelessMAN-OFDMA Advance System network reentry procedure to the target ABS per Section 16.2.6.3.5.2. The AMS may obtain target AAI only ABS information by blind scanning.

[delete Figure 416]

--------------------------------text end-----------------------------------------