| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on 16e-to-16m zone switching (16.2.6) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Xiangying Yang Intel Kelvin Chou MediaTek Yung-han Chen, ITRI Inuk Jung LGE Jaehyuk Jang Samsung | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.6.4.1.2.1” |  |
| Abstract | This contribution proposes text for Zone Switching procedure |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on 16e-to-16m zone switching (16.2.6)

Xiangying Yang

Intel

Kelvin Chou

MediaTek

Yung-han Chen,

ITRI

Inuk Jung

LGE

Jaehyuk Jang

Samsung

- Introduction

The text change propose to clarify zone switching procedure. Since whether network anchor is ready for zone switching ( security etc) is unknown to AMS, ASM initiated ZS option should be removed.

- Proposed Test

Text change: modify the text in Section 16.2.6.4.1.2.1 as follows:

--------------------------------text start-----------------------------------------

16.2.6.4.1.2.1 Zone Switch based Handover Procedure

The zone switch based HO begins with a decision for an AMS to HO from the serving WirelessMAN-OFDMA BS to the LZone of a target ABS. The HO decision, initiation and cancellation follow the same procedures as defined in section 6.3.21.2,. except that Zone Switch is only initiated either by AMS or ABS while the final decision shall be made by ABS. In case of an AMS initiated zone switch, the AMS transmits an RNG-REQ message with the 'Ranging purpose indicator' bit #5 set to 1, which implies the zone switch request. Upon reception of such RNG-REQ message, for which the ABS sends a RNG-RSP message including the zone switch TLV.

The AMS performs network reentry in the LZone of the target ABS following the same procedures as defined in section 6.3.21.2.7. In addition, upon knowing the AMS capability of supporting WirelessMAN-OFDMA Advanced System based on the MAC version obtained either from the RNG-REQ sent from AMS in the LZone or from the serving ABS over the backbone, the ABS may direct the AMS to switch from LZone to MZone during or after AMS network reentry to the LZone.

In case zone switch from Lzone to Mzone during network reentry is required, during an AMS performing network reentry in LZone, the ABS directs the AMS to stop the LZone network reentry and switch from LZone to MZone immediately by transmitting the RNG-RSP message containing Zone Switch TLV, as shown in Figure 414.

--------------------------------text end-----------------------------------------