| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on snooping based femtocell scanning (16.4.8) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Xiangying Yang Intel I-Kang Fu MediaTek Frank Ren ITRI Ying Li Samsung | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.4.8.1.2” |  |
| Abstract | This contribution proposes text for more accurate snooping operation |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on snooping based femtocell scanning (16.4.8)

Xiangying Yang

Intel

I-Kang Fu

MediaTek

Frank Ren

ITRI

Ying Li

Samsung

- Introduction

The text change propose to allow more accurate proximity detection during snooping operations.

- Proposed Test

Modify section 16.4.8.1.2 as follows

--------------------------------text start-----------------------------------------

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

For neighbor Femto ABSs, an AMS performs the scanning procedure as per 16.3.6.1.2 with exceptions described in this subsection. An AMS may scan femto ABSs according to the FAs included in the broadcast AAI_NBR-ADV message. AMS may scan femto ABSs that are not included in AAI_NBR-ADV based on SA-preamble partitioning information (see 16.4.4). In addition, an AMS may scan allowed femto ABSs based on the CSG White List. Based on location information, AMS may initiate the scanning procedure (See 16.2.6.1.2). The AMS may request additional scanning opportunity by sending AAI-SCN-REQ including the detected SA-preamble index and FA information. Upon reception of the AAI_SCN-REQ, the ABS shall respond with an AAI_SCN-RSP including neighbor accessible Femto ABS list based on the SA-preamble index.

Alternatively, an overlapped overlay Macro ABS may recommend CSG Femto ABS to monitor UL signaling of accessible its member AMS which is served by the Macro ABS. The serving Macro ABS should further select a set of the accessible CSG Femto ABS for an AMS based on AMS location information, or detected base station information reported by the AMS, if such information is available, before the serving Macro ABS recommends the CSG Femto ABS to monitor UL signaling. If the set of the accessible CSG Femto ABS selected by the serving Macro ABS is empty, the serving Macro ABS shall not recommend any CSG Femto ABS to monitor UL signaling. If CSG Femto ABS is in low-duty mode, the CSG Femto ABS can try to receive A-MAP information from overlapped overlay Macro ABS and then monitor accessible its member AMS’s UL signaling in associated allocations. If CSG Femto ABS is not in low-duty mode, overlapped Macro ABS may trigger dedicated periodic ranging (see 15.4.7.4.2) for the AMS. A CSG Femto ABS can monitor the ranging preamble at the dedicated ranging slot of Macro ABS. When the received signaling quality (e.g RSSI) of an AMS is stronger than a threshold, satisfies certain conditions to indicate its proximity to the Femto BS, CSG Femto ABS can request overlapped Macro ABS to send unsolicited AAI_SCN-RSP for the AMS to scan the CSG Femto ABS. In particular, Macro BS informs the current uplink transmission power of the AMS to the Femto BS so that Femto BS may estimate AMS’s proximity based on calculated path loss.

When the AMS has to request for information of neighboring Femto ABSs belonging to a CSG then the AMS can provide the desired CSGID(s) in the AAI_SCN-REQ message to the serving ABS. The ABS can respond with a list of ABSs, addressed by BSID belonging to the requested CSGID(s) in AAI_SCN-RSP message.

ABS may send an unsolicited AAI_SCN-RSP for the AMS to scan the Femto ABS.

--------------------------------text end-----------------------------------------