| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of SA-Preamble Partitioning for ABS Type (16.3.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Xiangying Yang Etemad Kamran Yang-seok Choi Muthaiah Venkatachalam Intel | Xiangying.yang@intel.com |
| Re: | Commets on P802.16m/D4 for recirculation #31 |  |
| Abstract | The contribution provides the proposed text of SA-Preamble partitioning for ABS type to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of SA-Preamble Partitioning for ABS Type (16.3.6)

Xiangying Yang, Etemad Kamran, Yang-seok Choi, Muthaiah Venkatachalam

Intel

- Introduction

In previous San Diego meeting, the high level concept of partitioning for SA-Preamble sequences were accepted for supporting partitioning of different ABS type. Given current Table 798, there is a potential IDcell wastage when one of the cell type does not exist in the network. On the other hand, current granularity of allocation may be “too fine scale” anyway, such as only giving 10 IDcells for a cell type. We propose to slightly modify the table to allow full usage of IDcell for one cell type.

- References

[1] IEEE 802.16m-09/1902r3, “Proposed Text Proposal of SA-Preamble Partition in IEEE 802.16m/D1”

[2] IEEE 802.16m-07/0034r2, “802.16m System Requirements”

[3] IEEE 802.16m/D4, “Advanced Air Interface (Draft)”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Modify Table-789 P802.16m/D4 as following

-------------------------------------------------------  Text Start  --------------------------------------------------------------

SA-Preamble sequences are partitioned and each partition is dedicated to specific base station type like macrocell ABS, Macro Hotzone ABS, Femto ABS. The base station types are categorized into macro ABS and non-macro ABS cells by hard partition with 258 sequences (86 sequences per segment * 3 segments) dedicated for macro ABS. The non-macro ABS information is broadcasted in a hierarchical structure, which composes of S-SFH SP3 and AAI_SCD message. In S-SFH SP3, non-macro ABS cell type is partitioned as public and CSG femto base stations. Total 16 cases of IDcell partition for public and CSG-femto ABS are shown in Table 798, which is composed of IDcell partitions based on 30 sequence (10 sequences per segmentation) granularity.

Table 798 —IDCell partitioning for public ABS and CSG-femto ABS

| Value indicated in 4bit-SFH SP3 | IDCell partition for public ABS (Number of sequence per each segmentation) | IDCell partition for CSG-femto ABS (Number of sequence per each segmentation) |
| --- | --- | --- |
| 0000 | 86+256*n ~ 8695+256*n (010) | 8696+256*n ~ 255+256*n (170160) |
| 0001 | 86+256*n ~ 105+256*n (20) | 106+256*n ~ 255+256*n (150) |
| 0010 | 86+256*n ~ 115+256*n (30) | 116+256*n ~ 255+256*n (140) |
| 0011 | 86+256*n ~ 125+256*n (40) | 126+256*n ~ 255+256*n (130) |
| 0100 | 86+256*n ~ 135+256*n (50) | 136+256*n ~ 255+256*n (120) |
| 0101 | 86+256*n ~ 145+256*n (60) | 146+256*n ~ 255+256*n (110) |
| 0110 | 86+256*n ~ 155+256*n (70) | 156+256*n ~ 255+256*n (100) |
| 0111 | 86+256*n ~ 165+256*n (80) | 166+256*n ~ 255+256*n (90) |
| 1000 | 86+256*n ~ 175+256*n (90) | 176+256*n ~ 255+256*n (80) |
| 1001 | 86+256*n ~ 185+256*n (100) | 186+256*n ~ 255+256*n (70) |
| 1010 | 86+256*n ~ 195+256*n (110) | 196+256*n ~ 255+256*n (60) |
| 1011 | 86+256*n ~ 205+256*n (120) | 206+256*n ~ 255+256*n (50) |
| 1100 | 86+256*n ~ 215+256*n (130) | 216+256*n ~ 255+256*n (40) |
| 1101 | 86+256*n ~ 225+256*n (140) | 226+256*n ~ 255+256*n (30) |
| 1110 | 86+256*n ~ 235+256*n (150) | 236+256*n ~ 255+256*n (20) |
| 1111 | 86+256*n ~ 255245+256*n (170160) | 255246+256*n ~ 255+256*n (010) |

-------------------------------------------------------  Text End  ----------------------------------------------------------------