| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on EBB HO capability clarification (16.2.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Xiangying Yang, Aran Bergman Intel | E-mail: xiangying.yang@intel.com, aran.bergman@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.6” |  |
| Abstract | This contribution proposes text for EBB HO capability |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on EBB HO capability clarification (16.2.6)

Xiangying Yang, Aran Bergman

Intel

- Introduction

The text change propose to complete the EBB HO capability negotiation in AAI_REG-REQ

- Proposed Test

Text proposal

Modify Section 16.2.6.3 as follows

------------------------------text start---------------------------------------

16.2.6.3.3 HO Preparation

During HO preparation phase, the serving ABS communicates with target ABS(s) selected for HO. The AAI-HO_CMD message should not include an ABS with cell bar information =1. A Femto ABS shall follow the handover procedure described in section 16.4.8. The target ABS may obtain AMS information from the serving ABS via backbone network for HO optimization.

During HO preparation phase, the target ABS may allocate a dedicated ranging code and dedicated ranging opportunity to the AMS via the serving ABS through the AAI_HO-CMD message, indicated by both Dedicated_Ranging_Code_Flag and Dedicated_Ranging_Opportunity_Flag. The dedicated code and opportunity is assigned to the AMS until the Ranging Initiation Deadline. The AMS may reuse the pre-assigned dedicate ranging code and opportunity during network re-entry. If the AMS fails to perform ranging before expiration of Ranging Initiation Deadline it shall stop using the dedicated code and opportunity but randomly pick a ranging code if further ranging is necessary. The target ABS shall select the dedicated ranging code from the group of codes which are allocated for dedicated handover ranging purpose.

Information regarding AMS identity (e.g.STID) and security context should be pre-updated during HO preparation. Any mismatched system information between AMS and the target ABS, if detected, may be provided to the AMS by the Serving ABS during HO preparation. If pre-allocated at target ABS, the serving ABS shall include an STID to be used at target ABS in the AAI_HO-CMD message. The pre-allocated STID shall be used in the target ABS by the AMS to communicate with the target ABS. The FIDs which are used to distinguish different connections are not updated during the handover procedure. If the network decides that certain service flow will not exist at the target ABS, this shall also be indicated in the AAI_HO-CMD message.

If HO_Reentry_Mode is set to 1, the serving ABS shall negotiate with the target ABS the EBB HO parameters. In the single carrier handover case, the EBB HO parameters include HO_Reentry_Interleaving_Interval and HO_Reentry_Iteration for the AMS to communicate with the serving ABS during network reentry, in which case HO_Reentry_Interleaving_Interval must be no less than the minimal values defined in AMS capability. In the multicarrier handover case, the EBB_HO parameters include the carrier information in the target ABS for the AMS performing network reentry while continuing communication with the serving ABS concurrently.

The ABS shall not set HO_Reentry_Mode to 1 unless EBB Support was declared (set to 1) by the AMS in AAI_REG-REQ.

------------------------------text end-----------------------------------------

II. Insert one row of capability in Table 682 within category “AMS capability negotiation parameters” as follows:

--------------------------------text start-----------------------------------------

16.2.3.7 AAI_REG-REQ

Table 682—Parameters for AAI_ REG-REQ

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capability negotiation parameters | Capabilities for Handover EBB mode | EBB support | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported |
|  |  | Minimal HO_Reentry_Interleaving_Interval | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. Shall be included only if EBB support is set to 1 |
|  |  | Reserved |  |

16.2.3.11 AAI_HO-CMD

Table 686—Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| … |  |  |
| HO Reentry Mode | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. Shall be included when Mode = 0b01 Shall not be set to 1 if AMS does not support EBB (according to AAI_REG-REQ). |
| … |  |  |

--------------------------------text end-----------------------------------------