| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Adding MAC layer CRC to improve reliability (16.2.2, 16.2.3.41.1, 16.2.3.41.2, 16.2.3.41.4, 16.2.3.41.5) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Muthaiah Venkatachalam, Aran Bergman, Frishman, Yaniv | Intel Corporation |
| Re: | Adding MAC layer CRC to improve reliability |  |
| Abstract | Adding MAC layer CRC to improve reliability |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Adding MAC layer CRC to improve reliability

# Introduction

According to 802-2001 [1], section 7.3, the error misdetection probability for a 802 protocol (at least for data, I guess), shall be less than 5x10^-14 per octet or MSDU length. It would seem that the 16-bit CRC at the burst level is not sufficient to meet this requirement when a single FEC block is used per burst, since only one 16-bit CRC is used in this case.

To reduce the error misdetection probability, need to add 4 byte CRC. Whether to add the CRC or not - based on FID configuration (DSA).

Besides meeting the 802-2001 requirements, there is a potential user-experience problem with the current D4 text:

Consider a large zip file transmitted over a 16m network without encryption (or with AES-CTR encryption) while the user is near the cell-edge so that allocations are rather small, constituting a single FEC block each. Each of these bursts contains a single 16-bit CRC protecting the burst.

The ABS (due to its link adaptation algorithm) aims at a first transmission error rate of ~20%. According to our simulations, using a PedB 3Km/h fading channel and bursts of ~4000 bits, when the burst is in error (which happens with a probability of about 0.2), more than 40 bits in the burst are flipped with very high probability. When this happens, the probability of error misdetection of erroneous bursts is ~2^-16. The probability that a burst would be delivered to the upper layer with undetected errors is thus greater than 0.2*2^-16 = 3e-6 (greater than, since we only considered first transmission error misdetection…)

So, if each burst contains 500 bytes of the file, any file greater than ~110MB has >0.5 probability of containing at least one undetected error. Correctly delivering a file of ~350MB has only about 10% chance of being correctly received without undetected errors.

If this file is a zip file, assuming the zip file’s CRC will eventually detect the error, this means a very frustrated user, who waited a very long time for the file to download/upload and cannot use the file, since windows / WinZip will declare the file corrupt.

Reference:

[1] IEEE 802.2001: IEEE Standard for Local and Metropolitan Area Networks: Overview and Architecture, 8 March 2002

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

16.2.2 MAC PDU formats

MAC PDUs shall be of the form illustrated in Figure 385. Each PDU shall begin with a MAC header. The header may be followed by one or more extended headers. The MAC PDU may also contain payload. The MAC PDU may also contain a 32 bit CRC if it is agreed upon during the connection establishment that CRC shall be included. The CRC shall cover the generic MAC header and the payload of the MAC PDU. The 32 bit CRC shall be generated according to section 6.3.3.5.2

Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS can be multiplexed into a single MAC PDU. The multiplexed unicast connections shall be associated with the same security association.

[Replace Figure 385 as follows]

Figure 385 – MAC PDU format

16.2.3.41.1 AAI_DSA-REQ

[Add the following parameter to the message]

| Name | Value | Usage |
| --- | --- | --- |
| CRC Indicator (1 bit) | 0 = No CRC 1 = 32 bit CRC | Indicates whether CRC shall be added to all the PDUs of this FID or not |

16.2.3.41.2 AAI_DSA-RSP

[Add the following parameter to the message]

| Name | Value | Usage |
| --- | --- | --- |
| CRC Indicator (1 bit) | 0 = No CRC 1 = 32 bit CRC | Indicates whether CRC shall be added to all the PDUs of this FID or not |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]