| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on Quick Access Message Format (16.2.11) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jie Hui Xiangying Yang Muthaiah Venkatachalam Aran Bergman Shaocheng Wang Intel Yih-Shen Chen Kelvin Chou Mediatek Inc. | E-mail: jie.hui@intel.com yihshen.chen@mediatek.com kelvin.chou@mediatek.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.11” |  |
| Abstract | This contribution proposes quick access message format and its encoding. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on Quick Access Message Format (16.2.11)

Jie Hui, Xiangying Yang, Muthaiah Venkatachalam, Aran Bergman, Shaocheng Wang

Intel Corp

Yih-Shen Chen, Kelvin Chou

Mediatek Inc.

- Introduction

Quick access message format is missing from D4. This contribution proposes its format and its encoding which is negotiated through AAI_DSx messages.

- Proposed Test

--------------------------------text start-----------------------------------------

16.2.11.1.5.1 Quick access message format

When a 3-step BW-REQ procedure is used, in step-1 the quick access message carries 12 bit information including AMS addressing information with quick access sequences carrying additional 4-bit BW-REQ information.

The following parameters are carried in step-1 of 3-step BW-REQ procedure the quick access message as shown in Table xxx. Detailed encoding method is provided in the section <<15.3.9.1.5>>.

•STID (12 bits)

•Predefined BR index (4 bits): The mapping between 'predefined BR index' and 'BR size & QoS level' is done negotiated during AAI-DSx procedure and its encoding is defined in section 16.2.3.45.  'BR size ' and QoS level ' is determined based on the QoS parameters of the flow in the DSx messages. 'BR size' is in unit of bytes and is mapped to Maximum Traffic Burst parameter, and 'QoS level' is mapped to UL Grant Scheduling Type parameter.

Table xxx—Quick Access Message Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Quick Access Message() {. |  |  |
| STID | 12 | Station ID |
| Predefined BR index | 4 | Range: 0-15. Definition is MS specific based on AAI_DSx negotiation. |
| } |  |  |

16.2.3.45.1 AAI_DSA-REQ

Following parameters may be included in the AAI_DSA-REQ message.

—SCID: To switch sleep cycle setting

—Predefined BR index parameters: To use in 3-step Bandwidth Request procedure, only included in ABS initiated DSA-REQ

Table xxx—Predefined BR Index Parameters

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Predefined BR Index Parameter () {. |  |  |
| Number of predefined BR Indexes (K) | 4 | Range: 0-15 |
| For (i=1 to K+1) { |  |  |
| Predefined BR index | 4 |  |
| BR Action | 2 | 00: ertPS service flow requests to resume to maximum sustained rate 01: aGP service flow requests to switch to Primary QoS parameters 10: BR 11: reserved |
| If (BR Action == 10){ |  |  |
| BR size | 11 | Number of bytes requested by the predefined BR index |
| } |  |  |
| } |  |  |
| } |  |  |

--------------------------------text end-----------------------------------------