| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of defragmentation behavior for non-ARQ and control connections |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Muthaiah Venkatachalam, Aran Bergman, Xiangying Yang Yih-Shen Chen, Kelvin Chou Ming-Hung Tao, Ying-Chuan Hsiao | Intel Corporation MediaTek Inc. ITRI |
| Re: | Clarification of defragmentation behavior |  |
| Abstract | Clarification of defragmentation behavior |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of defragmentation behavior for non-ARQ connections

# Introduction

In 802.16m we need to enhance the defragmentation state machine at the receiver for non-ARQ MAC connections that only use HARQ.

The current definition in the 16m standard would lead to dropping a lot of SDUs on such connections - "Upon loss, the receiver shall discard all SDU fragments on the connection until a new first SDU fragment is detected...". HARQ may temporarily be missing a burst which contains some fragment of a SDU because HARQ does not guarantee in-order delivery. If the standard does not define a mechanism to restore the order of the PDUs / Bursts before such a defragment mechanism, it will drop SDUs even though all of its fragments are correctly received (just not in order).

We propose to have a discard timer for each received fragment of an SDU of a non-ARQ MAC connection.

Whenever a fragment-k of an SDU is received on the non-ARQ connection, the discard timer for this fragment is started. If all the fragments of the SDU don’t arrive before the expiry of this discard timer, then all the fragments are discarded. Since there is no ARQ, the MAC layer will no longer retransmit this SDU – instead the retransmission will be left to higher layers (if such capability exists in the higher layers).

This discard timer value should be set by taking parameters into account - such as maximum delay time for a PDU (before the receiver gives up on receiving the missing SNs) and/or reordering buffer limitations in the receiver (which may be declared by the AMS, and influence the ABS's behavior on the DL).

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

16.2.4.3.1.1  Non-ARQ Transport Connections

For non-ARQ transport connections, fragments are transmitted once and in sequence. The SN assigned to each connection PDU carrying SDU fragment allows the receiver to recreate the original payload and to detect the loss of any intermediate fragments. A connection may be in only one fragmentation state at any given time. Upon loss, the receiver shall discard all SDU fragments on the connection until a new first SDU fragment is detected or a non-fragmented SDU is detected.

The exact mechanism for reordering PDUs and performing reassembly for non-ARQ transport connection is described in Section 16.2.x.

[Insert a new section after section 16.2.13]

### 16.2.14 Non-ARQ Reordering and Reassmebly Mechanism

The receiver shall maintain a reordering window to restore the order between SDUs which may have been changed due to HARQ retransmissions. This reordering window shall also guarantee the correct reassembly of received SDU fragments.

### 15.2.14.1 Non-ARQ Receiver Parameters and State Variables

The non-ARQ receiver makes use of the following parameters and state variables during its operation.

### 15.2.14.1.1 Receiver Parameters

NON_ARQ_SN_MODULUS is equal to the number of unique SN values in the FPEH, i.e., 210

NON_ARQ_WINDOW_SIZE is the maximum number of SNs in the sliding reordering window of PDUs that is managed by the receiver and is equal to half of NON_ARQ_SN_MODULUS.

NON_ARQ_REORDERING_TIMEOUT is that maximum amount of time the reordering and reassembly receiver waits for missing SNs. SNs not received when this timer expires are considered lost. Only one NON_ARQ_REORDERING timer is active per transport connection at any given time. This is the maximum time a PDU would be delayed in the reordering buffer.

### 15.2.14.1.2 Receiver Variables

NON_ARQ_RX_HIGHEST_SN: SN of the highest sequence number received plus one. It is initially set to zero.

NON_ARQ_RX_WINDOW_START: SN of the earliest SN considered for reordering and reassembly. It is initially set to zero.

NON_ARQ_REORDERING_SN holds the SN of the highest received sequence number when the NON_ARQ_REORDERING timer was triggered.

### 15.2.14.2 Sequence Number Comparison

The non-ARQ receiver operations includes comparing SNs and taking actions based on

which is larger or smaller. In this context, it is not possible to compare the numeric sequence number values

directly to make this determination. Instead, the comparison shall be made by normalizing the values

relative to the appropriate state machine base value and the maximum value of sequence numbers,

NON_ARQ_SN_MODULUS, and then comparing the normalized values. Normalization is accomplished by

using Equation (y).

SN' = (SN – SN_base) mod NON_ARQ_SN_MODULUS 	(y)

The base values for the non-ARQ receiver is (NON_ARQ_RX_HIGHEST_SN – NON_ARQ_WINDOW_SIZE).

### 15.2.14.3 Receiver Operation

When a PDU with SN = n is received from the HARQ layer, the receiver shall

If (NON_ARQ_WINDOW_START < n < NON_ARQ_RX_HIGHEST_SN) and n has already been received, or (NON_ARQ_RX_HIGHEST_SN – NON_ARQ_WINDOW_SIZE <= n < NON_ARQ_RX_WINDOW_START)

Discard the received PDU (of connection payload in cast multiplexing is used in the PDU)

Else

If (n >= NON_ARQ_RX_HIGHEST_SN) or (n < NON_ARQ_RX_HIGHEST_SN – NON_ARQ_WINDOW_SIZE)

NON_ARQ_RX_HIGHEST_SN = n + 1

Reassemble all PDUs with SN >= NON_ARQ_RX_HIGHEST_SN or SN < NON_ARQ_RX_HIGHEST_SN – NON_ARQ_WINDOW_SIZE and deliver to upper layer in order of normalized SN, if not delivered before.

If NON_ARQ_RX_WINDOW_START >= NON_ARQ_RX_HIGHEST_SN or ARQ_RX_WINDOW_START  < NON_ARQ_RX_HIGHEST_SN – NON_ARQ_WINDOW_SIZE

ARQ_RX_WINDOW_START = NON_ARQ_RX_HIGHEST_SN – NON_ARQ_WINDOW_SIZE

If reordering buffer includes PDU with SN == NON_ARQ_RX_WINDOW_START

NON_ARQ_RX_WINDOW_START = first SN which is larger than NON_ARQ_RX_WINDOW_START which has not been received yet.

Reassemble all PDUs with SN < NON_ARQ_RX_WINDOW_START and deliver to upper layer in order of normalized SN, if not delivered before.

If NON_ARQ_REORDERING_TIMEOUT is running

If (NON_ARQ_REORDERING_SN <= NON_ARQ_RX_WINDOW_START) or if NON_ARQ_REORDERING_SN  < (NON_ARQ_RX_HIGHEST_SN – NON_ARQ_WINDOW_SIZE) or NON_ARQ_REORDERING_SN  > NON_ARQ_RX_HIGHEST_SN

Stop NON_ARQ_REORDERING timer

If NON_ARQ_REORDERING timer is not running and NON_ARQ_RX_HIGHEST_SN > NON_ARQ_WINDOW_START

Start NON_ARQ_REORDERING timer

NON_ARQ_REORDERING_SN = NON_ARQ_RX_HIGHEST_SN

When NON_ARQ_REORDERING timer expires the receiver shall

Set NON_ARQ_WINDOW_START = first SN which is larger than or equal to  NON_ARQ_REORDERING_SN which has not been received yet.

Reassemble SDUs from PDU SN smaller than or equal to the new value of NON_ARQ_RX_WINDOW_START and deliver the complete SDUs to the upper layer in the order of SN.If NON_ARQ_RX_HIGHEST_SN > NON_ARQ_RX_WINDOWS_START

Start NON_ARQ_REORDERING timer

NON_ARQ_REORDERING_SN = NON_ARQ_RX_HIGHEST_SN

[-------------------------------------------------End of Text Proposal----------------------------------------------------]