| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on Service Specific BR Header (16.2.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jie Hui Xiangying Yang Muthaiah Venkatachalam Aran Bergman Intel Yih-Shen Chen Kelvin Chou Mediatek Inc. Vladimir Yanover Alvarion | E-mail: jie.hui@intel.com yihshen.chen@mediatek.com kelvin.chou@mediatek.com vladimir.yanover@alvarion.com |
| Re: | Call for LB #30b on “ P802.16m/D4”: Target topic: “16.2.2” |  |
| Abstract | This contribution proposes clarification text for Service Specific BR header |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on Service Specific BR Header (16.2.2)

Jie Hui, Xiangying Yang, Muthaiah Venkatachalam, Aran Bergman

Intel Corp

Yih-Shen Chen, Kelvin Chou

Mediatek Inc.

Vladimir Yanover

Alvarion

- Introduction

The current term of “implicit adaptation” and “explicit adaptation” in aGP service is a bit confusing since AMS still can send explicit signaling in the “implicit adaptation” case. To better reflect the operation, we propose to rename them to “ABS-initiated adaption” and “AMS-initiated adaptation”.

In order to support the Adaptation Request and Response, a new bit “Adaptation Response Indication” is introduced in the Service Specific BR header for aGP service scheduling type. If the “Adaptation Response Indication” bit is zero, it indicates an Adaptation Request from AMS, otherwise, it is an Adaptation Response from ABS. A similar Request/Response mechanism is also supported in the Sleep Control Header.

Because the Service Specific BR header now can be sent either by AMS or by ABS, it is not proper anymore to call it a BR header. Therefore, we propose to rename it to “Service Specific Scheduling Control Header” to better reflect its use.

- Proposed Test

--------------------------------text start-----------------------------------------

16.2.2.1.3 MAC Signaling Header

Table 656 – Type field encodings for MAC signaling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR with STID |
| 0001 | BR without STID |
| 0010 | Service Sspecific Scheduling Control BR without STID Header |
| 0011 | Sleep Control |
| 0100 | AMS Battery Level Report |
| 0101- 1111 | Reserved |

[Note: once the rename of Service Specific BR header is accepted, it needs to be changed throughout the standard.]

16.2.2.1.3.3 Service Specific Scheduling Control BR without STID Header

Service Specific Scheduling Control BR without STID Header is sent either by AMS through dedicated UL resource assigned to the AMS or by ABS, which needs to change the scheduling parameters of its service flow. If the Service Specific Scheduling Control Header is sent from AMS, BR type shall be included to indicate whether BR type is increment or aggregate, and it indicates Adaptation Request when it is sent for aGP service flow. The ABS may acknowledge or modify the QoS parameters for aGP service flow by sending the Service Specific Scheduling Control Header in downlink, which indicates Adaptation Response. Service Specific Scheduling Control BR without STID header format is defined in Table 659. If SCID change indicator is set to 1, the ABS shall send an acknowledgement to confirm the change of sleep mode configuration.

Table 659—Service Specific BR without STID Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Service Sspecific Scheduling Control BR without STID Hheader() {. |  |  |
| … |  |  |
| if (scheduling type == aGP Service) { |  |  |
| QoS parameter change indicator | 1 | QoS parameter change indicator 0: no change in QoS parameter 1: having changes in QoS parameter |
| SCID change indicator | 1 |  |
| if (QoS parameter change indicator = 1){ |  |  |
| Running Grant Polling Interval (GPI) | 6 | Indicating new GPI (frames) to use for future allocation. |
| Reserved | 9 |  |
| }else{ |  |  |
| QoS parameter set switch | 1 | 0: primary QoS parameter set. 1: secondary QoS parameter set |
| … |  |  |
| } |  |  |

--------------------------------text end-----------------------------------------