| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Enhanced LBS Support (16.8.3) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Lei Zhou, Fangmin Xu, Xufeng Zheng Beijing Samsung Telecom R&D Center Sangheon Kim, Brian Shim, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D4 |  |
| Abstract | This contribution proposes solution of E-LBS zone to enhance performance of LBS positioning |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Capability Negotiation for Location Based Service (16.8.3)

Lei Zhou, Fangmin Xu, Xufeng Zheng

Beijing Samsung Telecom R&D Center

Sangheon Kim, Brian Shim, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes solution of E-LBS zone to enhance performance of LBS positioning based on 802.16m Draft Amendment, IEEE P802.16m/D4, E-LBS zone need be supported in order to enhance performance of LBS positioning. At the same time, we also consider effect of E-LBS zone on HARQ timing and can’t affect current structure of subframe (5,6,7 symbols) and pilot pattern. Structure of localized E-LBS zone are proposed. The detail structure of E-LBS zone is as follows in the proposed text in AWD.

The detailed simulation parameter is shown in table 1. The simulation scenario is shown in Figure 1. Simulation result on proposed E-LBS zone with punctured rate is shown in Figure 2. Simulations result shows that the punctured subcarriers based on A-MAP won’t affect performance of LBS and the performance of proposed E-LBS zone is satisfied with E911.

| Scenario/Parameters | Baseline |
| --- | --- |
| Cells /Sectors | 19/3 |
| Frequency reuse | 3 |
| Reference signal | SA-preamble/D2 |
| Site to Site Distance | 1.5Km |
| Carrier | 2.5GHz |
| Operating Bandwidth | 10MHz |
| BS & MS Height | 32m&1.5m |
| BS &MS Power | 46dBm&23dBm |
| Pass Loss Model | Loss(dB)=130.19+37.6log10( R ) |
| Lognormal Shadowing Std.Dev | 8dB |
| Channel | PB |
| BS&MS Antenna Gain | Refer to LBS simulation Assumption doc |
| BS&MS Cable Loss | Refer to LBS simulation Assumption doc |
| Noise Figure | Refer to LBS simulation Assumption doc |

Table 1 Simulation parameter

Figure 1 Simulation scenario on 19 cells

Figure 2 Simulation result on proposed E-LBS zone with punctured rate

The Proposed Text in AWD

-------------------------------  Text Start  ---------------------------------------------------

-------------------------------  Proposal 1 Start -----------------------------------------------

[adopt the following text modification started from line# 62 Page 745 in P802.16m/D4]

16.8.3 Enhanced LBS Support

The Enhanced LBS (ELBS) is an optional LBS capability involving the following functions:

•ABS ability to transmit special ELBS waveforms to be used for location specific measurements to enable more accurate location determination.

•ABS’s ability to signal to the AMS the location of the special ELBS waveforms.

•AMS’s ability to detect, measure and report relevant measurements to the ABS.

16.8.3.1 Position of D-LBS zone in Frame Structure

The Enhanced LBS (ELBS) is an optional LBS capability. Example of the advanced air interface basic frame structure with E-LBS zone is illustrated in Figure NNN. Figure NNN shows the period of the E-LBS-zone is L superframes and The E-LBS zone transmission period is measured in superframe units. E-LBS zone consists of 5 continuous dedicated OFDM symbols and E-LBS zone locates in the first subframe of the last frame of the superframe. In the subframe with LBS-zone, A-MAP region and LBS-zone are included and frequency division multiplexing (FDM) is used for distinguishing between A-MAP region and LBS-zone.

Figure NNN Advanced air interface basic frame structure with E-LBS-zone

16.8.3.2 Transmission structure of Reference signal in E-LBS-zone

The D-LBS zone transmission plan specifies that the punctured SA-Preambles as reference signal of E-LBS-zone shall be transmitted . To define transmission plan the existing set of SA-Preambles shall be partitioned into 15 reference signals location (RSs). Relationship between IDcell and reference signals location RSidx shall be defined by equation (1) and equation (2) below:

RSidx=mod(mod(IDcell,256),q)  (1)

IDcell=256n+Idx                        (2)

where, q is total number of reference signal location and is set for 15;RSidx is the index of reference signals location that spans the following set of values [0 : 14], n is the index of the SA-Preamble carrier-set 0, 1 and 2 representing segment ID, Idx=2mod(q,128)+q128,q is a running index 0 to 255. 3 sectors/segments belonging to the same ABS shall use the same reference signals location to transmit reference signal ; in other word, one cell including 3 sectors/segments occupies one of 15 reference signal locations/resources.

The Predefined D-LBS zone transmission structure is specified by  and is shown in Figure NN.

Figure NN Predefined D-LBS zone transmission Structure

Table NNN. Predefined D-LBS zone transmission Structure.

| Allocated Frequency resource | D-LBS zone symbol number |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 |
| Frequency Resource 0 | RS 0 | RS 3 | RS 6 | RS 9 | RS12 |
| Frequency Resource 1 | RS 1 | RS4 | RS 7 | RS 10 | RS13 |
| Frequency Resource 2 | RS 2 | RS5 | RS 8 | RS 11 | RS14 |

---------------------------------------Proposal 1 End -----------------------------------------------

----------------------------------------  Text End  ------------------------------------------------