| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message Cleanup for AAI_SCD and AAI_ULPC_NI |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Joey Chou，Rongzhen Yang Intel Dongcheol Kim,Wookbong Lee LG Electronics | E-mail: joey.chou@intel.com， rongzhen.yang@intel.com Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com, wookbong.lee@lge.com |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the cleanup version for message AAI_SCD and AAI_ULPC_NI. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Message Cleanup for AAI_SCD and AAI_ULPC_NI

- Joey Chou, Rongzhen Yang 
Intel

- Dongcheol Kim,Wookbong Lee

- LG Electronics

- Introduction

The current text in AAI_SCD and AAI_ULPC_NI needs to be modified for message format and ASN.1 coding style.

- Proposed text

II.1 Proposed text 1

16.2.3.29 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Add a new table as shown below]

Table 707— AAI_ SCD Message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Change Configuration Change |  | 4 | ?? | N/A |
| M | BS_Restart_Count |  | 4 | ?? | N/A |
| ?? | SA_PreamblePartitionforBStype |  | ?? | ?? | ?? |
| M | DefaultTrigger_Averaging_parameter |  | 4 | 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 | N/A |
| ?? | NormalizedCINR |  | ?? | ?? | ?? |
| O | OL_MIMO_Parameters | OL-Region-ON | 1 | ?? | ?? |
| O |  | OL-Rank1-Configuration | 3 | ?? | ?? |
| O |  | SB-OL-Region-2Size | 4 | ?? | ?? |
| M | Parameters_GRA |  | ?? | ?? | ?? |
| O | periodicityOfRngChSync |  | 2 | the periodicity of ranging channel for synchronized AMSs allocation (Table 893) | This is ignored in Femtocell |
| O | cntlStartCodeOfRngChSync |  | 4 | the parameter ks controlling the start root index of ranging preamble codes for synchronized AMSs | This is ignored in Femtocell |
| O | rangingPreambleCodeSync |  | 2 | the number of codes for periodic ranging (Table 891) | This is ignored in Femtocell |
| O | UlpcDataChannelSet | gammaIotFp0 | 4 | gammaIotFp (IoT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) | The values are broadcasted when the related frequency partitions are used |
| O |  | gammaIotFp1 | 4 |  |  |
| O |  | gammaIotFp2 | 4 |  |  |
| O |  | gammaIotFp3 | 4 |  |  |
| M |  | alpha | 3 | alpha () is the factor according to the number of receive antennas at the ABS. It is 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved} | NA |
| M |  | beta | 1 | It is used to indicate disable or enable of the power de-boosting for uplink multi-stream transmission. 0: disable 1: enable | NA |
| M |  | dataSinrMin | 4 | dataSinrMin is the SINR requirement for the minimum data rate expected by ABS. SINRmin_Data has 4 bits to represent the value in dB among{ ,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} | NA |
| M |  | dataSinrMax | 4 | dataSinrMax is the maximum SINR threshold defined by ABS. SINRmmax_Data has 4 bits to represent the value in dB among {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 34, 36, 38, 40} | NA |
| M | UlpcControlChannelSet | targetHarqSinr | 4 | It is the HARQ feedback channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB | NA |
| M |  | targetSyncRangingSinr | 4 | It is the synchronized channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-9, -8.5, -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2, -1.5} dB | NA |
| M |  | targetPfbchSinr | 4 | It is the P-FBCH target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  | targetSfbchBaseSinr | 4 | It is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB. | NA |
| M |  | targetSfbchDeltaSinr | 3 | It is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23, 0.24, 0.25, 0.26} | NA |
| M |  | targetBwRequestSinr | 4 | It is the bandwidth request channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  | gammaIotSounding | 4 | It is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. | NA |
| M |  | soundingSinrMin | 4 | It is the minimum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { -4, -3.5, -3, -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5} | NA |
| M |  | soundingSinrMax | 4 | It is the maximum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20} | NA |
| ?? | T_ReTx_Interval |  | 3 | ?? | ?? |
| O | BR_Channel Configuration MIN Access Class |  | 2 | ?? | ?? |
| ?? | SoundingMultiplexingType | decimationValueD | 3 | decValueD0 (4), decValueD1 (6), decValueD2 (8), decValueD3 (9), decValueD4 (12), decValueD5 (16), decValueD6 (18), decValueD7 (36) | ?? |
| ?? |  | maxCyclicShiftIndexP | 3 | csIndexP0 (4), csIndexP1 (6), csIndexP2 (8), csIndexP3 (9), csIndexP4 (12), csIndexP5 (16), csIndexP6(18), csIndexP7(36) | ?? |
| ?? | shiftValueUForSoundingSymbol |  | 8 | ?? | ?? |
| O | RELAY_Support_Parameters | 16m_Relay_zone_AMS_allocation_indicator | 1 | ?? | ?? |

------------------------------------------------- End of proposed text I --------------------------------------------------