| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message Cleanup for AAI_SCD and AAI_ULPC_NI |  |
| Date Submitted | 2010-03-18 |  |
| Source(s) | Joey Chou，Rongzhen Yang Intel Dongcheol Kim,Wookbong Lee LG Electronics | E-mail: joey.chou@intel.com， rongzhen.yang@intel.com Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com, wookbong.lee@lge.com |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the cleanup version for message AAI_SCD and AAI_ULPC_NI. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Message Cleanup for AAI_SCD and AAI_ULPC_NI

- Joey Chou, Rongzhen Yang 
Intel

- Dongcheol Kim,Wookbong Lee

- LG Electronics

- Introduction

The current text in AAI_SCD and AAI_ULPC_NI needs to be modified for message format and ASN.1 coding style.

- Proposed text

II.1 Proposed text 1

16.2.3.29 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

------------------------------------------------- Start of proposed text I --------------------------------------------------

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration ChangeINTEGER (0 ~ 15)

BS_Restart_CountINTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultTrigger_Averagingparameter (usedc in equation (1)) :: = INTEGER (0..9)

0x0: 1

0x1: 1/2

0x2: 1/4

0x3: 1/8

0x4: 1/16

0x5: 1/32

0x6: 1/64

0x7: 1/128

0x8: 1/256

0x9: 1/512

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ONINTEGER (0..1) OPTIONAL

OL-Rank1-ConfigurationINTEGER (0..7) OPTIONAL

SB-OL-Region-2SizeINTEGER (0..15) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

-- the periodicity of ranging channel for synchronized AMSs allocation (Table 893)

-- This is ignored in Femtocell

periodicityOfRngChSync INTEGER (0..3),

-- the parameter ks controlling the start root index of ranging preamble codes for

synchronized AMSs. This is ignored in Femtocell

cntlStartCodeOfRngChSync INTEGER (0..15),

-- the number of codes for periodic ranging (Table 891). This is ignored in Femtocell

rangingPreambleCodeSync INTEGER (0..3)

ULPC_DataChannel_Parameters :: = SEQUENCE {

--

GAMMA_IOT_FP0INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3INTEGER (0..15) OPTIONAL

Alpha (a)INTEGER (0.. 7) OPTIONAL

Beta (ß)INTEGER (0..1) OPTIONAL

SINRmin_DataINTERGER (0..15) OPTIONAL

SINRmax_DataINTERGER (0..15) OPTIONAL

}

ULPC_ControlChannel_Parameters :: = SEQUENCE {

SINR_Target_HARQINTEGER (0..15) OPTIONAL

SINR_Target_SyncRangingINTEGER (0..15) OPTIONAL

SINR_Target_PFBCHINTEGER (0..15) OPTIONAL

SINR_Target_SFBCH_BASEINTEGER (0..15) OPTIONAL

SINR_Target_SFBCH_DeltaINTEGER (0..7) OPTIONAL

SINR_Target_BWRequestINTEGER (0..15) OPTIONAL

GAMMA_IOT_SOUNDINGINTEGER (0..15) OPTIONAL

SINRmin_SOUNDINGINTERGER (0..15) OPTIONAL

SINRmax_SOUNDINGINTERGER (0..15) OPTIONAL

}

T_ReTx_IntervalINTERGER (0..8)

BR_Channel Configuration MIN Access ClassINTERGER (0..4) OPTIONAL

--Sounding sequence

-- D is decimation value for frequency division multiplexing

-- P is maximum cyclic shift for code division multiplexing

SoundingMultiplexingType CHOICE {

decimationValueDENUMERATED {decValueD0 (4), decValueD1 (6),

decValueD2 (8), decValueD3 (9), decValueD4 (12),

decValueD5 (16), decValueD6 (18), decValueD7 (36)},

maxCyclicShiftIndexPENUMERATED {csIndexP0 (4), csIndexP1 (6),

csIndexP2 (8), csIndexP3 (9), csIndexP4 (12),

csIndexP5 (16), csIndexP6(18), csIndexP7(36)},

}

shiftValueUForSoundingSymbolINTEGER (0...255),

RELAY_Support_Parameters :: = SEQUENCE {

16m_Relay_zone_AMS_allocation_indicatorINTEGER (0..1) OPTIONAL

}

}

-- ASN1STOP

[Add a new table as shown below]

Table 707— AAI_ SCD Message

| M/O |  | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M |  | Change Configuration Change |  | 4 | The value is increased whenever the contents of this message is changed The value rolls over from 0 to 15 | N/A |
| M |  | BS_Restart_Count |  | 4 | The value is incremented by one whenever BS restarts The value rolls over from 0 to 15 | N/A |
| M |  | SA_PreamblePartitionforBStype |  |  | Indicates the SA-Preamble partition information | M |
| M |  | DefaultTrigger_Averaging_parameter |  | 4 | Provides default trigger averaging parameter of the BS 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 | N/A |
|  |  | NormalizedCINR |  |  |  |  |
| O |  | OL_MIMO_Parameters | OL-Region-ON | 1 | 0 or 1 | OL MIMO parameters |
| O |  |  | OL-Rank1-Configuration | 3 | 0 to 7 | OL MIMO parameters |
| O |  |  | SB-OL-Region-2Size | 4 | 0 to 15 | OL MIMO parameters |
| M |  | Parameters_GRA |  |  |  |  |
| O |  | periodicityOfRngChSync |  | 2 | the periodicity of ranging channel for synchronized AMSsthe S-RCH allocation (Table 893901) | This is ignored in Femtocell |
| O |  | cntlStartCodeOfRngChSync |  | 4 | the parameter ks controlling the start root index of ranging preamblethe RP codes for the S-RCH synchronized AMSs. | This is ignored in Femtocell |
| O |  | rangingPreambleCodeSync |  | 2 | the number of the RP codes for periodic ranging (Table 891899). This is ignored in Femtocell | This is ignored in Femtocell |
| O |  | UlpcDataChannelSet | gammaIotFp0 | 4 | gammaIotFp (IoT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) | The values are broadcasted when the related frequency partitions are used |
| O |  |  | gammaIotFp1 | 4 |  |  |
| O |  |  | gammaIotFp2 | 4 |  |  |
| O |  |  | gammaIotFp3 | 4 |  |  |
| M |  |  | Alpha | 3 | alpha () is the factor according to the number of receive antennas at the ABS. It is 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved} | NA |
| M |  |  | Beta | 1 | It is used to indicate disable or enable of the power de-boosting for uplink multi-stream transmission. 0: disable 1: enable | NA |
| M |  |  | dataSinrMin | 4 | dataSinrMin is the SINR requirement for the minimum data rate expected by ABS. SINRmin_Data has 4 bits to represent the value in dB among{-INF ,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} | NA |
| M |  |  | dataSinrMax | 4 | dataSinrMax is the maximum SINR threshold defined by ABS. SINRmmax_Data has 4 bits to represent the value in dB among {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 34, 36, 38, 40} | NA |
| M |  | UlpcControlChannelSet | targetHarqSinr | 4 | It is the HARQ feedback channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB | NA |
| M |  |  | targetSyncRangingSinr | 4 | It is the synchronized channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-9, -8.5, -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2, -1.5} dB | NA |
| M |  |  | targetPfbchSinr | 4 | It is the P-FBCH target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  |  | targetSfbchBaseSinr | 4 | It is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB. | NA |
| M |  |  | targetSfbchDeltaSinr | 3 | It is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23, 0.24, 0.25, 0.26} | NA |
| M |  |  | targetBwRequestSinr | 4 | It is the bandwidth request channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  |  | gammaIotSounding | 4 | It is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. | NA |
| M |  |  | soundingSinrMin | 4 | It is the minimum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { -4, -3.5, -3, -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5} | NA |
| M |  |  | soundingSinrMax | 4 | It is the maximum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20} | NA |
|  |  | T_ReTx_Interval |  | 3 | 0 – 8 |  |
| O |  | BR_Channel Configuration MIN Access Class |  | 3 |  |  |
| O |  | BR_Channel Configuration MIN Access Class of the (i+0)-th frame |  | 3 | INTERGER (0..4) |  |
| O |  | BR_Channel Configuration MIN Access Class of the (i+1)-th frame |  | 3 | INTERGER (0..4) |  |
| O |  | BR_Channel Configuration MIN Access Class of the (i+2)-th frame |  | 3 | INTERGER (0..4) |  |
| O |  | BR_Channel Configuration MIN Access Class of the (i+3)-th frame |  | 3 | INTERGER (0..4) |  |
| O |  | SoundingMultiplexingType | decimationValueD | 3 | decValueD0 (4), decValueD1 (6), decValueD2 (8), decValueD3 (9), decValueD4 (12), decValueD5 (16), decValueD6 (18), decValueD7 (36) | Sounding sequence: Presents when Sounding Multiplexing Type is set to Decimation_ValueD |
|  |  |  | maxCyclicShiftIndexP | 3 | csIndexP0 (4), csIndexP1 (6), csIndexP2 (8), csIndexP3 (9), csIndexP4 (12), csIndexP5 (16), csIndexP6(18), csIndexP7(36) |  |
|  |  | shiftValueUForSoundingSymbol |  | 8 | 0 – 255 |  |
| O |  | RELAY_Support_Parameters | 16m_Relay_zone_AMS_allocation_indicator | 1 | 0 – 1 | Relay support parameters |
| O |  | E-MBS Configuration Parameters | Zone_Allocation-Bit-MAP | DSACMAX-1 | See section 16.9.3.1. | Present when E-MBS is configured |
|  |  |  | ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1. |  |
|  |  |  | MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16) |  |
|  |  |  | E-MBS AAI frame offset | variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3 bits MSI length == 0b01: 4 bits MSI length == 0b10: 5 bits MSI length == 0b11: 6 bits |  |
| O | O | uLFeedbackInfo | primaryCarrierIndex | 6 | the primary carrier index | Loop of NumPC |
|  | M |  | startDLRUIndex |  | the start DLRUs index for feedback channel | Loop of NumPC |
|  | M |  | dLRUNum |  | the number of DLRUs for feedback channel | Loop of NumPC |
|  | M |  | hARQChannelNum |  | the number of HARQ feedback channel per HARQ feedback region | Loop of NumPC |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II --------------------------------------------------

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

[Add the following ASN.1 code to Annex P.2]

-- System Configuration Descriptor Message

AAI-SCD ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

configChangeCount              INTEGER (0..15),

bsRestartCount                 INTEGER (0..15),

defaultTriggerAveraging        ENUMERATED {

one        (0),

oneOver2   (1),

oneOver4   (2),

oneOver8   (3),

oneOver16  (4),

oneOver32  (5),

oneOver64  (6),

oneOver128 (7),

oneOver256 (8)},

olMimoParameters               SEQUENCE {

olRegionOn                   INTEGER (0..1) OPTIONAL,

olRank1Config                INTEGER (0..7) OPTIONAL,

sbOlRegion2Size              INTEGER (0..15) OPTIONAL},

-- the periodicity of ranging channel for synchronized AMSs

-- allocation (Table 893)

periodicityOfRngChSync         INTEGER (0..3),

-- the parameter Ks controlling the start root index of ranging

-- preamble codes for synchronized AMSs

cntlStartCodeOfRngChSync       INTEGER (0..15),

-- the number of codes for periodic ranging (Table 891), and is

-- ignored in Femtocell

rangingPreambleCodeSync 	INTEGER (0..3),

ulpcDataChannelIe              UlpcDataChannelSet,

ulpcControlChannelIe           UlpcControlChannelSet,

-- Sounding sequence

-- D is decimation value for frequency decimation multiplexing

-- P is number of codes for code division multiplexing

soundingMultiplexingType       CHOICE {

decimationValueD             ENUMERATED {

decValueD0 (6),

decValueD1 (8),

decValueD2 (9),

decValueD3 (12),

decValueD4 (16),

decValueD5 (18),

decValueD6 (36)},

maxCyclicShiftIndexP         ENUMERATED {

csIndexP0 (4),

csIndexP1 (6),

csIndexP2 (8),

csIndexP3 (9),

csIndexP4 (12),

csIndexP5 (16),

csIndexP6 (18),

csIndexP7 (36)}

},

shiftValueUForSoundingSymbol   INTEGER (0..255),

relaySupportParameter          SEQUENCE {

relayZoneAmsAlocIndc         INTEGER (0..1) OPTIONAL}

}

UlpcDataChannelSet ::= SEQUENCE {

gammaIotFp0                    INTEGER (0..15) OPTIONAL,

gammaIotFp1                    INTEGER (0..15) OPTIONAL,

gammaIotFp2                    INTEGER (0..15) OPTIONAL,

gammaIotFp3                    INTEGER (0..15) OPTIONAL,

alpha                          INTEGER (0..7),

beta                           INTEGER (0..1),

dataSinrMin                    INTEGER (0..15),

dataSinrMax                    INTEGER (0..15)}

UlpcControlChannelSet ::= SEQUENCE {

targetHarqSinr                 INTEGER (0..15),

targetSyncRangingSinr          INTEGER (0..15),

targetPfbchSinr                INTEGER (0..15),

targetSfbchBaseSinr            INTEGER (0..15),

targetSfbchDeltaSinr           INTEGER (0..7),

targetBwRequestSinr            INTEGER (0..15),

gammaIotSounding               INTEGER (0..15),

soundingSinrMin                INTEGER (0..15),

soundingSinrMax                INTEGER (0..15)}

------------------------------------------------- End of proposed text II --------------------------------------------------

II.3 Proposed text 3

------------------------------------------------- Start of proposed text III --------------------------------------------------

16.2.3.30 AAI_UL Noise and Interference Level Broadcast Message

An AAI_ULPC_NI broadcast NI values for the UL power control.

-- ASN1START

AAI_ULPC_NIMessage ::= SEQUENCE {

Table 707—ULPC_DataChannel_Parameters Field Descriptions

| Parameters | Description |
| --- | --- |
| GAMMA_IOT_FP0, GAMMA_IOT_FP1, GAMMA_IOT_FP2, GAMMA_IOT_FP3 | GAMMA_IOT ( ) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) |
| Alpha ( ) | is the factor according to the number of receive antennas at the ABS. It is 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved} |
| Beta( ) | is set to be 0 or 1 by one bit, which is used to indicate disable or enable of the power de-boosting for uplink multi-stream transmission. |
| SINRmin_Data | SINRmin_Data is the SINR requirement for the minimum data rate expected by ABS. SINRmin_Data has 4 bits to represent the value in dB among{ ,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} |
| SINRmax_Data | SINRmax_Data is the maximum SINR threshold defined by ABS. SINRmmax_Data has 4 bits to represent the value in dB among {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 34, 36, 38, 40} |

UL_NI_Information:: = SEQUENCE {

IOT_Sounding	            INTEGER (0..127) OPTIONAL,

IOT_FP0			INTEGER (0..127) OPTIONAL,

IOT_FP1			INTEGER (0..127) OPTIONAL,

IOT_FP2			INTEGER (0..127) OPTIONAL,

IOT_FP3			INTEGER (0..127) OPTIONAL

Table 708 —AAI_ULPC_NI Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | UlNiInfoSet | iotSounding | 7 | IoT value of sounding, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. | Present when sounding is used |
| O |  | iotFP0 | 7 | IoT value of Frequency Partition #0, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. | Present when FP0 is used |
| O |  | iotFP1 | 7 | IoT value of Frequency Partition #1, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. | Present when FP1 is used |
| O |  | iotFP2 | 7 | IoT value of Frequency Partition #2, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. | Present when FP2 is used |
| O |  | iotFP3 | 7 | IoT value of Frequency Partition #3, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. | Present when FP3 is used |

For the open-loop power control, UL noise and interference level shall be broadcast to AMSs in the given ABS coverage by ABS. All the UL noise and interference noise level are quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB.

The absolute power level of NI is transformed from IoT level as:

NI=PTN+IoT+10log10(Δf)

Where,

PTN is the thermal noise power density in 00C

Δf is the subcarrier spacing.

| Parameters | Description |
| --- | --- |
| SINR_Target_HARQ | SINR_Target_HARQ is the HARQ feedback channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB |
| SINR_Target_SyncRanging | SINR_Target_SyncRanging is the synchronized channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-9, -8.5, -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2, -1.5} dB |
| SINR_Target_PFBCH | SINR_Target_PFBCH is the P-FBCH target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB |
| SINR_Target_SFBCH_BASE | SINR_Target_SFBCH_BASE is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB. |
| SINR_Target_SFBCH_Delta | SINR_Target_SFBCH_Delta is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23, 0.24, 0.25, 0.26} |
| SINR_Target_BWRequest | SINR_Target_BWRequest is the bandwidth request channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB |
| GAMMA_IOT_SOUNDING | GAMMA_IOT_SOUNDING is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. |
| SINRmin_SOUNDING | SINRmin_SOUNDING is the minimum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { -4, -3.5, -3, -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5} |
| SINRmax_SOUNDING | SINRmax_SOUNDING is the maximum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20} |

------------------------------------------------- End of proposed text III --------------------------------------------------

II.4 Proposed text 4

------------------------------------------------- Start of proposed text IV --------------------------------------------------

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

[Add the following ASN.1 code to Annex P.2]

AAI-ULPC-NI ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

UlNiInfoSet ::= SEQUENCE {

iotSounding                    INTEGER (0..127) OPTIONAL,

iotFP0                         INTEGER (0..127) OPTIONAL,

iotFP1                         INTEGER (0..127) OPTIONAL,

iotFP2                         INTEGER (0..127) OPTIONAL,

iotFP3                         INTEGER (0..127) OPTIONAL}

}

------------------------------------------------- End of proposed text I --------------------------------------------------