| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN (non security aspects) (Section 16.2.3.1) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty, Avishay Shraga, Pouya Taaghol, Jose Puthenkulam Masoud Olfat, David McGinnis, Chunmei Liu Shailesh Awasthy, Vinod Bhatia Oleg Marinchenco, Daniel Cohn, Vladimir Yanover Kiran Kuchi, Klutto Milleth Yih-Shen Chen, I-Kang Fu and Kelvin Chou Kalyan Pal, Raja Srinivas, BP Tiwari, Neeraj Sonker, Deepak Mittal Phillip Barber, Ronald Mao Ken Falkenstein, Asan Khan, Dave Urban Ming Hung Thao Stavros Tzavidas, Joe Schumacher, Shahab Sayeedi Rakesh Taori, Hyunjeong Kang, Yongkyo Baek, Jungshin Park Yang Liu | Intel Corporation, Clearwire, Reliance Communications, Alvarion, CeWIT, MediaTek, Tata Communications, Huawei, Comcast, ITRI, Motorola, Samsung, ZTE |
| Re: | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN |  |
| Abstract | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN

Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty, Avishay Shraga, Pouya Taaghol, Jose Puthenkulam, Intel

Masoud Olfat, David McGinnis, Chunmei Liu, Clearwire

Shailesh Awasthy, Vinod Bhatia, Reliance Communications

Oleg Marinchenco, Daniel Cohn, Vladimir Yanover, Alvarion

Kiran Kuchi, Klutto Milleth, CeWIT

Yih-Shen Chen, I-Kang Fu and Kelvin Chou, MediaTek

Kalyan Pal, Raja Srinivas, BP Tiwari, Neeraj Sonker, Deepak Mittal, Tata Communications

Phillip Barber, Ronald Mao, Huawei

Ken Falkenstein, Asan Khan, Dave Urban, Comcast

Ming Hung Thao, ITRI

Stavros Tzavidas, Joe Schumacher, Shahab Sayeedi, Motorola

Rakesh Taori, Hyunjeong Kang, Yongkyo Baek, Jungshin Park, Samsung

Yang Liu, ZTE

# Introduction

Operators have WiMAX 1.0/1.5 networks in deployment today. This uses 802.16e air interface. 802.16m air interface promises much higher throughputs compared to 802.16e. Some more improvements are achieved in 16m by changing UMAC functionality that affects the ASN-GW and in particular R6 interface. Expecting the operators and vendors to upgrade the ASN-GW in order to start deploying 16m BSs will slow down the 16m adoption. The goal is to standardize a mechanism that allows operators to upgrade to 802.16m from 802.16e in phases where the first phase is using 16m BS without any changes to the existing network infrastructure and in particular no change on R6. No changes to ASN-GW and other network elements

In summary, we enable a model where 16m BSs can be simply plugged into the existing WiMAX ASN-GWs.

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Change-1 Add the following to the overview section and text to 802.16m draft]

16.10.3 Migrating to Advanced Air Interface without impacting the deployed legacy network

The migration to WirelessMAN OFDMA Advanced Air Interface may be done without impacting the deployed legacy network elements. The ABS should be able to connect to legacy access and core network elements. If the ABS is connected to legacy network, the ABS shall communicate to the AMSs that it is attached to the legacy network and the AMSs shall function in accordance to legacy network requirements.

Some examples include:

- AMS privacy via AMSID* shall not be used. AMS provides actual MAC address in the AAI_RNG-REQ message for network entry/re-entry and idle mode location update. ABS provides the hash of the actual MAC address in the AAI_PAG-ADV message.

- Features such as DCR, multiple paging groups per AMS shall not be supported.

[Change-2:  (removed. Editor to ignore)]

[Change-3: Add to the end of the Paragraph in Page 295, line 58, section 16.2.15.3 as follows:]

16.2.15.3 Initial ranging and automatic adjustments

In the legacy ASN mode, where the ABS is connected to the legacy ASN, the AMS provides its actual MAC address in the AAI_RNG-REQ message.

[Change-4: Modify section 16.2.17.4.2 by addition the following to the end of this subsection:]

16.2.17.4.2 Location update process

In the legacy ASN mode, where the ABS is connected to the legacy ASN, the AMS provides its actual MAC address in the AAI_RNG-REQ message, instead of providing the DID.

[Change-5: Modify section 16.2.17.5 by addition the following to the end of this subsection:]

16.2.17.5 Network reentry from idle mode

In the legacy ASN mode, where the ABS is connected to the legacy ASN, the AMS provides its actual MAC address in the AAI_RNG-REQ message, instead of providing the DID.

[Change-6: Modify Table 676 as follows:]

16.2.3.1 AAI_RNG-REQ

Table 676—Parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| Legacy ASN indicator | 1 bit Value of “0” means ABS is connected to advanced ASN. Value of “1” means ABS is connected to legacy ASN. | Indicates whether the target ABS is connected to legacy ASN or advanced ASN. |
| AMSID*/ MAC address | It's the hash value of AMSID in order to protect AMS privacy, which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. In the legacy ASN mode, where the ABS is connected to the legacy ASN, the AMS provides its actual MAC address instead. | It shall be included when the AMS is attempting network entry without its STID/DID which the ABS/Paging Controller assigns. |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |

…

| Paging Controller ID | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall To be included when the AMS is attempting to perform reentry or location update. In the legacy ASN mode, where the ABS is connected to the legacy ASN, DID shall not be included |
| --- | --- | --- |
| Deregistration Identifier (DID) | The ID which the AMS is assigned for idle mode and currently maintains. |  |
| PGID | The identification of the paging group that the AMS is previously belonging to. |  |
| Paging Cycle | PAGING_CYCLE applied to the AMS |  |
| Paging Offset | PAGING_OFFSET applied to the AMS |  |

[Change-7: Modify Table 677 as follows:]

16.2.3.2 AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Status | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success | It shall be included in the AAI_RNG-RSP message |
| Temporary STID | Used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID/DID yet. |
| AMSID*/MAC address | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. In the legacy ASN mode, where the ABS is connected to the legacy ASN, the actual MAC address of the AMS is used instead. |  |

[Change-8:  Add legacy ASN indication in HO CMD as a parameter. Add the following row at the end of Table 686 as follows.]

16.2.3.11 AAI_HO-CMD

| Name | Value | Usage |
| --- | --- | --- |
| Legacy ASN indicator | 1 bit Value of “0” means ABS is connected to advanced ASN. Value of “1” means ABS is connected to legacy ASN. | Indicates whether the target ABS is connected to legacy ASN or advanced ASN. |

[Change-9: Add the following at the end of section 16.2.6.3.4:]

16.2.6.3.4 HO Execution

If the target ABS is attached to the legacy ASN, this shall be indicated to the AMS in the HO-CMD with the “Legacy ASN indicator” parameter. A value of 1 for this parameter signifies to the AMS that the target ABS is attached to the legacy ASN.

[Ed Note: There is an opinion that Change-8 and Change-9 may not be needed for legacy ASN operation. If consensus is reached on this, Change-8, Change-9 will be removed from this doc]

[Change-10: Add legacy ASN indication in AAI_PAG-ADV as a parameter. Add the text to the end of Section 16.2.17.2.1; and modify Table 767 as follows]

16.2.17.2.1 Broadcast paging message

If the ABS is attached to the legacy ASN, this shall be indicated to the AMS in the AAI_PAG-ADV with the “Legacy ASN indicator” parameter. A value of 1 for this parameter signifies to the AMS that the ABS is attached to the legacy ASN. Under this scenario, the AMS index as used in 802.16-2009 shall be used for paging the AMS.

[Add the following to the end of table 767]

| Name | Value | Usage |
| --- | --- | --- |
| Legacy ASN indicator | 1 bit Value of “0” means ABS is connected to advanced ASN. Value of “1” means ABS is connected to legacy ASN. | Indicates whether the ABS is connected to legacy ASN or advanced ASN. If attached to the legacy ASN, AMS index as used in 802.16-2009 shall be used for paging the AMS (instead of DID) |

[Ed Note: There is an opinion that Change-10 may not be needed for legacy ASN operation. If consensus is reached on this, Change-10 will be removed from this doc]

[-------------------------------------------------End of Text Proposal----------------------------------------------------]