| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |
| --- | --- |
| Title | Interference Avoidance and Mitigation to Support Femtocells (16.4.11) |
| Date Submitted | 2010-03-16 |
| Source(s) | Ying Li, Ji-Yun Seol, Young Bo Cho, Zhouyue Pi, Anshuman Nigam, Jung Je Son yli2@sta.samung.com Samsung Electronics Jin Lee, Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Youngsoo Yuk, HanGyu Cho, Jin Sam Kwak [jins978, cooper, ksryu, ronnykim, hgcho, sixs, samji] @lge.com LG Electronics, Inc. Yung-Han Chen, Yan-Xiu Zheng, Chun-Yuan Chiu chenyunghan@itri.org.tw ITRI Linghang Fan, Hassan Al-kanani, Nader Zein, Andreas Maeder, Tetsu Ikeda [Linghang.fan, nader.zein, hassan.alkanani] @eu.nec.com andreas.maeder@nw.neclab.eu, t-ikeda@ap.jp.nec.com NEC I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng IK.Fu@mediatek.com MediaTek Inc. |
| Re: | IEEE 802.16 Working Group Letter Ballot#31 |
| Abstract | This contribution is to propose text changes for 16.4.11. |
| Purpose | To be discussed and adopted by WG LB |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |

# Interference Avoidance and Mitigation to Support Femto ABS (16.4.11)

Ying Li, Ji-Yun Seol, Young Bo Cho, Zhouyue Pi, Anshuman Nigam, Jung Je Son

yli2@sta.samung.com
Samsung Electronics

Jin Lee, Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Youngsoo Yuk, HanGyu Cho, Jin Sam Kwak

[jins978, cooper, ksryu, ronnykim, hgcho, sixs, samji] @lge.com

LG Electronics, Inc.

Yung-Han Chen, Yan-Xiu Zheng, Chun-Yuan Chiu

chenyunghan@itri.org.tw

ITRI

Linghang Fan, Hassan Al-kanani, Nader Zein, Andreas Maeder, Tetsu Ikeda

[Linghang.fan, nader.zein, hassan.alkanani] @eu.nec.com

andreas.maeder@nw.neclab.eu,

t-ikeda@ap.jp.nec.com

NEC

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

IK.Fu@mediatek.com

MediaTek Inc.

# Introduction

The section of interference avoidance and interference mitigation needs to be clarified and cleaned up.

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.4.11 Interference Avoidance and Interference Mitigation

Large interference from an inaccessible Femto ABS may trigger a nearby AMS to report the interference to the serving ABS, and the report information should include system information of the inaccessible Femto ABS (e.g., BS_ID of the Femto ABS). The inaccessible ABS includes the CSG-closed Femto ABS and the Femto ABS that AMS unable to enter due to mobility.

An AMS may initiate or be requested to report the signal strength measurements of its neighbor ABSs to its serving ABS. The reported information can be used by the serving ABS to coordinate interference mitigation (e.g. reducing transmission power and/or blocking resource regions) with its neighbor ABSs. Upon receiving the interference coordination sent by the serving ABS, the interfering ABS may perform interference mitigation (e.g. reducing transmission power and/or blocking resource regions.

If the AMS is connected to macro ABS and the AMS is interfered by a CSG-Closed Femto ABS where it is not a member, it may indicate this problem to the macro ABS by reporting the signal strength measurement of its neighbor ABSs.

If an AMS is placed into outage by an inaccessible ABS (e.g. the CSG-closed Femto ABS of which it is not a member) and only if the AMS has no connection with neighbor macro ABS, it can may indicate this problem to that Femto ABS by sending an AAI_RNG-REQ with the “Femto Interference” bit set to 1 based on configured trigger conditions.

Upon inaccessible Femto ABS receiving an AAI_RNG-REQ with the "Femto Interference" bit set to 1 from AMS, it continues the network entry procedure. The Femto ABS may perform interference mitigation based on the measurement and legitimate of the AMS. The Femto ABS may either reduce its transmit power locally or communicate with the network entity to cooperate on the interference mitigation process.

Upon communication with the network entity, the CSG-Closed Femto ABS may convert to a CSG-Open Femto ABS or reduce its transmit power as directed by the network entity or refrain from transmitting on certain resource regions as indicated by the network entity.

Upon receiving an AAI_RNG-REQ with the “Femto Interference” bit set to 1, and if there are no AMSs attached to the CSG-Closed Femto ABS and there are no AMSs in network (re)entry process, the CSG-Closed Femto ABS may operate in LDM for a time interval to reduce interference. "

The interference between Femto and/or macro can be mitigated by static or semi-static radio resource reservation and resource sharing using FDM and/or TDM manner and/or DL power control. The operation of resource reservation shall not contradict with the FFR operation defined in 20.1. Femto ABSs shall also utilize FFR partitions in the DL and UL using the same signaling and procedures as used by the macro ABSs. FFR partitions used by the Femto ABSs may be different in terms of size, subchannel assignment, and transmit power level than those used on macro ABSs. One or more FFR partitions may be used as the radio resource region where Femto ABSs are not allowed to transmit. The blocked region size may increase if the number of AMSs interfered by Femto ABS becomes larger; and the blocked region size may decrease if the number of AMSs interfered by Femto ABS becomes smaller. A Femto ABS may detect and reserve the resources autonomously, or in cooperation with the overlay macro ABS.

A Femto ABS may select the carrier frequency, or frequency partition to avoid the mutual interference between macro/micro and Femto ABSs or among Femto ABSs based on the measurement result of surrounding reception power.

Femto ABS may measure the signal strength for the carrier frequency, or frequency partition of the neighbor macro/micro/femto cells. In addition, the Femto ABS may receive A-Preamble from the neighbor macro/micro/femto cells and obtain information on cell type. The Femto ABS may select its carrier frequency, or frequency partition based on signal strength and the cell type information of its neighbors.

AMS sends AAI_RNG-REQ with the "Femto Interference" bit set to 1 based on configured trigger conditions.

Upon CSG-Closed Femto ABS receiving an AAI_RNG-REQ with the "Femto Interference" bit set to 1, it may either reduce its transmit power locally or communicate with the network entity.

Upon communication with the network entity, the CSG-Closed Femto ABS may convert to a CSG-Open Femto ABS or reduce its transmit power as directed by the network entity or refrain from transmitting on certain resource regions as indicated by the network entity.

Semi-static/dynamic interference mitigation (IM) may have lower priority than HO.

AMS shall follow the scanning procedure for HO first. The AMS may scan inaccessible ABS blindly or based on the instruction. The serving ABS may request AMS to report the scanning result of inaccessible ABS via AAI_SCN-REP. Even if not requested, the AMS can report inaccessible ABS to the serving ABS via AAI_SCN-REP on trigger defined in Table xxx. The serving ABS may initiate IM with the interfering femto ABS via backhaul.

Non-member AMS may send AAI_RNG-REQ on the trigger defined in Table xxx, to the inaccessible CSG femto ABS, with a Femto Interference indicator to indicate the AMS is in a coverage hole generated by the femto ABS.

The serving ABS may send the interfering BS_ID(s) for which the resource reservation is coordinated via AAI_SCN-RSP to the AMS and it may request the AMS to scan the femto ABS performed coordinated IM via unsolicited AAI_SCN-RSP. The scan can be periodical. AMS should scan and report the scanning result to the serving ABS via AAI_SCN-REP, if certain trigger conditions are met, as defined in Table xxx. The report may be sent periodically. The serving ABS may terminate IM with the interfering femto ABS via backhaul.

16.4.11.1 Trigger condition definitions

The ABS may define trigger conditions for the following actions:

- Conditions that define when the AMS should report scanning measurement results of the inaccessible femto ABS to the serving ABS for IM initiation

- Conditions that define when the AMS should report scanning measurement results of the inaccessible femto ABS to the serving ABS for IM termination

- Conditions that define when non-member AMS should initiate signaling to CSG femto about the coverage hole due to interference of the CSG femto, by sending AAI_RNG-REQ.

- Conditions that define when the AMS should start scanning procedure for inaccessible femto ABS for IM termination

The trigger TLV (type xx) in Table xxx is encoded using the description in Table XX. The trigger TLV for another ABS type (which includes ABS type parameter: such as macro, macro hot-zone, Femto and relay, etc) is encoding using the description in Table XX

Table xxx—Trigger TLV Description

| Name | Type | Length (Bytes) | Value |
| --- | --- | --- | --- |
| Type/Function/Action | xx.1 | 1 | See Table XX—for description |
| Trigger Value | xx.2 | 1 | Trigger value is the value used in comparing measured metric for determining a trigger condition |
| Trigger averaging duration | xx.3 | 1 | Trigger averaging duration is the time measured in number of frames over which the metric measurements are averaged. |

Table XX—Trigger; Type/Function/Action Description

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Type | 2(MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric 0x2: RTD metric 0x3: Number of missed superframes metric |
| Function | 3 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor inaccessible CSG femto ABS is greater than absolute value 0x2: Metric of neighbor inaccessible CSG femto ABS is less than absolute value 0x3: Metric of neighbor inaccessible CSG femto ABS is greater than serving ABS metric by relative value 0x4: Metric of neighbor inaccessible CSG femto ABS is less than serving ABS metric by relative value 0x5: Metric of serving ABS greater than absolute value 0x6: Metric of serving ABS less than absolute value 0x7: Reserved NOTE-0x1-0x4 not applicable for RTD trigger metric NOTE-When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from -32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5 or 0x6, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as -103.75 dBm and 0xFF is interpreted as -40 dBm NOTE-Type 0x3 can only be used together with function 0x5 or function 0x6 |
| Action | 3(LSB) | Action performed upon reaching trigger condition: 0x0: Reserved 0x1: Respond on trigger with AAI_SCN-REP to initiate IM 0x2 : Respond on trigger with AAI_SCN-REP to terminate IM 0x3: Respond on trigger with AAI_RNG-REQ with ‘Femto Interference Indicator’. 0x4: Respond on trigger with AMS scanning to terminate IM 0x5, 0x6 and 0x7: Reserved NOTE -0x2, 0x4 are only applicable after IM is initiated. |

The ABS may define complex trigger conditions by including multiple Trigger TLV encodings in the same compound TLV. In this case, all included triggers shall have the same Action code (as defined in Table XX). The AMS shall perform a logical AND of all the conditions in a complex trigger condition and invoke the action of the trigger only when all trigger conditions are met.

Whenever the condition of a simple trigger or all the conditions of a complex trigger are met, the AMS shall invoke the action of the trigger. If multiple trigger conditions are met simultaneously the AMS shall invoke the action of at least one of the triggers for which the trigger condition was met.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------