| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_GRP-CFG message cleanup |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text of AAI_L2-XFER message clean-up |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- AAI_GRP-CFG message clean-up

- Joey Chou

- Intel

- Introduction

This contribution cleans up AAI_GRP-CFG message.

- Proposed text

1. 	Proposed Text I

16.2.3.48 Group Configuration MAC Control Message (AAI_GRP-CFG)

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Add a new Table xxx as the following]

Table xxx: AAI_GRP-CFG

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M | Message type |  |  | 8 |  |  |
| M | Deletion Flag |  |  | 1 | Flag to signal whether this message includes addition or deletion information. 0: Flow is added to a group 1: Flow is explicitly deleted from a group |  |
| M | Flow ID |  |  | 4 | Identifies the flow that is added to group for GRA |  |
| O | GRA Info for added flow | groupId |  | 12 | ID of the group to which the flow is added | Present if Deletion Flag == 0 |
|  |  | Long TTI Indicator |  | 1 | Defines number of AAI subframes spanned by the allocated resource. 0: 1 AAI subframe (default) 1: 4 DL/UL AAI subframes for FDD or all DL/UL AAI subframe for TDD |  |
|  |  | LSB Nrb BSID |  | 16 x number of Nrbs | 16-bit LSB of unique 48-bit identifier of neighbor BS |  |
|  |  | Periodicity |  | 2 | Periodicity of allocation of this group. This specifies the periodicity with which the corresponding GRA A-MAP IE will be transmitted. 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |  |
|  |  | MIMO mode set |  |  | Signals the 2-bit MIMO mode set corresponding to this group |  |
|  |  | User Bitmap Size |  | 2 | The size of user bitmap in bits. The size remains fixed for a group. The size determines the maximum number of flows that can be supported in one group. 0: 4 1: 8 2: mode 2 3: mode 2 and 4 |  |
|  |  | User Bitmap Index |  | 5 | Signals index of the flow in group's user bitmap |  |
|  |  | Initial_ACID |  | 4 | Signals the starting ACID of the range of ACIDs assigned to the GRA flow |  |
|  |  | N_ACIDs |  | 3 | The number of ACIDs assigned to the GRA flow |  |
|  |  | Burst Size Indicator (b) |  | 1 | Signals whether the HARQ burst sizes supported in the group are included in this message or are same as the last GRA allocation for this flow. 0: Burst sizes same as last GRA allocation for this flow, hence not included in the message 1: Burst sizes explicitly included in the message |  |
|  |  | Resource size indicator (r) |  | 1 | Signals whether the resource sizes supported in the group are included in this message or are same as the previous group 0: Resource sizes same as last GRA allocation for this flow, hence not included in the message 1: Resource sizes explicitly included in the message |  |
|  |  | Array [1..4] | Burst size i | 5 | ith burst size of the 4 burst sizes supported in the group | Present if b == 1 |
|  |  |  | Resource Bitmap Flag (f) | 1 | 0b0: The resource size inclusion bitmap for this burst size is the same as the bitmap defined for the (i-1) th burst size. 0b1: Define a resource size inclusion bitmap for the i-th burst size | Present if r == 1 |
|  |  |  | Resource size inclusion bitmap | 16 | Bitmap to signal which 8 resource sizes are supported in the group out of the range of [1,16] LRUs supported for GRA. If nth bit in the bitmap is set to 1, it signals that a resource size of n LRUs is supported in the group. | Present if f == 1 |

------------------------------------------------- End of proposed text I --------------------------------------------------