| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_GRP-CFG message cleanup |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Joey Chou Shweta Shrivastava Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text of AAI_GRP-CFG message clean-up |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- AAI_GRP-CFG message clean-up

- Joey Chou, Shweta Shrivastava

- Intel

- Introduction

This contribution cleans up AAI_GRP-CFG message.

- Proposed text

1. 	Proposed Text I

16.2.3.48 Group Configuration MAC Control Message (AAI_GRP-CFG)

------------------------------------------------- Start of proposed text I --------------------------------------------------

-- Group Configuration Message

AAI-GRP-CFG ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

-- identify the flow to be added or deleted

flowId                         INTEGER (0..15),

-- Signals whether the group corresponds to DL allocations or UL allocations.

dlUlIndicator                  ENUMERATED {

dlAllocation (0),

ulAllocation (1)

}

-- present when a flow is added to a GRA

graInfoForAddedFlow       GroupRsrcAllocInfo OPTIONAL

}

GroupRsrcAllocInfo ::= SEQUENCE {

-- ID of the group to which the flow is added

groupId                        BIT STRING (SIZE(12)),

-- Defines number of subframes spanned by the allocated resource

-- If number of DL AAI subframes, D is less than number of UL AAI

-- subframes, U, and dlUlIndicator = 0b1, then longTtiIndicator = 0b1.

longTtiIndicator               ENUMERATED {

oneSubframe (0),

fourSubframe (1)

} DEFAULT oneSubframe,

-- the periodicity with which the corresponding GRA A-MAP IE will be

-- transmitted.

periodicity                    ENUMERATED {

oneFrame   (0),

twoFrame   (1),

fourFrame  (2),

eightFrame (3)},

-- Signals the 2-bit MIMO mode set corresponding to this group.

mimoModeSet                    ENUMERATED {

mode0     (0),

mode1and2 (1),

mode2     (2),

mode2and4 (3)},

-- The size of user bitmap in bits. The size determines the maximum

-- number of flows that can be supported in one group

userBitmapSize                 ENUMERATED {

four      (0),

eight     (1),

mode2     (2),

mode2and4 (3)},

-- Signals index of the flow in group’s user bitmap

userBitmapIndex                INTEGER (0..31),

-- Signals the starting ACID of the range of ACIDs assigned to the

-- GRA flow

initialAcid                    INTEGER (0..15),

-- The number of ACIDs assigned to the GRA flow

numberOfAcid                   INTEGER (0..7),

burstSizeList                  BurstSizeList OPTIONAL,

resourceBitmapList             ResourceBitmapList OPTIONAL

}

BurstSizeList ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..31)

ResourceBitmapList ::= SEQUENCE (SIZE (1..4)) OF ResourceBitmap

ResourceBitmap::= SEQUENCE {

resourceInclusionBitmap        BIT STRING {

b0 (0),  -- Bitmap to signal which

b1 (1),  -- 8 resource sizes are

b2 (2),  -- supported in the group

b3 (3),  -- out of the range of

b4 (4),  -- [1,16] LRUs supported

b5 (5),  -- for GRA. If nth bit n

b6 (6),  -- the bitmap is set to1,

b7 (7),  -- it signals that are

b8 (8),  -- source size of n LRUs

b9 (9),  -- is supported in the

b10 (10),   -- group.

b11 (11),

b12 (12),

b13 (13),

b14 (14),

b15 (15)} (SIZE(16)) OPTIONAL

[Add a new Table xxx as the following]

Table xxx: AAI_GRP-CFG

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M | Message type |  |  | 8 |  |  |
| M | Deletion Flag |  |  | 1 | Flag to signal whether this message includes addition or deletion information. 0: Flow is added to a group 1: Flow is explicitly deleted from a group |  |
| M | dlUlIndicato |  |  | 1 | 0: DL allocation 1: UL allocation |  |
| M | Flow ID |  |  | 4 | Identifies the flow that is added to or deleted from GRA group |  |
| O | GRA Info for added flow | groupId |  | 12 | ID of the group to which the flow is added | Present if Deletion Flag == 0 |
|  |  | Long TTI Indicator |  | 1 | Defines number of AAI subframes spanned by the allocated resource. 0: 1 AAI subframe (default) 1: 4 DL/UL AAI subframes for FDD or all DL/UL AAI subframe for TDD |  |
|  |  | Periodicity |  | 2 | Periodicity of allocation of this group. This specifies the periodicity with which the corresponding GRA A-MAP IE will be transmitted. 0b00: 1 frame 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |  |
|  |  | MIMO mode set |  | 2 | Signals the 2-bit MIMO mode set corresponding to this group. The MIMO mode set codes are specified in table 751 and table 752. |  |
|  |  | User Bitmap Size |  | 2 | The size of user bitmap in bits. The size remains fixed for a group. The size determines the maximum number of flows that can be supported in one group. 0: 4 1: 8 2: 16 3: 32 |  |
|  |  | User Bitmap Index |  | 5 | Signals index of the flow in group's user bitmap |  |
|  |  | Initial_ACID |  | 4 | Signals the starting ACID of the range of ACIDs assigned to the GRA flow |  |
|  |  | N_ACIDs |  | 3 | The number of ACIDs assigned to the GRA flow |  |
|  |  | Burst Size Indicator (b) |  | 1 | Signals whether the HARQ burst sizes supported in the group are included in this message or are same as the last GRA allocation for this flow. 0: Burst sizes same as last GRA allocation for this flow, hence not included in the message 1: Burst sizes explicitly included in the message |  |
|  |  | Resource size indicator (r) |  | 1 | Signals whether the resource sizes supported in the group are included in this message or are same as the previous group 0: Resource sizes same as last GRA allocation for this flow, hence not included in the message 1: Resource sizes explicitly included in the message |  |
|  |  | Burst&ResourceInfo [1..4] | Burst size i | 5 | ith burst size of the 4 burst sizes supportedin the group | Present if b == 1 |
|  |  |  | Resource Bitmap Flag (f) | 1 | 0b0: The resource size inclusion bitmap for this burst size is the same as the bitmap defined for the (i-1) th burst size. 0b1: Define a resource size inclusion bitmap for the i-th burst size | Present if r == 1 |
|  |  |  | Resource size inclusion bitmap | 16 | Bitmap to signal which 8 resource sizes are supported in the group out of the range of [1,16] LRUs supported for GRA. If nth bit in the bitmap is set to 1, it signals that a resource size of n LRUs is supported in the group. | Present if f == 1 |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

------------------------------------------------- Start of proposed text III --------------------------------------------------

[Add the following ASN.1 code to Annex P.2]

-- Group Configuration Message

AAI-GRP-CFG ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

-- identify the flow to be added or deleted

flowId                         INTEGER (0..15),

-- Signals whether the group corresponds to DL allocations or UL allocations.

deletionFlag                   ENUMERATED {

flowAdded (0),

flowDeleted (1)

}

-- Signals whether the group corresponds to DL allocations or UL allocations.

dlUlIndicator                  ENUMERATED {

dlAllocation (0),

ulAllocation (1)

}

-- present when a flow is added to a GRA

graInfoForAddedFlow            GroupRsrcAllocInfo OPTIONAL

}

GroupRsrcAllocInfo ::= SEQUENCE {

-- ID of the group to which the flow is added

groupId                        BIT STRING (SIZE(12)),

-- Defines number of subframes spanned by the allocated resource

longTtiIndicator               ENUMERATED {

oneSubframe (0),

fourSubframe (1)

} DEFAULT oneSubframe,

-- the periodicity with which the corresponding GRA A-MAP IE will be

-- transmitted.

periodicity                    ENUMERATED {

oneFrame   (0),

twoFrame   (1),

fourFrame  (2),

eightFrame (3)},

-- Signals the 2-bit MIMO mode set corresponding to this group.

mimoModeSet                    ENUMERATED {

mode0     (0),

mode1and2 (1),

mode2     (2),

mode2and4 (3)},

-- The size of user bitmap in bits. The size determines the maximum

-- number of flows that can be supported in one group

userBitmapSize                 ENUMERATED {

size4  (0),

size8  (1),

size16 (2),

size32 (3)},

-- Signals index of the flow in group’s user bitmap

userBitmapIndex                INTEGER (0..31),

-- Signals the starting ACID of the range of ACIDs assigned to the

-- GRA flow

initialAcid                    INTEGER (0..15),

-- The number of ACIDs assigned to the GRA flow

numberOfAcid                   INTEGER (0..7),

burstResourceInfo              BurstResourceInfo,

burstSizeList                  BurstSizeList OPTIONAL,

resourceBitmapList             ResourceBitmapList OPTIONAL

}

BurstResourceInfo ::= SEQUENCE {

burstResource1                 BurstResourceSet OPTIONAL,

burstResource2                 BurstResourceSet OPTIONAL,

burstResource3                 BurstResourceSet OPTIONAL,

burstResource4                 BurstResourceSet OPTIONAL}

BurstResourceSet ::= SEQUENCE {

burstSize                      INTEGER (0..31) OPTIONAL,

resourceBitmapFlah             BOOLEAN OPTIONAL,

resourceBitmap                 BIT STRING {

b0 (0),  -- Bitmap to signal which

b1 (1),  -- 8 resource sizes are

b2 (2),  -- supported in the group

b3 (3),  -- out of the range of

b4 (4),  -- [1,16] LRUs supported

b5 (5),  -- for GRA. If nth bit n

b6 (6),  -- the bitmap is set to1,

b7 (7),  -- it signals that are

b8 (8),  -- source size of n LRUs

b9 (9),  -- is supported in the

b10 (10),   -- group.

b11 (11),

b12 (12),

b13 (13),

b14 (14),

b15 (15)} (SIZE(16)) OPTIONAL

}

------------------------------------------------- End of proposed text III --------------------------------------------------