| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Multiplexing with Unicast (16.2.3.29, 16.3.5.6, 16.3.6.5.3, & 16.9.3.1) |  |
| Date Submitted | 2009-03-05 |  |
| Source(s) | Yujian Zhang, Jong-kae Fwu Intel Corporation | E-mail: yujian.zhang@intel.com, jong-kae.fwu@intel.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 Working Group Letter Ballot Recirc # 31 / Topic: E-MBS (Section 16.2.3.29, 16.3.5.6, 16.3.6.5.3, & 16.9.3.1) |  |
| Abstract | This contribution proposes cleanup on E-MBS multiplexing with unicast. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

E-MBS Multiplexing with Unicast

Yujian Zhang, Jong-kae Fwu

Intel Corporation

- Introduction

This contribution proposes pure FDM as discussed in [5]. The proposed text is developed so that it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. Following changes are for current amendment text [1].

- Remove E-MBS_SUBFRAME_INDICATOR.

- For E-MBS-DATE_IE, remove the restriction that the end resource index for the last burst is not indicated.

  - It is also corrected that E-MBS AAI subframe offset should indicate where the data burst ends, to be consistent with E-MBS Resource Indexing.

- Move E-MBS subband information to AAI_SCD.

- References

- IEEE P802.16m/D4, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- C802.16m-10/0383, “E-MBS Multiplexing with Unicast”

- Text proposal for inclusion in the P802.16m/D4

Instruction to editor:

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy#1 – Modify the text in section 16.2.3.29, line 48, page 116 as follows]

…

shiftValueUForSoundingSymbol	INTEGER (0...255),

RELAY_Support_Parameters :: = SEQUENCE {

16m_Relay_zone_AMS_allocation_indicator	INTEGER (0..1) OPTIONAL

}

-- DSAC_MAX denotes the maximum number of subbands, and it equals 21, 10, 4 for

-- 20, 10 and 5 MHz respectively, refer 16.3.5.2.1.

-- For details of Zone_Allocation-Bit-MAP, refer 16.9.3.1.

Zone_Allocation-Bit-MAP :: = SEQUENCE (SIZE (1..DSAC_MAX)) OF INTEGER (0..1)

-- Zone Flag bit, which indicates the use of the last zone.

-- 0b0: Unicast; 0b1: E-MBS. For usage, refer 16.9.3.1

ZF ::= INTEGER (0..1)

}

-- ASN1STOP

[Remedy#2 – Modify the text in section 16.3.5.6, line 12, page 407 as follows]

16.3.5.6 Downlink physical structure for E-MBS support

In a carrier that is not dedicated to E-MBS traffic, it may be required to multiplex E-MBS and unicast traffic. When multiplexed with unicast data, the E-MBS traffic is FDMed with the unicast traffic in the downlink subframes. More specifically, the E-MBS traffic is transmitted using subbands assigned in frequency partition FP0. The number of subbands used for E-MBS data, denoted by KSB,E-MBS, is indicated in the E-MBS_SUBBAND_INDICATOR in AAI-E-MBS_CFG message. This set of sub-bands form the E-MBS region in the downlink subframes. The E-MBS_SUBFRAME_INDICATOR indicates which sub-frames in a frame carry E-MBS traffic of all E-MBS Zones on that carrier. The E-MBS_SUBBAND_INDICATOR and E-MBS_SUBFRAME_INDICATOR are is valid for the set of super-frames over which the AAI-E-MBS_CFG indicators are valid. For MBSFN support, KSB,E-MBS subbands are assigned from the lowest SLRU in frequency partition FP0. If the KSB,E-MBS is larger than the number of subbands in FP0, i.e., KSB,E-MBS > KSB,FP0, then, the remaining (KSB,E-MBS - KSB,FP0) subbands are assigned equitably to the three partitions FPi (i > 0) one by one starting from the lowest SLRU of the each FP. When FPCT = 3, KSB,E-MBS subbands are assigned equitably from FPi (i > 0) one by one starting from the lowest SLRU of the lowest FP.

[Remedy#3 – Modify the text in section 16.3.6.5.3, line 39, page 527 as follows]

16.3.6.5.3 E-MBS Control Information Elements

The resource mapping for E-MBS allows are defined in E-MBS-DATA_IEs as defined in Table 841.

Table 841—E-MBS DATA Information Elements

| … | … | … |
| --- | --- | --- |
| if(i<No. of Multicast STIDs+FIDs - 1){ | - | - |
| E-MBS AAI subframe offset | Variable | Includes the location of the AAI subframe where the E-MBS data burst begins ends MSI == 0b00: 6 bits MSI == 0b01: 7 bits MSI == 0b10: 8 bits MSI == 0b11: 9 bits |
| E-MBS Resource Indexing | 7 | Includes the location of the SLRU index where the E-MBS data burst ends |
| … | … | … |

[Remedy#4 – Modify the text in section 16.9.3.1, line 35, page 751 as follows]

16.9.3.1 E-MBS Configuration Indicators

…

Table 949—AAI_E-MBS-CFG_Message format

| … | … | … |
| --- | --- | --- |
| E-MBS_CFG_LIFETIME (m). | 4. | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. Duration of E-MBS_CFG_LIFETIME: 16 (m+1) superframes. |
| Zone_Allocation Bit-MAP. | variable. | Indicates the zone configuration. The size is as below. 20 bits for 20 MHz 9 bits for 10MHz 3 bits for 5 MHz. |
| ZF. | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS. |
| … | … | … |

Zone_Allocation Bit-MAP: Zone_Allocation Bit-MAP consists of sub-band indices reserved for all E-MBS zones the BS belongs to. The Zone Allocation Bit-MAP in the AAI-SCDAAI-E-MBS-CFG message identifies the use of the resource comprising a set of contiguous subbands in a DL AAI subframe. The Bit-MAP determines the size (S) of each E-MBS zone in number of contiguous SLRUs in frequency domain within the subframe, the index (L) of each E-MBS zone from where the allocated zone begins and the total number (Num_E-MBS_Zones) of allocated zones.