| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Multiplexing with Unicast (16.2.3.29, 16.3.5.6, & 16.9.3.1) |  |
| Date Submitted | 2009-03-16 |  |
| Source(s) | Yujian Zhang, Jong-kae Fwu Intel Corporation Kaiying Lv, Bo Sun ZTE Corporation Kaushik Josiam, Jaejeong Shim Samsung Electronics Yan-Xiu Zheng ITRI Eunkyung Kim ETRI Jeongki Kim LG Electronics | E-mail: yujian.zhang@intel.com, jong-kae.fwu@intel.com lv.kaiying@zte.com.cn k.josiam@samsung.com zhengyanxiu@itri.org.tw ekkim@etri.re.kr jeongki.kim@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 Working Group Letter Ballot Recirc # 31 / Topic: E-MBS (Section 16.2.3.29, 16.3.5.6, & 16.9.3.1) |  |
| Abstract | This contribution proposes cleanup on E-MBS multiplexing with unicast. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS Multiplexing with Unicast

Yujian Zhang, Jong-kae Fwu

Intel Corporation

Kaiying Lv, Bo Sun

ZTE Corporation

Kaushik Josiam, Jaejeong Shim

Samsung Electronics

Yan-Xiu Zheng

ITRI

Eunkyung Kim

ETRI

Jeongki Kim

LG Electronics

- Introduction

This contribution proposes pure FDM as discussed in [5]. The proposed text is developed so that it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. Following changes are for current amendment text [1].

- Remove E-MBS_SUBFRAME_INDICATOR.

- Move E-MBS subband information to AAI_SCD.

- Move MSI length to AAI_SCD, and MSI length is common for all E-MBS zones which ABS belongs to.

- Indicate the end frame offset for the last E-MBS burst in AAI_SCD

- References

- IEEE P802.16m/D4, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- C802.16m-10/0383, “E-MBS Multiplexing with Unicast”

- Text proposal for inclusion in the P802.16m/D4

Instruction to editor:

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy#1 – Modify the text in section 16.2.3.29, line 48, page 116 as follows]

…

Table XXX— AAI_SCD Message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| … | … |  | … | … | … |
| O | E-MBS Configuration Parameters | Zone_Allocation-Bit-MAP | DSACMAX-1 | See section 16.9.3.1. | Present when E-MBS is configured |
|  |  | ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1. |  |
|  |  | MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16) |  |
|  |  | E-MBS AAI frame offset | variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3 bits MSI length == 0b01: 4 bits MSI length == 0b10: 5 bits MSI length == 0b11: 6 bits |  |

[Remedy#2 – Modify the text in section 16.3.5.6, line 12, page 407 as follows]

16.3.5.6 Downlink physical structure for E-MBS support

In a carrier that is not dedicated to E-MBS traffic, it may be required to multiplex E-MBS and unicast traffic. When multiplexed with unicast data, the E-MBS traffic is FDMed with the unicast traffic in the downlink subframes. More specifically, the E-MBS traffic is transmitted using subbands assigned in frequency partition FP0. The number of subbands used for E-MBS data, denoted by KSB,E-MBS, is indicated in the E-MBS_SUBBAND_INDICATOR in AAI-E-MBS_CFG message. This set of sub-bands form the E-MBS region in the downlink subframes. The E-MBS_SUBFRAME_INDICATOR indicates which sub-frames in a frame carry E-MBS traffic of all E-MBS Zones on that carrier. The E-MBS_SUBBAND_INDICATOR and E-MBS_SUBFRAME_INDICATOR are is valid for the set of super-frames over which the AAI-E-MBS_CFG indicators are valid. For MBSFN support, KSB,E-MBS subbands are assigned from the lowest SLRU in frequency partition FP0. If the KSB,E-MBS is larger than the number of subbands in FP0, i.e., KSB,E-MBS > KSB,FP0, then, the remaining (KSB,E-MBS - KSB,FP0) subbands are assigned equitably to the three partitions FPi (i > 0) one by one starting from the lowest SLRU of the each FP. When FPCT = 3, KSB,E-MBS subbands are assigned equitably from FPi (i > 0) one by one starting from the lowest SLRU of the lowest FP.

[Remedy#3 – Modify the text in section 16.9.3.1, line 35, page 751 as follows]

16.9.3.1 E-MBS Configuration Indicators

…

Table 949—AAI_E-MBS-CFG_Message format

| … | … | … |
| --- | --- | --- |
| E-MBS_CFG_LIFETIME (m). | 4. | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. Duration of E-MBS_CFG_LIFETIME: 16 (m+1) superframes. |
| Zone_Allocation Bit-MAP. | variable. | Indicates the zone configuration. The size is as below. 20 bits for 20 MHz 9 bits for 10MHz 3 bits for 5 MHz. |
| ZF. | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS. |
| for (i = 0; i< Num_E-MBS_Zones; i++){. |  |  |
| E-MBS_Zone_ID. | 7. | The E-MBS_Zone_ID to which this E-MBS MAP applies.. |
| MSI Length (NMSI). | 2. | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16). |
| … | … | … |

Zone_Allocation Bit-MAP: Zone_Allocation Bit-MAP consists of sub-band indices reserved for all E-MBS zones the BS belongs to. The Zone Allocation Bit-MAP in the AAI-SCDAAI-E-MBS-CFG message identifies the use of the resource comprising a set of contiguous subbands in a DL AAI subframe. The Bit-MAP determines the size (S) of each E-MBS zone in number of contiguous SLRUs in frequency domain within the subframe, the index (L) of each E-MBS zone from where the allocated zone begins and the total number (Num_E-MBS_Zones) of allocated zones.