| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Finer SA-Preamble Partitioning for ABS Type (16.3.6) |  |
| Date Submitted | 2010-03-06 |  |
| Source(s) | Youngbo Cho, Sung-Eun Park Samsung Electronics Co., Ltd | {youngbo.cho, se.park}@samsung.com |
| Re: | Commets on P802.16m/D4 for recirculation #31 |  |
| Abstract | The contribution provides the proposed text of SA-Preamble partitioning for ABS type to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of SA-Preamble Partitioning for ABS Type (16.3.6)

Youngbo Cho, Sung-Eun Park

Samsung Electronics Co., Ltd

- Introduction

In previous Jeju meeting, the high level concept of partitioning for SA-Preamble sequences were accepted for supporting partitioning of different ABS type [1]. This contribution provides a proposed text which modifies the current sequence number for macro ABS as other proper value and suggests details for finer partition of remaining SA-Preamble sequences.

- Contents of this proposal

Current status in D4 regarding SA-Preamble partitioning is

- Hard partitioning for macro ABS: 258 sequences for macro ABS (86 sequences per segment)

- Soft partitioning for non-macro ABS: Indicated in S-SFH SP3 and AAI_SCD message

  - In S-SFH SP3: 4bit SA-Preamble sequence soft partitioning information indicates partitioning for public or CSG-femto ABS

  - In AAI_SCD message: 8 bit SA_PreamblePartitionforBStype indicates finer partitioning of hotzone/relay/ OSG-femto/etc for public ABS and CSG-closed/open femto for CSG-femto ABS

Provided contents in this proposal are following.

- Finer partitioning in AAI_SCD message

  - Based on sequence partitioning in S-SFH SP3, finer partitioning in AAI_SCD can be applied with same granularity of sequence partitioning in S-SFH SP3 (by 8 bit-indicator for each ABS type).

  - AAI_SCD includes 8-bit SA_PreamblePartitionforBStype indicators, which includes ABS type and IDcell range information for each ABS type; macro hot-zone, relay, OSG femto, CSG open femto and CSG closed femto ABS. The MSB 4-bits of SA_PreamblePartitionforBStype indicates the type of corresponding ABS as follows:

- Table xxx: ABS type

| MSB 4 bit value indicated by SA_PreamblePartitionforBStype | ABS type |
| --- | --- |
| 0000 | Macro |
| 0001 | Macro hotzone |
| 0010 | Relay |
| 0011 | OSG femto |
| 0100 | CSG open femto |
| 0101 | CSG closed femto |
| 0110~1111 | reserved |

  - The LSB 4-bits of SA_PreamblePartitionforBStype indicates the last index of preamble sequence of the corresponding ABS type as shown in Table yyy. The range of the preamble sequence of the corresponding ABS type is above the last index of the preamble sequence for the previous ABS type and to SA_PreamblePartitionforBStype.

- Table yyy: IDCell partitioning for each ABS

| LSB 4 bit value indicated by SA_Preamble PartitionforBStype | The Last index of IDCell for the Public ABS | The Last index of IDCell for the CSG ABS |
| --- | --- | --- |
| 0000 | 95+256*n | 105+256*n |
| 0001 | 105+256*n | 115+256*n |
| 0010 | 115+256*n | 125+256*n |
| 0011 | 125+256*n | 135+256*n |
| 0100 | 135+256*n | 145+256*n |
| 0101 | 145+256*n | 155+256*n |
| 0110 | 155+256*n | 165+256*n |
| 0111 | 165+256*n | 175+256*n |
| 1000 | 175+256*n | 185+256*n |
| 1001 | 185+256*n | 195+256*n |
| 1010 | 195+256*n | 205+256*n |
| 1011 | 205+256*n | 215+256*n |
| 1100 | 215+256*n | 225+256*n |
| 1101 | 225+256*n | 235+256*n |
| 1110 | 235+256*n | 245+256*n |
| 1111 | 245+256*n | 255+256*n |

  - For example, SA_PreamblePartitionforBStype indicators have values of 00010011(for macro hot-zone),  00110111 (for OSG femto), and 01001001 (for CSG open) and 01011111 (for CSG closed), ranges of IDCell for each ABS is as follows:

Macro hot-zone:  		86+256*n~125+256*n

OSG femto:     		126+256*n~165*256*n

CSG open femto: 		166+256*n~195*256*n

CSG closed femto: 	196+256*n~255+265*n

- References

[1] IEEE 802.16m-09/1902r3, “Proposed Text Proposal of SA-Preamble Partition in IEEE 802.16m/D1”

[2] IEEE 802.16m-07/0034r2, “802.16m System Requirements”

[3] IEEE 802.16m/D4, “Advanced Air Interface (Draft)”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Insert the paragraph starting in line 45 of page 411 of P802.16m/D4 as following

-------------------------------------------------------  Text Start  --------------------------------------------------------------

The AAI_SCD message provides finer partition information. The non-macro ABS base stations can be further categorized different types, such as: hotzone, relay, OSG and etc. The CSG femto base stations can be further categorized, such as: CSG-closed and CSG-open.

AAI_SCD includes 8-bit SA_PreamblePartitionforBStype indicators, which is ordered by the ABS type; macro hot-zone, relay, OSG femto and CSG open femto. And, remained IDcell indices which is not indicated is assigned for CSG closed femto ABSs. The MSB 4-bits of SA_PreamblePartitionforBStype indicates the type of corresponding ABS as shown in Table xxx.

Each SA_PreamblePartitionforBStype indicates the last index of preamble sequence of the corresponding ABS type as shown in Table xxx. The range of the preamble sequence of the corresponding ABS type is above the last index of the preamble sequence for the previous ABS type and to SA_PreamblePartitionforBStype. For macro hot-zone, the range of IDcell partition is above the last index of macro ABS and to SA_PreamblePartitionforBStype.

Table xxx: ABS type

| MSB 4 bit value indicated by SA_PreamblePartitionforBStype | ABS type |
| --- | --- |
| 0000 | Macro |
| 0001 | Macro hotzone |
| 0010 | Relay |
| 0011 | OSG femto |
| 0100 | CSG open femto |
| 0101 | CSG closed femto |
| 0110~1111 | reserved |

The LSB 4-bits of SA_PreamblePartitionforBStype indicate the last index of preamble sequence of the corresponding ABS type as shown in Table yyy. The range of the preamble sequence of the corresponding ABS type is above the last index of the preamble sequence for the previous ABS type and to the last index which is indicated by corresponding SA_PreamblePartitionforBStype.

Table yyy: IDCell partitioning for each ABS

| LSB 4 bit value indicated by SA_Preamble PartitionforBStype | The Last index of IDCell for the Public ABS | The Last index of IDCell for the CSG ABS |
| --- | --- | --- |
| 0000 | 95+256*n | 105+256*n |
| 0001 | 105+256*n | 115+256*n |
| 0010 | 115+256*n | 125+256*n |
| 0011 | 125+256*n | 135+256*n |
| 0100 | 135+256*n | 145+256*n |
| 0101 | 145+256*n | 155+256*n |
| 0110 | 155+256*n | 165+256*n |
| 0111 | 165+256*n | 175+256*n |
| 1000 | 175+256*n | 185+256*n |
| 1001 | 185+256*n | 195+256*n |
| 1010 | 195+256*n | 205+256*n |
| 1011 | 205+256*n | 215+256*n |
| 1100 | 215+256*n | 225+256*n |
| 1101 | 225+256*n | 235+256*n |
| 1110 | 235+256*n | 245+256*n |
| 1111 | 245+256*n | 255+256*n |

-------------------------------------------------------  Text End  ----------------------------------------------------------------