| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN Format for MAC Management Messages for Sleep Operation in IEEE 802.16m (16.2.3) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Joey Chou, Shantidev Mohanty Intel Corporation Yih-Shen Chen MediaTek Yeongmoon SonSamsung Ming-Hung Tao ITRI Giwon Park LG | E-mail: shantidev.mohanty@intel.com yihshen.chen@mediatek.com ym1004.son@samsung.com MHTao@itri.org.tw giwon.park@lge.com |
| Re: |  |  |
| Abstract | This contribution proposes methods to handle concurrent sleep cycle configuration change initiation in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN Format for MAC Management Messages for Sleep Operation in IEEE 802.16m (16.2.3)

Joey Chou, Shantidev Mohanty

Intel Corporation

Yih-Shen Chen

MediaTek

Yeongmoon Son

Samsung

Ming-Hung Tao

ITRI

Giwon Park

LG

1. Introduction

The existing format of MAC management messages used for sleep operation in D4 is in ASN format. This contribution proposes ASN format for MAC management messages used for sleep operation in IEEE 802.16m.

2. Proposed Text

Text proposal for inclusion in the P802.16m/D4

======================== Start of Proposed Text #1==============================

[Note to Editor Change 1: Replace Table “Table 701—parameters for AAI_ SLP-REQ” in section 16.2.23, starting from line 38, page 103 as with the following Table]

Table 701 – parameters for AAI_SLP-REQ

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  | 8 | MAC control message type | NA |
| M | Operation |  | 2 | This indicates operation request type of AAI_SLP-REQ message 0b00 : Exit from Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting | NA |
| O | SCID |  | 3 | Sleep Cycle ID. 0~15 | When (operation != 0b00) |
| O | Start Frame Number |  | 6 | 0~63 | When (operation != 0b00) |
| O | M | TIMF | 1 | Traffic Indication Message Flag 0 : AAI_TRF-IND message is not sent for the AMS 1 : AAI_TRF-IND message is sent to the AMS during every Listening Window 0~1 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | LWEF | 1 | Listening window Extension Flag. 0~1 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00 ~ 0b11 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). 0~15 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). 0~1023 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | Listening Window | 6 | Assigned duration of AMS’s default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. 0~63 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | Listening subframe bitmap | 1 | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake. The size of the bitmap equals to the number of AAI subframes of a frame. Bit #0 is mapped to the first AAI subframe.. Each bit in the bitmap indicates: 1: AMS wakes up at the specific AAI subframe 0: AMS does not wake up at the specific AAI subframe | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | O | New Initial Sleep Cycle | 5 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). 0~31 | When (operation == 0b01 ¦¦ operation == 0b10) && (NSCF == 0b10) |
|  | O | T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | When (operation == 0b01 ¦¦ operation == 0b10) && (LWEF == 1) |

[Note to Editor Change 2: Replace Table “Table 702—parameters for AAI_ SLP-RSP” in section 16.2.24, starting from line 32, page 106 as with the following Table]

Table : AAI_SLP-RSP

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  | 8 | MAC control message type | NA |
| M | Response_Code |  | 2 | This indicates response type of AAI_SLP-RSP message. 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI_SLP-REQ 0b10 : Rejection of AAI_SLP-REQ 0b11 : Reserved | NA |
| O | Operation |  | 2 | This indicates operation type of AAI_SLP-RSP message. 0b00 : Exit Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Mode 0b11 : Switch Sleep Cycle setting | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) |
| O | SCID |  | 4 | Sleep Cycle ID 0~15 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation != 0b00) |
| O | CQI_Operation |  | 2 | 0: the CQICH assigned to the AMS is kept 1: the CQICH is de-allocated at the frame specified by Start_Frame_Number 2: the CQICH is automatically de-allocated at the beginning of the sleep window whenever the CQICH is newly assigned to the AMS during the Listening Window 0~3 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation != 0b00) |
| O | Start Frame Number |  | 6 | Least Significant 6 bits of Frame Number. 0~63 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation != 0b00) |
| O | M | TIMF | 1 | Traffic Indication Message Flag 0 : AAI_TRF-IND message is not sent for the AMS 1 : AAI_TRF-IND message is sent to the AMS during every Listening Window | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
|  | M | LWEF | 1 | Listening window Extension Flag 0~1 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
|  | M | NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00~0b11 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
|  | M | Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). 0~15 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
|  | M | Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). 0~1023 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
|  | M | Listening Window | 6 | Assigned duration of AMS’s default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. 0~63 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
|  | M | Listening subframe bitmap | 1 | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake The size of the bitmap equals to the number of AAI subframes of a frame. Each bit in the bitmap indicates: 1: AMS wakes up at the specific AAI subframe 0: AMS does not wake up at the specific AAI subframe | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
|  | O | SLPID | 10 | This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID and SLP Group ID while the AMS is still in sleep mode. 0~1023 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) && (TIMF == 1) |
|  | O | New Initial Sleep Cycle | 6 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). 0~63 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) && (NSCF == 0b10) |
|  | O | T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) && (LWEF == 1) |
| O | REQ_duration |  | 8 | Waiting value for the AAI_SLP-REQ message retransmission , which is the Least Significant 8 bits of Frame Number. 0~255 | When (Response_Code == 0b10) |

[Note to Editor Change 3: Replace Table “Table 703—parameters for AAI_ TRF-IND” in section 16.2.25, starting from line 4, page 111 as with the following Table]

Table : AAI_TRF-IND

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  | 8 | MAC control message type | NA |
| M | FRMT |  | 1 | This indicates type of Traffic Indication in AAI_TRF-IND message. 0 : It indicates the SLPID bitmap-based traffic indication 1: It indicates the SLPID-based traffic indication | NA |
| O | M | SLPID Group Indication Bitmap | 32 | It indicates the existence of each SLPID group N-th bit of SLPID-Group Indication Bitmap [MSB corresponds to N = 0] is allocated to SLPID Group that includes AMS with SLPID values from N*32 to N*32+31 0: There is no traffic for any of the 32 AMSs that belong to the SLPID-Group 1: There is traffic for at least one AMS in SLPID-Group. | When FRMT ==0 |
|  | M | Traffic indication bitmap | N*32 | It indicates the traffic indication for 32 AMSs in each SLPID group Each Traffic Indication bitmap comprises multiples of 32-bit long Traffic Indication units. A Traffic Indication unit for 32 SLPIDs is added to AAI_TRF-IND message whenever its SLPID Group is set to 1 32 bits of Traffic Indication Unit (starting from MSB) are allocated to AMS in the ascending order of their SLPID values: 0: Negative indication 1: Positive indication | When FRMT ==0 |
| O | M | Num_of_SLPIDs | 6 | It indicates the number of SLPID included in AAI_TRF-IND message 0~63 | When FRMT ==1 |
|  | M | SLPID | 10 | Each SLPID is used to indicate the positive traffic indication for an AMS 0~1023 | When FRMT ==1 |

[Note to Editor Change 4: Replace Table “Table 704—parameters for AAI_ TRF-IND-REQ” in section 16.2.26, starting from line 4, page 113 as with the following Table]

Table : AAI_TRF_IND-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | MAC control message type | NA |
| M | Frame_Number | 4 | This indicates the least significant 4 bits of frame number in which the AMS expected to receive the AAI_TRF-IND. 0~15 | NA |

[Note to Editor Change 5: Replace Table “Table 705—parameters for AAI_ TRF-IND-RSP” in section 16.2.27, starting from line 37, page 113 as with the following Table]

Table : AAI_TRF_IND-RSP

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | MAC control message type | NA |
| M | Frame_Number | 4 | This indicates the least significant 4 bits of frame number. 0~15 | NA |
| M | Traffic_Indication | 1 | It indicates the traffic indication which the ABS has transmitted to the AMS. 0: Negative Traffic Indication 1: Positive Traffic Indication | NA |

========================= End of Proposed Text#1 ==============================

======================== Start of Proposed Tex t#2==============================

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

[Add the following ASN.1 code to Annex P.2]

-- Sleep Mode Request MAC Control Message

AAI-SLP-REQ ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the purpose of configuration request

opCode	                 ENUMERATED  {

exitFromSleepMode(0),

enterSleepMode(1),

changeSleepMode(2),

switchSleepCycleSetting (3)},

-- sleep cycle ID

scid				INTEGER (0..15) OPTIONAL,

-- the least significant 6 bits of Frame Number

startFrameNumber		INTEGER (0..63) OPTIONAL,

-- parameter set for sleep mode configuration request

sleepModeRequestInfo       SleepModeRequestInfo OPTIONAL

-- data structure of SleepModeRequestInfo

SleepModeRequestInfo ::= SEQUENCE {

-- traffic indication message flag

timf				ENUMERATED {DISABLE(0), ENABLE(1)},

-- listening window extension flag

lwef				ENUMERATED {DISABLE(0), ENABLE(1)},

-- new sleep cycle indicator

nscf				INTEGER (0..3), --0b00: reset to Initial

sleep cycle;

-- 0b01: min (2*previous

sleep cycle, final

sleep cycle)

-- 0b10: reset to another

initial sleep cycle

value

-- 0b11: reserved

-- initial sleep cycle (measured in frames)

initialSleepCycle		INTEGER (0..15),

-- final sleep cycle (measured in frames)

finalSleepCycle		INTEGER (0..1023),

-- default listen window (measured in frames)

listenWindow			INTEGER (0..63),

-- listening subframe bitmap

listeningSubframeBitmap	BIT STRING {

b0 (0),   -- Bitmap to signal which subframe

b1 (1),   -- the AMS is required to keep awake.

b2 (2),   -- If nth bit in the bitmap is set to 1,

b3 (3),   -- it signals that the AMS should

b4 (4),   -- remain awake in the nth subframe.

b5 (5),

b6 (6),

b7 (7),

b8 (8)} (SIZE(8)),

-- new initial sleep cycle (measured in frames)

newInitialSleepCycle	INTEGER (0..31) OPTIONAL,

-- timer for listening window extension (measured in frames)

tAms				INTEGER (0..31) OPTIONAL

}

}

-- Sleep Mode Response MAC Control Message

AAI-SLP-RSP ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the purpose of configuration response

responseCode	                 ENUMERATED  {

unsolictedResponseByAbs(0),

requestApproval(1),

requestRejection(2),

reserved (3)},

-- operation code

opCode					ENUMERATED  {

exitSleepMode(0),

enterSleepMode(1),

changeSleepMode(2),

switchSleepCycleSetting (3)} OPTIONAL,

-- sleep cycle ID

scid				INTEGER (0..15) OPTIONAL,

-- CQI operation indication

cqiOperation			INTEGER (0..2) OPTIONAL, -- 0: the CQICH assigned

-- to the AMS is kept.

-- 1: the CQICH is de

-- allocated at the frame

-- specified by

-- startFrameNumber.

-- 2: the CQICH is

-- automatically

-- deallocated at the

-- beginning of the sleep

-- window whenever the

-- CQICH is newly

-- assigned to the AMS

-- during the Listening

-- Window.

-- the least significant 6 bits of Frame Number

startFrameNumber		INTEGER (0..63) OPTIONAL,

-- parameter set for sleep mode configuration response

sleepModeResponseInfo       SleepModeResponseInfo OPTIONAL,

-- waiting value for AAI_SLP-REQ message retransmission

reqDuration			INTEGER (0..255) OPTIONAL –- This value is the

-- least significant 8

-- bits of Frame Number.

-- data structure for SleepMode ResponseInfo

SleepModeResponseInfo ::= SEQUENCE {

-- traffic indication message flag

timf				ENUMERATED {DISABLE(0), ENABLE(1)},

-- listening window extension flag

lwef				ENUMERATED {DISABLE(0), ENABLE(1)},

-- new sleep cycle indicator

nscf				INTEGER (0..3), --0b00: reset to Initial

sleep cycle;

-- 0b01: min (2*previous

sleep cycle, final

sleep cycle)

-- 0b10: reset to another

initial sleep cycle

value

-- 0b11: reserved

-- initial sleep cycle (measured in frames)

initialSleepCycle		INTEGER (0..15),

-- final sleep cycle (measured in frames)

finalSleepCycle		INTEGER (0..1023),

-- default listen window (measured in frames)

listenWindow			INTEGER (0..63),

-- listening subframe bitmap

listeningSubframeBitmap	BIT STRING {

b0 (0),   -- Bitmap to signal which subframe

b1 (1),   -- the AMS is required to keep awake.

b2 (2),   -- If nth bit in the bitmap is set to 1,

b3 (3),   -- it signals that the AMS should

b4 (4),   -- remain awake in the nth subframe.

b5 (5),

b6 (6),

b7 (7),

b8 (8)} (SIZE(8)),

-- SLPID

slpid				INTEGER (0..1023),

-- new initial sleep cycle (measured in frames)

newInitialSleepCycle	INTEGER (0..31) OPTIONAL,

-- timer for listening window extension (measured in frames)

tAms				INTEGER (0..31) OPTIONAL

}

}

-- Traffic Indication MAC Control Message

AAI-TRF-IND ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the indication type

frmt	           ENUMERATED  {

mod0(0),  -- mod0: SLPID bitmap-based traffic

-- indication

mod1(1)}, -- mod1: SLPID-based traffic

-- indication

-- traffic indication for SLPID bitmap-based traffic indication

trfIndInfomod0	SEQUENCE (SIZE(1..32)) OF TrfIndInfoMod0	OPTIONAL,

-- traffic indication for SLPID-based traffic indication

trfIndInfomod1	TrfIndInfoMod1	OPTIONAL

}

TrfIndInfoMod0 ::= SEQUENCE {

slpidGroupIndicationBitmap	BOOLEAN,

trafficIndicationBitmap		BIT STRING {

b0 (0),   -- 0: Negative indication

b1 (1),   -- 1: Positive indication

b2 (2),

b3 (3),

b4 (4),

b5 (5),

b6 (6),

b7 (7),

b8 (8),

b9 (9),

b10 (10),

b11 (11),

b12 (12),

b13 (13),

b14 (14),

b15 (15),

b16 (16),

b17 (17),

b18 (18),

b19 (19),

b20 (20),

b21 (21),

b22 (22),

b23 (23),

b24 (24),

b25 (25),

b26 (26),

b27 (27),

b28 (28),

b29 (29),

b30 (30),

b31 (31)

} (SIZE(32))	OPTIONAL

TrfIndInfoMod1 ::= SEQUENCE {

numOfSlpids		INTEGER(0..63)

slpid			SEQUENCE (SIZE(1..numOfSlpids)) OF INTEGER(0..1023)

}

}

-- Traffic Indication Request MAC Control Message

AAI-TRF-IND-REQ ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the least 4 bits of the frame number in which the AMS expected to

-- receive AAI_TRF-IND

frameNumber	INTEGER (0..15)

}

-- Traffic Indication Response MAC Control Message

AAI-TRF-IND-RSP ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

-- identify the least 4 bits of the frame number

frameNumber	INTEGER (0..15)

trafficIndication	BOOLEAN –- 0: Negative traffic indication

-- 1: Positive traffic indication

}

======================== End of Proposed Tex t#2==============================