| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS identification during coverage loss recovery procedure (16.2.15.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Youngkyo Baek Hyunjeong Kang Jungshin Park Jicheol Lee Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.15.6” |  |
| Abstract | This contribution proposes MS identification scheme during coverage loss recovery procedure to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS identification during coverage loss recovery procedure (16.2.15.6)

Youngkyo Baek, Hyunjeong Kang,Jungshin Park,Jicheol Lee

Samsung Electronics

# 1. Introduction

According to 16m/D4[1], CRID is used for MS identification when the AMS performs Network reentry from the coverage loss/DCR mode. However, some network architecture related problems are ambiguous. For example, which Network entity manages the CRID and AMS context and how ABS should retrieve AMS context, etc.

Regarding the TBD items, there were some decisions in NWG. NWG[4] suggests that anchor authenticator may maintain AMS context and its CRID and that the authenticator ID be 48bit long and the MS identifer be 24bit long. However, there is no decision about whether the authenticator ID and the MS identifier are merged in to 72bit CRID or the MS identifier separately defined from authenticator ID(i.e. 24bit CRID and 48bit authenticator ID).

In this contribution, we suggest the 72bit CRID which consists of anchor authenticator ID and the MS identifier.

# 2. Solution

When the AMS perform NW entry, through AAI_REG-RSP message a 72bit CRID which the anchor authenticator assigns to AMS.

Once coverage loss occurs and the AMS performs the NW reentry, AAI_RNG-REQ message contains the CRID. In response to the AAI_RNG-REQ, AAI_RNG-RSP message contains the new CRID if the CRID is changed. When anchor authenticator is relocated with re-authentication, the new CRID can be notified to AMS though key agreement procedure. (Especially key agreement MSG#3 can be used for it)

# 3. Text Proposal

# 3.1 Text Proposal #1

Inset the proposed subclause at page 38, line 23 as follows

======================== Start of Proposed Text =====================

16.2.1.2.4 Context Retention Identifier (CRID)

The network shall assign an72bit CRID to each AMS during network entry or zone switch to Mzone. The CRID consists of 48bits anchor authenticator ID and 24bits AMS identification information. The AMS is identified by the CRID in coverage loss recovery and DCR mode, where the CRID allows the network to retrieve AMS context. The network may assign the AMS a new CRID if necessary.

============================== End of Proposed Text ===============

# 3.2 Text Proposal #2

Add the following parameters into the table 723 (key agreement MSG#3) at page 133, line 36 as follows

======================== Start of Proposed Text =====================

| attribute | contents |
| --- | --- |
| Disguise ID | 72bit identifier used for transferring a new CRID securely, where disguise ID = current CRID  new CRID. AMS retrieves the new CRID by disguise ID  current CRID. It shall be included when anchor authenticator is changed with re-authentication. |

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.

- NWG_2.0_WID_Session_Minutes r2.doc, http://members.wimaxforum.org/apps/org/workgroup/nwg/download.php/52103/latest/NWG_2.0_WID_Session_Minutes r2.doc, March 2010.