| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up capability negotiation parameters (16.2.3.4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Youngkyo Baek Hyeonjeong Kang Jeongho Park Jaeweon cho Ji-Yun Seol Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.3.4” |  |
| Abstract | This contribution provides to clean up the capability negotiation parameter |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up capability negotiation parameters (16.2.3.4)

Youngkyo Baek, Hyeonjeong Kang, Jeongho Park,Jaeweon cho,Ji-Yun Seol

Samsung Electronics

# Introduction

AMS’s capabilities are negotiated through either basic capability negotiation procedure or registration procedure. But, what AMS capabilities to be negotiated are not decided yet. So we need some clean-up on AMS capabilities to be negotiated and those parameters are negotiated through which procedure( basic capability nego. Procedure v.s. registration procedure).

We first list up the capabilities to be negotiated.

And, if parameters are used after network entry completion, it is classified as ‘nego by REG procedure’.

If not, they are classied as ‘nego by SBC procedure’.

| Nego parameters | value | Nego messages |
| --- | --- | --- |
| channel BW and Cyclic prefix | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) | REG-REQ/RSP |
| frame configuration to support legacy(5MHz) | If Bit#0 =1,it supports(TDD only) | REG-REQ/RSP |
| frame configuration to support legacy(10MHz) | If Bit#0 =1,it supports(TDD only) | REG-REQ/RSP |
| Long TTI for UL | If Bit#0 =1,it supports | SBC-REQ/RSP |
| Long TTI for DL | If Bit#0 =1,it supports | SBC-REQ/RSP |
| OFDMA system support | If Bit#0 =1,it supports | REG-REQ/RSP |
| UL sounding | If Bit#0 =1,it supports | SBC-REQ/RSP |
| DL FFR | If Bit#0 =1,it supports | SBC-REQ/RSP |
| 3-step BR | If Bit#0 =1,it supports | REG-REQ/RSP |
| dynamic ranging for HO | If Bit#0 =1,it supports | REG-REQ/RSP |
| Max. Number of streams for SU-MIMO in DL MIMO | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) | SBC-REQ/RSP |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1/2(1bits; The number that is higher by 1 than this field) | SBC-REQ/RSP |
| DL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports | SBC-REQ/RSP |
| feedback support for DL | If Bit#0 =1, base mode supports If Bit#1 =1, trasnformation mode supports If Bit#2 =1, differential mode supports If Bit#3 =1, MIMO feedback mode 0 supports If Bit#4 =1, MIMO feedback mode 1 supports If Bit#5=1, MIMO feedback mode 2 supports If Bit#6 =1, MIMO feedback mode 3 supports If Bit#7 =1, MIMO feedback mode 4 supports If Bit#8 =1, MIMO feedback mode 5 supports If Bit#9 =1, MIMO feedback mode 6 supports If Bit#10 =1, MIMO feedback mode 7 supports If Bit#11=1, covariance matrix feedback supports | SBC-REQ/RSP |
| Number of Tx antenna of MS | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) | SBC-REQ/RSP |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 1/2/3/4(2bits; The number that is higher by 1 than this field) | SBC-REQ/RSP |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) | SBC-REQ/RSP |
| UL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports | SBC-REQ/RSP |
| multi-BS support | If Bit#0 =1,it supports | REG-REQ/RSP |
| Modulation scheme | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports | SBC-REQ/RSP |
| Persistent Allocation support | If Bit#0 =1,it supports | REG-REQ/RSP |
| Group Resource Allocation support | If Bit#0 =1,it supports | REG-REQ/RSP |
| MAX Tx Power | Bit 0–7: Maximum transmitted power for QPSK. Bit 8–15: Maximum transmitted power for 16-QAM Bit 15–23: Maximum transmitted power for 64-QAM. | SBC-REQ |
| UL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. | SBC-REQ/RSP |
| DL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. | SBC-REQ/RSP |
| AMS DL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. | SBC-REQ/RSP |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. | SBC-REQ/RSP |
| FFT size(2048/1024/512) | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports | SBC-REQ/RSP |
| Authorization policy support | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported | SBC-REQ/RSP |
| HO trigger metric | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports | REG-REQ/RSP |
| ARQ parameters | As D4 is | REG-REQ/RSP |
| capability for IM support | As D4 is | REG-REQ/RSP |
| LBS capability | As D4 is | REG-REQ/RSP |
| MC capabilities | As D4 is | REG-REQ/RSP |
| EMBS capabilities | As D4 is | REG-REQ/RSP |
| colocated coexistence support | As D4 is | REG-REQ/RSP |
| AMS scanning capabilities | As D4 is | REG-REQ/RSP |
| Covergence sublayer capabilities | As D4 is | REG-REQ/RSP |
| Host configuration parameters | As D4 is | REG-REQ/RSP |
| Redirection info | As D4 is | REG-RSP |
| AMS Scanning Capability | As D4 is | REG-REQ/RSP |

# 1Proposed Text#1

Modify the sentences, line 40, page 62 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

The following parameters(Table xxx)  may be included and parameter sets are mapped to capability index (the mapping is TBD):

•Authorization policy support

•-If Bit #0=0, EAP-based authorization is not supported

•-If Bit #0=1: EAP-based authorization is supported

•PN Window Size : Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks

•Auth type for EAP : Auth Type for EAP shall only be included when EAP-based authorization is supported.

•If Bit #0=0, device authentication

•If Bit #0=1, user authentication

Table xxx. AMS Capabilility parameters to be trasmitted to ABS

| Nego parameters | value |
| --- | --- |
| Long TTI for UL | If Bit#0 =1,it supports |
| Long TTI for DL | If Bit#0 =1,it supports |
| UL sounding | If Bit#0 =1,it supports |
| DL FFR | If Bit#0 =1,it supports |
| Max. Number of streams for SU-MIMO in DL MIMO | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1/2(1bits; The number that is higher by 1 than this field) |
| DL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |
| feedback support for DL | If Bit#0 =1, base mode supports If Bit#1 =1, trasnformation mode supports If Bit#2 =1, differential mode supports If Bit#3 =1, MIMO feedback mode 0 supports If Bit#4 =1, MIMO feedback mode 1 supports If Bit#5=1, MIMO feedback mode 2 supports If Bit#6 =1, MIMO feedback mode 3 supports If Bit#7 =1, MIMO feedback mode 4 supports If Bit#8 =1, MIMO feedback mode 5 supports If Bit#9 =1, MIMO feedback mode 6 supports If Bit#10 =1, MIMO feedback mode 7 supports If Bit#11=1, covariance matrix feedback supports |
| Number of Tx antenna of MS | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 1/2/3/4(2bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |
| UL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |
| Modulation scheme | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports |
| MAX Tx Power | Bit 0–7: Maximum transmitted power for QPSK. Bit 8–15: Maximum transmitted power for 16-QAM Bit 15–23: Maximum transmitted power for 64-QAM. |
| UL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
| DL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| AMS DL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| FFT size(2048/1024/512) | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# 2Proposed Text#2

Modify the sentences, line 46, page 71 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

The following parameters(Table yyy) may be included and parameter sets are mapped to cabability index(the mapping is TBD):

•Authorization policy support

•If Bit #0=0, EAP-based authorization is not supported;

•If Bit #0=1: EAP-based authorization is supported

•PN Window Size : Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks

Table yyy. AMS Capabilility parameters to be confirmed by ABS

| Nego parameters | value |
| --- | --- |
| Long TTI for UL | If Bit#0 =1,it supports |
| Long TTI for DL | If Bit#0 =1,it supports |
| UL sounding | If Bit#0 =1,it supports |
| DL FFR | If Bit#0 =1,it supports |
| Max. Number of streams for SU-MIMO in DL MIMO | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1/2(1bits; The number that is higher by 1 than this field) |
| DL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |
| feedback support for DL | If Bit#0 =1, base mode supports If Bit#1 =1, trasnformation mode supports If Bit#2 =1, differential mode supports If Bit#3 =1, MIMO feedback mode 0 supports If Bit#4 =1, MIMO feedback mode 1 supports If Bit#5=1, MIMO feedback mode 2 supports If Bit#6 =1, MIMO feedback mode 3 supports If Bit#7 =1, MIMO feedback mode 4 supports If Bit#8 =1, MIMO feedback mode 5 supports If Bit#9 =1, MIMO feedback mode 6 supports If Bit#10 =1, MIMO feedback mode 7 supports If Bit#11=1, covariance matrix feedback supports |
| Number of Tx antenna of MS | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 1/2/3/4(2bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |
| UL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |
| Modulation scheme | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports |
| UL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
| DL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| AMS DL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| FFT size(2048/1024/512) | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# 3Proposed Text#3

Modify the table 682, line 1, page74 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

The following parameters(table 782)  may be included:

Table 682- parameters for AAI_REG-REQ

| Name |  |  | value |
| --- | --- | --- | --- |
| AMS capability negotiation parameters | …….. |  | ….. |
|  | AMS DL HARQ buffering capability |  | Bits [6:0]: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
|  | AMS UL HARQ buffering capability |  | Bits [6:0]: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
|  | AMS DL processing capability per sub-frame |  | Bits [6:0] The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
|  | AMS UL processing capability per sub-frame |  | Bits [6:0] The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
|  | channel BW and Cyclic prefix |  | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |
|  | frame configuration to support legacy(5MHz) |  | If Bit#0 =1,it supports(TDD only) |
|  | frame configuration to support legacy(10MHz) |  | If Bit#0 =1,it supports(TDD only) |
|  | OFDMA system support |  | If Bit#0 =1,it supports |
|  | 3-step BR |  | If Bit#0 =1,it supports |
|  | dynamic ranging for HO |  | If Bit#0 =1,it supports |
|  | Persistent Allocation support |  | If Bit#0 =1,it supports |
|  | Group Resource Allocation support |  | If Bit#0 =1,it supports |
|  | colocated coexistence support |  | If Bit#0 =1,it supports |
|  | HO trigger metric |  | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |
|  | AMS Scanning Capability |  | This field indicates properties of the AMS that the ABS needs to know for scanning purposes: -Number of Center Frequencies AMS can scan during one scan iteration. -Maximum number of preambles per Center Frequency AMS can scan during one iteration. -Number of frames required to complete scanning of all preambles for a given frequency. The length of the field is 3 Bytes, which are divided as follows: •Bit 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame •Bit 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed. •0b000 = 0 sub-frames •0b111 = 7 sub-frames •Bits 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned. •0b000 = 0 sub-frames •0b111 = 7 sub-frames •Bits 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data. •0b000 = 0 sub-frames •0b111 = 7 sub-frames •Bit 14-15: Minimum supported scan duration •(0b000 = no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames) •Bit 16-23: Maximum supported scan duration •0x0 = no limit •All other values: maximum scan duration in number of sub-frames |
|  | ……. |  | …….. |
|  | Co-located coexistence capability support |  | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |
|  | Capabilities for interference mitigation support | DL PMI coordination capability | 0: AMS is not DL PMI coordination capable 1: AMS is not DL PMI coordination capable |
|  |  | DL collaborative multi-BS MIMO capability | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is not DL collaborative multi-BS MIMO capable |
| Convergence sublayer capabilities | ….. |  | ………… |
|  | ROHC support |  | Indicates the level of ROHC support. 0: not support 1: support |
|  | …….. |  | ……… |
| …. | ……. |  | ……….. |

The following parameters may be included:

AMS Scanning Capability:

This field indicates properties of the AMS that the ABS needs to know for scanning purposes:

-Number of Center Frequencies AMS can scan during one scan iteration.

-Maximum number of preambles per Center Frequency AMS can scan during one iteration.

-Number of frames required to complete scanning of all preambles for a given frequency.

The length of the field is 3 Bytes, which are divided as follows:

•Bit 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame

•Bit 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed.

•0b000 = 0 sub-frames

•0b111 = 7 sub-frames)

•Bits 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned.

•0b000 = 0 sub-frames

•0b111 = 7 sub-frames)

•Bits 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data.

•0b000 = 0 sub-frames

•0b111 = 7 sub-frames)

•Bit 14-15: Minimum supported scan duration

•(0b000 = no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames)

•Bit 16-23: Maximum supported scan duration

•(0x0 = no limit

•All other values: maximum scan duration in number of sub-frames))

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# 4Proposed Text#4

Modify the table 683, line 63, page78 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

The following parameters(table 683) may be included:

Table 683- parameters for AAI_REG-RSP

| Name |  |  | value |
| --- | --- | --- | --- |
|  | ……. |  | …….. |
|  | Capabilities for interference mitigation support | DL PMI coordination capability | 0: ABS is not DL PMI coordination capable 1: ABS is not DL PMI coordination capable |
|  |  | DL collaborative multi-BS MIMO capability | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is not DL collaborative multi-BS MIMO capable |
|  |  | DL closed-loop multi-BS macro diversity capability | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is not DL closed-loop multi-BS macro diversity capable |
|  |  | UL PMI combination capability | 0: ABS is not UL PMI combination capable 1: ABS is not UL PMI combination capable |
| AMS capability negotiation parameters | …….. |  | ….. |
|  | Co-located coexistence capability supported |  | Indicates if co-located coexistence is supported. If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |
|  | Capabilities for MIMO support |  | TBD |
|  | channel BW and Cyclic prefix |  | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |
|  | frame configuration to support legacy(5MHz) |  | If Bit#0 =1,it supports(TDD only) |
|  | frame configuration to support legacy(10MHz) |  | If Bit#0 =1,it supports(TDD only) |
|  | OFDMA system support |  | If Bit#0 =1,it supports |
|  | 3-step BR |  | If Bit#0 =1,it supports |
|  | dynamic ranging for HO |  | If Bit#0 =1,it supports |
|  | Persistent Allocation support |  | If Bit#0 =1,it supports |
|  | Group Resource Allocation support |  | If Bit#0 =1,it supports |
|  | HO trigger metric |  | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |
|  | AMS Scanning Capability |  | This field indicates properties of the AMS that the ABS needs to know for scanning purposes: -Number of Center Frequencies AMS can scan during one scan iteration. -Maximum number of preambles per Center Frequency AMS can scan during one iteration. -Number of frames required to complete scanning of all preambles for a given frequency. The length of the field is 3 Bytes, which are divided as follows: •Bit 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame •Bit 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed. •0b000 = 0 sub-frames •0b111 = 7 sub-frames •Bits 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned. •0b000 = 0 sub-frames •0b111 = 7 sub-frames •Bits 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data. •0b000 = 0 sub-frames •0b111 = 7 sub-frames •Bit 14-15: Minimum supported scan duration •(0b000 = no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames) •Bit 16-23: Maximum supported scan duration •0x0 = no limit •All other values: maximum scan duration in number of sub-frames |
| … | ……. |  | …….. |
| Convergence sublayer capabilities | ….. |  | ………… |
|  | ROHC support |  | Indicates the level of ROHC support. 0: not support 1: support |
|  | …….. |  | ……… |
| … | ……. |  | …….. |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# References

- IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.