| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Clean up for Sleep Operation Section in IEEE 802.16m (16.2.16) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yang Liu ZTE Elad Levy, Maruti Gupta, Aran Bergman, Shantidev Mohanty Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: |  |  |
| Abstract | This contribution proposes methods using which AMS in sleep mode can improved power saving in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Clean up for Sleep Operation Section in IEEE 802.16m (16.2.16)

Yang Liu

ZTE

Elad Levy, Maruti Gupta, Aran Bergman, Shantidev Mohanty

- Intel Corporation

- Introduction

There are several editorial as well minor technical inconsistencies in the sleep mode operation text in Section 16.2.16. This contribution proposes changes to different parts of this section to clean up the text in this section.

- Text proposal for inclusion in the P802.16m/D4

========================== Start of Proposed Text ==============================

[Editor’s Note: Incorporate following changes to Section 16.2.16].

- Editorial Change: page 298, Line 43: Change from “used” to “defined”: For each AMS, the AMS and ABS shall keep up to 16 previously used negotiated Sleep Cycle settings and associated SCIDs.

- Editorial Change: page 298, lines 46-48: If an AMS requests to switch the current Sleep Cycle setting to the previous  a pre-negotiated Sleep Cycle setting using the AAI_SLP-REQ including associated SCID, the ABS shall respond with the AAI_SLP-RSP including approval or other SCID.

- Editorial Change: page 299 line 23-

Proposed Text: “A Sleep Cycle, except for the first cycle, shall begin with a Listening Window”

- Editorial Change: page 300, line 3: Add NSCF as one of the parameters in list of Other parameters after TIMF flag.

- Editorial clarification: Page 300, line 11-17: “If the traffic indication message is positive for the AMS, then the length of the current Sleep Cycle shall be determined based on the value of Change Current Sleep Cycle flag (CCSCF) NSCF which was included in the AAI_SLP-RSP or SCH or SCEH.”

[Comment]: The flag is called Next Sleep Cycle flag, so does this apply for current sleep cycle in which positive TRF_IND has been received or next Sleep cycle? This needs clarification. Change flag name to Change Current Sleep Cycle flag (CCSCF)

- Clarification: page 300, lines 20-21: Add following: “In this case, after receiving positive TRF_IND, the sleep cycle will be reset to some other value as defined by NSCF parameter.

- Clarification required: Section 16.2.16.2.2 Sleep window operations: Line 18 on pg 298 and line 30-31 on pg 300 should be consistent. What can the MS not receive during Sleep window? It should be both DL unicast traffic and MAC control signaling both. Also what about multicast or broadcast messages?

- Clarification required: Section 16.2.16.2.3.1 Traffic Indication: Page 301, Lines 34-37:

Current Text: “If the AMS receives neither the traffic indication message nor any unicast data in the Listening Window, the AMS shall remain awake until it receives its own traffic indication using AAI_TRF_IND-REQ/RSP exchange. The AMS shall then send an AAI_TRF_IND-REQ message”

Proposed Text: “If the AMS receives neither the traffic indication message nor any Unicast   data in the Listening Window, then this can cause a loss of synchronization since the AMS cannot tell whether the traffic indication was positive or negative. To resolve this ambiguity, the MS waits until the end of Listening window and sends a Unicast AAI_TRF_IND_REQ message to the BS and receives AAI_TRF_IND_RSP message in response.”

9. The conditions for starting the T_AMS timer are not clear. Is it always started when default listen window starts or just when any DL/UL MAC PDUs are exchanged?

10. Section 16.2.16.2.5 CQI operation during Sleep Mode- can we specify explicitly that CQI is not sent during Sleep window?

- Page 305 line 44- Replace the word “phase” with “frame location”. Same for line 29.

- Page-298, Line-16

During the activation of Sleep Mode, the AMS is provided with a series of Sleep Cycles that typically consists of a Listening Window followed by a Sleep Window.

- Page-298, Line-18

During Sleep Window in of Sleep Mode, the ABS shall not transmit DL unicast MAC PDU to the AMS;, therefore the AMS may power down one or more physical operation components or perform other activities that do not require communication with the ABS.

- Page-298, Line-29

Upon wakeup from sleep state mode, if the AMS detects that it is not synchronized, then it shall stay awake until the AMS is successfully synchronized with the serving ABS.

- Page-298, Line-33

If the AMS detects that the system configuration information it has is not up-to-date, then it shall not transmit in the Listening Window until it receives the up-to-date system configuration information.

- Page-298, Line-46

If an AMS requests to switch Sleep Cycle setting to the previously negotiated kept Sleep Cycle setting by the AAI_SLP-REQ including associated SCID, the ABS shall respond with the AAI_SLP-RSP including approval or other SCID.

- Page-298, Line-49

Per AMS, a single Sleep Cycle setting is indicated by a Sleep Cycle ID (SCID) and it shall be applied across all the active connections of the AMS, and is indicated by the Sleep Cycle ID (SCID).

- Page-298, Line-55

Sleep Mode activation/entry may be initiated either by an AMS or an ABS. When an AMS in the Active mode decides to enter the Sleep Mode, it shall negotiate parameters of the Sleep Cycle setting with the ABS. ABS makes the final decision regarding the AMS request and instructs the AMS to enter Sleep Mode.

- Page-298, Line-64

The AMS may initiate the negotiation by sending an AAI_SLP-REQ message and shall expect an AAI_SLP-RSP message from the serving ABS in as response.

- Page-299, Line-5

In the event that the ABS-initiated request (i.e., Unsolicited Sleep responseAAI_SLP-RSP) and an AMS-initiated request for Sleep Mode entry is being handled concurrently, the ABS-initiated request shall take precedence over the AMS-initiated Request.

- Page-299, Line-19

The first sleep cycle on entry when entering to Sleep Mode from Active Mode contains only a sleep window which equals to the Initial Sleep Cycle.

- Page-299, Line-23

A Sleep Window shall follow the Listening Window and shall continue to the end of the current Sleep Cycle if the Listening Window does not occupy the full Sleep Cycle.

- Page-299, Line-36

The length of the Listening Window length within a Sleep Cycle may be dynamically extended, as specified in section 16.2.16.2.3.2.

- Page-300, Line-32

Handling of MAC control signaling during sleep mode is specified in 16.2.16.2.6.

- Page-300, Line-48

Listening window is measured in units of frames.

- Page-300, Line-58

on reaching the end of the Sleep Cycle

- Page-301, Line-40

On receiving the the traffic indication, the AMS shall behave in accordance with the traffic indication.

========================== End of Proposed Text ==============================