| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes for manual Femto ABSs selection(16.4.7.5) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | E-mail: sjjung@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31:Announcement |  |
| Abstract | Proposed text to modify the AAI_RNG-REQ message and Femto ABS reselection by AMS |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes for manual Femto ABSs selection (16.4.7.5)

Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# 1. Introduction

According to the section “16.4.7.3 Femto ABS Access Restrictions” of “IEEE P802.16m/D4” [1], an AMS may access or initiate handover to a CSG-Closed Femto ABS, which is not contained in the CSG whitelist, in case of manual Femto ABS selection. Upon reception of the AAI_RNG-REQ message, the Femto ABS shall determine whether the AMS performs manual Femto ABS selection or not. Therefore, the AAI_RNG-REQ message shall include indicator of manual selection. And manual selection may be considered as an exception when a Femto ABS determines the grant or rejection of access for the AMS.

We propose to add the following text for AAI_RNG-REQ messages and modify the text related to Femto ABS reselection by AMS.

# 2. Text Proposal

------------------------------------------------------------Start of the Text-------------------------------------------------------

[Add the text on page 699 line 42 as follows]

16.4.7.2. Manual Femto ABS selection

Manual femto ABS selection enables a human user to select a femto ABS and override automatic selection. In manual femto ABS selection, the AMS may scan neighbor femto ABSs accessible to the AMS and reports the list of accessible femto ABS to the user.

An AMS may attempt to access a femto ABS not contained in the CSG white list based on manual selection provided the access credentials can be obtained. Based on the result of the network entry at the femto ABS, the AMS’s CSG white list may be updated.

In manual Femto ABS selection, AMS should not include any CSGID in the AAI_RNG-REQ message.

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE P802.16m/D4, “Advanced Air Interface(Draft),” Feb. 2010.