| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the determination of paging carrier index (16.2.8.2.10.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | E-mail: sjjung@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31:Announcement |  |
| Abstract | Proposed text to modify the description of the paging carrier index |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the determination of paging carrier index (16.2.8.10.2)

Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# 1. Introduction

Paging carrier index in equation (2) indicates the index of carrier which the AMS has to monitor to receive the paging messages (i.e. AAI_PAG-ADV). But, the definition of paging carrier index is ambiguous. For idle mode AMS, carrier index can only be physical carrier index defined in AAI_Global-Config message. The paging carrier index derived from the equation (2) may not correspond to the physical carrier index supported by an ABS. For example, an ABS may support the carrier of which physical carrier index is (N+1) although the paging carrier index derived from equation (2) ranges from 0 to (N-1).

In addition, according to the section 16.2.8.2.10.2, the paging message for an AMS shall be transmitted in only one of the carriers. And an AMS determines the paging carrier index using the DID and N. The value of N can be the number of carrier per PGID or a set of PGIDs in an ABS that are used for transmitting paging message. And this can be all the carriers or a subset of carriers used by a multicarrier ABS. However, If the value of N is the number of carriers per PGID that are used for transmitting paging message, an AMS which is assigned multiple PGIDs may have to monitor multiple carriers for paging message. Or, if the value of N is the number of all the carriers used by a multicarrier ABS without check whether each carrier is used for transmitting paging message, the carrier corresponding to the determined paging carrier index by the AMS may be not used for transmitting paging message.

Therefore, the determination of paging carrier index is in need of clarification. We propose that the value of N in equation (2) can be the number of carriers in an ABS that are used for transmitting paging message. And we propose a mapping between paging carrier index and physical carrier index in an ABS.

# 2. Text Proposal

------------------------------------------------------------Start of the Text--------------------------------------------------------

[Modify and add the following texts on page 241 line 46]

The value of N is the number of carriers per PGID or a set of PGIDs in an ABS that are used for transmitting paging message for idle mode AMS(s). This can be all the carriers or a subset of carriers used by a multicarrier ABS. The paging carrier index corresponds to the ascending order of physical carrier index of the carriers which are used for transmitting paging message in the ABS. Paging carrier indication bit is used to specify if a carrier is a paging carrier or not.

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE P802.16m/D4, “Advanced Air Interface(Draft),” Feb. 2010.