| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the paging information message to support multicarrier operation (16.2.8.2.10.2) |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI Daniel Cohn Alvarion | E-mail: sjjung@etri.re.kr E-mail: Daniel.cohn@alvarion.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31:Announcement |  |
| Abstract | Proposed text to modify the PGID_Info message |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to paging information message to support multicarrier operation (16.2.8.2.10.2)

Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

Daniel Cohn

Alvarion

# 1. Introduction

According to the section 16.2.8.2.10.2, an idle mode AMS has to determine the carrier index for monitoring paging message in a multicarrier ABS. And the AMS uses DID and the value N to determine. Therefore, an idle AMS shall acquire the value of N at the preferred ABS. And paging carrier indication bit is used to specify if a carrier is a paging carrier or not. Paging carrier indication bits for available carriers in ABS are included in the PGID_Info message and may be included in the AAI_NBR-ADV message or the AAI_MC-ADV message.

However, there is no specific description about this information in the PGID_Info message, AAI_NBR-ADV message or the AAI_MC-ADV message in current D4.

Therefore we propose that an ABS shall provide the value of N using the paging indication bits in the PGID_Info message.

# 2. Text Proposal

------------------------------------------------------------Start of the Text--------------------------------------------------------

[Add the following rows to Table 768. PGID_Info Format]

Table 768 –PGID_Info Format

| Name | Value |
| --- | --- |
| Num_PGIDs | Number of Paging groups in the ABS |
| PGID | Identifier of paging group(0~216) |
| m | Time domain hash parameter(1~4) used to determine the frame number of a superframe for paging message transmission of an idle mode AMS. |
| Multicarrier indication | 0: single carrier ABS 1: multi-carrier ABS |
| Paging carrier indication bit(s) | This bitmap is included if multicarrier indication is set to 1 This bitmap is used to specify if carriers are a paging carrier or not. The size of paging carrier indication bitmap equals to the number of carriers in the AAI_Global_Config message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI_Global_Config message. The value N used to determine the paging carrier index is equal to the number of bits set to 1 and supported by AMS. |

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE P802.16m/D4, “Advanced Air Interface(Draft),” Feb. 2010.