| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for SFH SPs in IEEE 802.16m (16.3.6.5.1.2) |  |
| Date Submitted | 2009-03-05 |  |
| Source(s) | Xiangying Yang, Muthaiah Venkatachalam, Sassan Ahmadi, Hujun Yin, Aran Bergman, Hsuan Yi, Shantidev Mohanty Intel | Emails: Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.3.6.5.1.2” |  |
| Abstract | The contribution proposes the modifications to the DL Control Structure section to be included in 802.16m LB#31. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m L#31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for SFH SPs in IEEE 802.16m (16.3.6.5.1.2)

Xiangying Yang, Muthaiah Venkatachalam, Sassan Ahmadi, Hujun Yin,

Aran Bergman, Hsuan Yi, Shantidev Mohanty

Intel

- 1 Text Proposal for Change of S-SFH IE information fields

It is essential for an AMS to learn if an ABS is a marco ABS or a femto ABS before it can successfully decode the SFH. However, there is no mechanism in D4 to do this.

This contribution proposes methods using which an AMS can determine if an ABS is a marco ABS or a femto ABS so that it can successfully decode the SFH.

Proposed Text

-------------------------------  Proposed Text Start  ---------------------------------------------------

[Note to Editor: Insert the following field to Table 811 between lines 50 and 52 in “16.3.6.5.1.2 S-SFH IE” ]

Table 811: S-SFH SP1 IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ABS Type | 1 | Indicates whether the ABS is a macro ABS or Femto ABS. Value of “0” means ABS is a macro ABS. Value of “1” means ABS is a femto ABS. |

- - - - - - - - - - - - - - - - - - - - - - - - - - - End Proposed Text - - - - - - - - - - - - - - - - - - - - - - - - -