| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for the single-BS MIMO feedback MAC message (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Taeyoung Kim, Sangheon Kim, Jaeweon Cho Samsung Electronics | ty33.kim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4 section 16.2.3”. |  |
| Abstract | This contribution proposes modifications for the single-BS MIMO feedback MAC message AAI_SingleBS_MIMO_FBK |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications for the single-BS MIMO feedback MAC message (16.2.3)

Taeyoung Kim, Sangheon Kim, Jaeweon Cho

Samsung Electronics Co., Ltd.

# Introduction

This contribution proposes several changes related to the single-BS MIMO feedback MAC message, e.g., AAI_SingleBS_MIMO_FBK message.

- Add MFM 0, 1, 4 and 7

- References

[1] Draft Amendment, P802.16m/D4, Feb. 2010

# Text proposal for inclusion in the 802.16m amendment

<< Modify section “16.2.3.37 AAI_SingleBS_MIMO_FBK” on page 123 line 54 >>

-------------------------------  Start of Text Proposal ---------------------------------------------------

16.2.3.37 AAI_SingleBS_MIMO_FBK

The AAI_SingleBS_MIMO_FBK message format is defined in Table 705. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 2, 3, 5 or 6. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 8 transmit antennas. Variables MaxMt, Codebook_subset, Codebook_mode, Num_best_subbands, long period q and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. Best_subbands_index is encoded as described in section 15.3.9.3.1.4. The feedback information reported in the AAI_SingleBS_MIMO_FBK message depends on currently assigned feedback processes and reporting times.

Table 705 - AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type | 8 |  |
| If ( ((MFM == 3) and (q > 0)) or ((MFM == 6) and (q > 0)) or ((MFM == 4) and (q > 0)) or ((MFM == 7) and (q > 0))){ |  | MFM and long period q are indicated in Feedback Polling A-MAP IEs relevant to currently assigned feedback processes. |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 15.3.7.2.5.6 |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 15.3.7.2.5.6 |
| } |  |  |
| } |  |  |
| } |  |  |
| MFM_bitmap | 4 8 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling IE. LSB #0: MFM 20 LSB #1: MFM 31 LSB #2: MFM 52 LSB #3: MFM 63 LSB #4: MFM 4 LSB #5: MFM 5 LSB #6: MFM 6 LSB #7: MFM 7 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 as specified in Feedback Polling A-MAP IE |
| Wideband CQI |  |  |
| Wideband STC rate |  |  |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 as specified in Feedback Polling A-MAP IE |
| Wideband CQI |  |  |
| } |  |  |
| If (LSB #02 in MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| STC_Rate | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| } |  |  |
| } |  |  |
| If (LSB #13 in MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| STC_Rate | variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 |  |
| Wideband STC rate | variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits This field shall be omitted if MaxMt = 1. |
| If (q == 0) { |  |  |
| Wideband PMI | variable | Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #2#5 in MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Stream index | variable | Best stream index of m-th subband indicated by Best_subbands_index If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| } |  |  |
| If (LSB #3#6 in MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 |  |
| If (q == 0) { |  |  |
| Wideband PMI | variable | Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal ---------------------------------------------------