| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to DL MIMO control parameters (16.3.7.2.5.4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Taeyoung Kim Samsung Electronics | ty33.kim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4 section 16.3.7.2.5.4”. |  |
| Abstract | Modifications are proposed for DL MIMO control parameters |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to DL MIMO control parameters (16.3.7.2.5.4)

Taeyoung Kim

Samsung Electronics Co., Ltd.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “16.3.7.2.5.4 Downlink signaling support of DL-MIMO modes” on line 1 page 543 as follows>

-------------------------------  Start of Text Proposal ---------------------------------------------------

16.3.7.2.5.4 Downlink signaling support of DL-MIMO modes

The BS shall send some parameters necessary for DL MIMO operation in a broadcast message. The broadcast information is carried in the S-SFH SP3 IE or in the additional broadcast information such as AAI_SCD or AAI_DL_IM messages..

The BS shall send some parameters necessary for DL MIMO operation in a unicast message. The unicast information is carried in the DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE, Feedback Polling A-MAP IE, and Feedback Allocation A-MAP IE.

Table 850 - DL MIMO control parameters specifies the DL control parameters required for MIMO operation.

| Table 850 - DL MIMO control parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Description | Value | Notes |
| Broadcast Information |  |  |  |
| Nt | Number of transmit antennas at the BS | 2, 4, 8 | Indicated in S-SFH (system information) SP3 IE |
| OL_Region | OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of MIMO streams (1 or 2) | OL-Region-ON (1 bit): Signal the existence of OL region OL-Rank1-Config (3 bit): to signal the combination of sub-band and mini-band in OL region type 1. Refer to Error: Reference source not found. SB-OL-Region-2-Size (4 bit) : signal the number of sub-bands in OL region type 2. Refer to AAI_SCD message (16.2.3.29) | Indicated in AAI_SCD Message |
| BC_SI | Rank-1 base codebook subset indication for interference mitigation with PMI coordination | BitMAP: 8 bits if Nt = 2 16 bits if Nt = 4, 8 | Rank-1 codebook element restriction/recommendation information It shall be ignored if CM = 0b00, 0b01 or 0b10 Indicated in AAI_DL_IM Message Its usage is enabled by CCE or CM. |
| Unicast Information |  |  |  |
| MEF | MIMO encoder format | SFBC Vertical encoding Horizontal encoding | MIMO encoder format. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| Mt | Number of MIMO streams in transmission | 1 to 8 | Number of MIMO streams in the transmission. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| SI | Index of allocated pilot MIMO stream | 1 to 4 | SI shall be indicated if MEF is HE Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| MFM | MIMO feedback mode | Refer to Error: Reference source not found | To decide the feedback content and related MS processing Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| MaxMt | Maximum number of MIMO streams | If Nt=2 (any MFM): 0b0: 1 0b1: 2 If Nt=4 (any MFM): 0b00: 1 0b01: 2 0b10: 3 0b11: 4 If Nt=8 (SU-MIMO MFM 0, 1, 2, 3, 4): 0b00: 1 0b01: 2 0b10: 4 0b11: 8 If Nt=8 (MU-MIMO MFM 5, 6, 7): 0b00: 1 0b01: 2 0b10: 3 0b11: 4 | If MFM indicates a SU feedback mode: the maximum STCrate scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| CS indication Codebook_subset | Codebook subset type for CL MIMO modes 2 and 4 | Base codebook or codebook subset | Depending on the MFM and CS indicationCodebook_subset, the MS shall feedback a PMI from the SU or MU base codebook, or from a subset of the SU or MU of the base codebook. Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| CM Codebook_mode | Codebook feedback mode for CL MIMO modes 2 and 4 | 0b00: base mode with codebook coordination disabled 0b01: transformation mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: base mode with codebook coordination enabled | This field specifies the codebook feedback mode. If codebook coordination is enabled by setting Codebook_mode to 0b11 or by setting Codebook_coordination to 0b1, and if the AMS reports STC rate equal to 1, then the AMS shall find the rate-1 PMI from the codebook entries broadcasted in BC-SI in AAI_DL_IM Message. |

When CS indicationCodebook_subset indicates the use of a codebook subset and MFM indicates a CL SU MIMO mode (MFM = 3 and 4), the MS shall use the SU base codebook subset of Error: Reference source not found when Nt=4. When CS indicationCodebook_subset indicates the use of a codebook subset and MFM indicates a CL MU MIMO mode (MFM = 6 and 7), the MS shall use the MU base codebook subset of Error: Reference source not found when Nt =4.

-------------------------------  End of Text Proposal  ---------------------------------------------------