| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for S-SFH SPs Modification in IEEE 802.16m (15.3.6.5.1.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang Intel | Emails: Shantidev.mohanty@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.3.6.5.1.2” |  |
| Abstract | The contribution proposes the modifications to the DL Control Structure section to be included in 802.16m LB#31. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m L#31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for S-SFH SPs Modification in IEEE 802.16m (15.3.6.5.1.2)

Shantidev Mohanty, Muthaiah Venkatachalam, Xiangying Yang

Intel

- 1 Text Proposal for Change of S-SFH IE information fields

It is essential for an AMS to learn the IP subnet that it is residing in. However, current D4 does not provide any mechanisms using which an AMS can learn its IP subnet.

This contribution proposes methods for an AMS to discover its IP subnet.

In addition, this contribution also proposes other network operation enhancements by modifying the contents of S-SFH IEs.

Proposed Text

-------------------------------  Proposed Text Start  ---------------------------------------------------

[Note to Editor: Add the following parameter to table 813, S-SFH SP3 IE page 453]

Table 813: S-SFH SP3 IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| IP subnet prefix | TBD | Specifies the IP subnet that the BS broadcasting this SFH belongs to. |

-------------------------------  Proposed Text End ---------------------------------------------------