| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to UL Basic Assignment A-MAP IE to avoid UL SPID un-sync (16.3.6.5.2.4.2) |  |
| Date Submitted | 2010-03-12 |  |
| Source(s) | Sun Bo, Xu Jin, Ning Ding ZTE Corporation | E-mail: sun.bo1@zte.com.cn |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution proposes the texts related to UL Basic Assignment A-MAP IE. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to UL HARQ signaling (16.3.6.5.2.4.2)

Sun Bo, Xu Jin, Ning Ding

ZTE Corporation

- Introduction

As analyzed in C80216m-10/0178, there’s possibility that when AMS mis-detects UL Basic Assignment A-MAP IE for resource reallocation for a retransmission, the SPID recorded at ABS side and AMS side might be un-synchronized. Even we think SPID un-sync is a rare case, we propose to improve UL Basic Assignment A-MAP IE to provide a simple and proper way to avoid above SPID un-sync issue. In details, we propose to restore SPID indicator in UL Basic Assignment A-MAP IE for this purpose.

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] IEEE C802.16m-09/0178, “Proposed changes to UL HARQ signaling (16.2.14)”

- Text proposal for the 802.16m DRAFT amendment

=========================Proposed Text Start===============================

[Modify the reserved item in table822 “UL Basic Assignment A-MAP IE”, in section 16.3.6.5.2.4.2 UL basic assignment A-MAP IE, in ln.39/pg.470, as follows:]

Table 822- UL Basic Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A- MAP_IE() { |  |  |
| … | … | … |
| Reserved SPID | 2 | Reserved bits HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| } |  |  |

=============================Proposed Text End===============================