| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed texts to Update of S-SFH (16.2.23) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Mihyun Lee, Jiyun Seol, Hyunjeong Kang, Jaeweon Cho, Hokyu Choi Samsung Electronics Co., Ltd. Jinsoo Choi, Heejeong Cho, Kiseon Ryu, Jin Sam Kwak LG Electronics | mihyun.mac.lee@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4, section 16.2.23”. |  |
| Abstract | The contribution provides the clean up text for update of S-SFH |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed texts to Update of S-SFH (16.2.23)

Mihyun Lee, Jiyun Seol, Hyunjeong Kang, Jaeweon Cho, Hokyu Choi

Samsung Electronics Co., Ltd.

Jinsoo Choi, Heejeong Cho, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This contribution provides the clarification on the update of S-SFH.

Removed text

Added text

- References

[1] IEEE P802.16m/D4, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-02-03

- Proposed Text Changes

[Remedy#1: Adopt the following modification in page 336, line 14, section 16.2.23]

When the AMS learns the super-frame number where S-SFH contents has changed, it uses the "Rate of change of S-SFH Info" field in S-SFH SP3 to determine the super-frame number until which the contents of this S-SFH remain the same. For example, when "Rate of change of S-SFH Info" field in S-SFH SP3 specifies that the contents of the S-SFH IE remains same over 32 * the periodicity of S-SFH SP1 superframes.

The ABS shall indicate any changes of S-SFH IE at the specific super-frame number(SFN) that satisfies SFN modulo S-SFH change cycle = 0. During S-SFH change cycle, contents of each S-SFH SPx IE shall remain the same.

S-SFH change cycle = Rate of change of S-SFH Info ⅹ the periodicity of S-SFH SP1 IE / 20ms.

Where the “Rate of change of S-SFH Info” and “the periodicity of S-SFH SP1 IE” are indicated in S-SFH SP3 IE.

Each bit of the S-SFH SP change bitmap indicates the changing status of the corresponding S-SFH SP IE in association with the S-SFH change count. The bit # 0 (LSB), bit #1 and bit #2 (MSB) are mapped to S-SFH SP1 IE, S-SFH SP 2 IE and S-SFH SP3 IE, respectively. If any of the values (except MSBs of superframe number in S-SFH SP1 IE) of an S-SFH SP IE are changed, the bit corresponding to the changed S-SFH SP IE is set to 1 toggled.

The S-SFH applying offset The Start super-frame offset where new S-SFH information is used in the P-SFH IE is used to indicate which S-SFH change count shall be considered for applying the system parameters in the S-SFH SPx IEs. According to the S-SFH applying offset, if the S-SFH applying offset is set to ‘0’, the AMS shall use the system parameters of S-SFH SPx IEs associated with the current S-SFH change count. Otherwise, if the S-SFH applying offset is ‘1’, the AMS shall store the changed parameters of S-SFH SP IEs associated with the current S-SFH change count, but use the parameters associated with previous S-SFH change count(= (the current S-SFH change count -1) modulo 16). To guarantee enough time for AMSs to receive the changed S-SFH SPx IE(s), the ABS shall set the S-SFH applying offset to ‘1’ for a number of superframes, which is two times of the periodicity of the changed S-SFH SP IEs, after the S-SFH change count is incremented by 1. When multiple SPx IE are changed in the S-SFH change count, the periodicity of the SPx IE having the longer period is used to set the S-SFH applying offset. Rate of change of S-SFH information shall be set so that S-SFH SP3 can be sent at least 2 times in the S-SFH change cycle. All S-SFH SPx IE, whose contents are changed in the S-SFH change count, are applied at the same superframe.

According to the S-SFH change count, S-SFH SP change bitmap, and S-SFH applying offset Start super-frame offset where new S-SFH information, AMS knows if it is needed to decode S-SFH IE in the current superframe to update the system parameters broadcasted within the S-SFH SP IE, and the AMS may decide to disregard the S-SFH IE with the presumption of there being no change in S-SFH SP IE compared to that already stored in the AMS.

AMS shall compare values of each S-SFH change count in the last received P-SFH IE and the last stored P-SFH IE whenever it receives P-SFH IE.

a) If there is no difference of two S-SFH change counts, AMS may not decode S-SFH IE in the superframe.

b) Else if the difference of two S-SFH change counts is one, AMS shall update the S-SFH SP IE(s) whose bit in the S-SFH SP change bitmap is set to 1. Else, AMS shall check that change bits are toggled as the value of the difference.

1) If the number of toggled bits in the last received S-SFH SP change bitmap is the same as the difference, AMS shall update the S-SFH SP IE(s) whose bit of the S-SFH SP change bitmap is toggled. Additionally, AMS shall update SCD change count and S-SFH SP change bitmap.

2)Else, AMS shall update all S-SFH SP IEs, SCD change count and S-SFH SP change bitmap.

c) Else if the difference of two S-SFH change counts is greater than one, AMS shall update all S-SFH SP IEs.

In case of b and c, AMS shall save the S-SFH change count.

If the S-SFH SP IE indicated by S-SFH scheduling information corresponds to the changed S-SFH SP IE as indicated in the S-SFH change bitmap, the AMS shall decode the following S-SFH IE and store the parameters in association with the current S-SFH change count. The AMS shall update and store store and update all the changed S-SFH SP IEs associated with the increment of S-SFH change count at the scheduled superframe for each S-SFH SP IE.

[Remedy#2: Adopt the following modification in page 336, line 1, section 16.2.23]

Every superframe, the ABS transmits P-SFH IE containing the S-SFH Scheduling information bitmap, S-SFH change count, S-SFH SP change bitmap, and S-SFH applying offset Start super-frame offset where new S-SFH information is used. The S-SFH change count shall remain unchanged as long as all of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH SP IEs remain unchanged. The S-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH IEs changes. The changed S-SFH SP IE(s) shall be transmitted at the scheduled superframes of period corresponding to each S-SFH SP IE.

[Remedy#3: Adopt the following modification in page 446, Table 809, section 16.3.6.5.1.1]

| Table 809 - P-SFH IE format |  |  |
| --- | --- | --- |
| Syntax | Size (bit) | Notes |
| P-SFH IE format () { |  |  |
| …… |  |  |
| S-SFH applying offset Start super-frame offset where new S-SFH information is used | 2 1 | Indicate the associated S-SFH change count in this superfame 0b0: Use S-SFH SPx IEs associated with the current S-SFH change count 0b1: Use S-SFH SPx IEs associated with (the current S-SFH change count – 1) modulo 16 |
| …… |  |  |
| } |  |  |

S-SFH SP change bitmap

A bitmap indicating the S-SFH SPx IE changed in association with the current S-SFH change count. If any field of S-SFH SPx IE is changed (except MSB of Superframe Number in S-SFH SP1), the bit corresponding to the S-SFH SPx IE is set to 1.toggled

S-SFH applying offset

A one bit flag that indicates AMSs which S-SFH change count shall be considered for applying the system parameters in the S-SFH SPx IEs. If the S-SFH applying offset is set to ‘0’, the AMS shall use the system parameters of S-SFH SPx IEs associated with the current S-SFH change count. Otherwise, if the S-SFH applying offset is ‘1’, the AMS shall use the parameters associated with previous S-SFH change count(= (the current S-SFH change count -1) modulo 16).

[Remedy#4: Adopt the following modification in page 453, Table 813, section 16.3.6.5.1.2]

Table 813—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| … |  |  |
| Rate of change of S-SFH info | 4 | It is used to calculate mMinimum duration (S-SFH change cycle) over which the contents of the S-SFH does not change remain the same. 0b0000: 16 0b0001: 32 0b0010: 64 0b0001 ~ 0b1111: reserved |
| ...... |  |  |