| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to SFH SP1 (Table 811) and Femto ranging (16.4.7.4) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Ying Li, Mihyun Lee, Sudhir Ramakrishna, Jung Je Son, Youngbo Cho, Ji-Yun Seol, Zhouyue Pi Samsung Electronics | E-mail: yli2@sta.samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Working Group Letter Ballot #31 Section 16.4.7 Network entry |  |
| Abstract | This contribution suggests changes on the current text in 16.4.7. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to SFH SP1 (Table 811) and femto ranging (16.4.7.4)

Ying Li, Mihyun Lee, Sudhir Ramakrishna, Jung Je Son, Youngbo Cho, Ji-Yun Seol, Zhouyue Pi

Samsung Electronics

# Introduction

The femto initial ranging is made different from other type of the cells in D4. This makes the decoding of SFH of femto different from macro, and it makes an additional bit to indicate whether the cell is femto or not in SFH.

However, the use case of such femto initial ranging different other cells does not add much value to the system, comparing the cost of one bit always sent in SFH of every BS in 16m system.

Even if the femto initial ranging is different other cells, there is no need to always send 1 bit indication in SFH since the affected case will be only the initial network entry where AMS dose not detect macro. The reason is if the AMS can always try to perform initial network entry with macro after SA-preamble is detected where the macro and non-macro are differentiated by a hard partition on the SA-preamble. Again, the only affected case is the initial network entry where AMS does not detect macro, which is very rare case.

# Proposed Text Change

.

------------------------------------------------- Start of Proposed Text Change-------------------------------------------------

Delete line 24-43, on page 449, Table 811.

Make the following changes on line 54-56, page 699, in section 16.4.7.4.1

The indication in SFH of the The ranging channel for synchronized AMSs is used for initial ranging, handover ranging and periodic ranging of a femto cell keeps the same format as other cells. BW REQ is done in exactly the same manner as in a regular Macro cell.

----------------------------------------------- End of Proposed Text Change---------------------------------------------------