| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Activation Deadline for secondary carrier activation (16.2.8.2.11) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim LG Electronics Jia Lin Huawei Technologies | E-mail: eunjong.lee@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com, ronnyyoungho.kim@lge.com linjia@huawei.com, |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Activation Deadline for secondary carrier activation (16.2.8.2.11)

Eunjong Lee, Youngsoo Yuk, Kiseon Ryu and Ronny Yongho Kim

LG Electronics

Jia Lin

Huawei Technologies

- Introduction

In the case of secondary carrier activation, the time for deadline should be defined for AMS/ABS to determine whether the secondary carrier activation is succeeded or not.

- Discussions

In the current carrier management procedure, the AAI_CM-CMD message is used for the ABS to instruct the AMS to perform secondary carrier activation, and the AMS transmits AAI_CM-IND message to inform the ABS of the readiness of target carrier.

As shown in Table 750 of D4, the AAI_CM-IND message only contains ‘Action code’ as contents. This means that the action directed by ABS would be certainly completed if the AMS successfully receives the AAI_CM-CMD message. However, we need to consider a case that AMS unsuccessfully completes the action directed by ABS due to any reasons, e.g., H/W error. Both the ABS and AMS should have the criteria to determine the secondary carrier activation failure.

Therefore, we propose to define the ‘Activation Deadline’ field in the AAI_CM-CMD message AMS/ABS to determine whether the secondary carrier activation is succeeded or not. If the ABS does not receive the AAI_CM-IND message within the Activation Deadline, the ABS considers the secondary carrier activation as failed.

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

------------------------------------------------------------------Start of the Text------------------------------------------------------------------

[Insert the text at line 17 on page 242 as follows:]

16.2.8.2.11.1 Secondary Carrier Activation/De-activation

The activation or deactivation of secondary carrier(s) is decided by the ABS based on QoS requirement, load condition of carriers and other factors. The ABS activates and/or deactivates secondary carrier with the AAI_CM-CMD MAC control message. The ABS sends the AAI_CM-CMD MAC control message on the primary carrier and includes the following information:

•	Indication Type per DL/UL: Activation, Deactivation

•	List of Secondary Carriers: (referred by logical carrier index)

•	Ranging indicator for the activated carrier

•	Activation Deadline: The time for the AMS to confirm activation of secondary carrier by sending the AAI_CMD-IND message before its expiration.

The ABS sends the AAI_CM-CMD message with Polling set to 1 in MCEH and the AMS, upon receiving the AAI_CM-CMD message, transmits an AAI_MSG-ACK message or Message ACK EH to inform that the AAI_CM-CMD message has been successfully received. The AMS transmits the AAI_CM-IND MAC control message through the primary carrier, where this message confirms with the ABS that the AMS has successfully activated/deactivated the carriers listed in the AAI_CM-CMD message. In case of activation, the AAI_CM-IND message is sent by the AMS when DL/UL of the newly activated carrier is ready to be used to transport data traffic. If the AAI_CM-IND message is not received by the ABS within the Activation Deadline, the ABS considers the secondary carrier activation as failed. In this case, the ABS may send an AAI_CM-CMD message for the AMS to activate another secondary carrier.

[Insert the text at line 48 in Table 749 on page 245 as follows:]

16.2.8.2.11.4.1 Carrier Management Command (AAI_CM-CMD) MAC control message

Table 749—AAI_CM-CMD MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management Command Control Message Format(){ |  |  |
| Control message type | 8 | AAI_CM-CMD |
| Action code | 1 | 0: Secondary carrier management 1: Primary carrier change |
| If(Action code==0){ |  | This message is for secondary carrier activation and/or deactivation |
| Indication Type | 2 | For Activation and/or Deactivation Bit#0:0:No actoin, 1:Activation Bit#1: 0: No action, 1: Deactivation |
| If(Indication Type#0==1){ |  |  |
| Activation Deadline | 6 | LSB bits of Superframe number after the AAI_CM-CMD is sent for the AMS to confirm the activation of secondary carrier by sending the AAI_CM-IND message |
| Num of target carrier | 3 | The number of to be activated carrier(s) |
| For(i=0;i<Num of target carrier;i++){ |  |  |
| … |  |  |

------------------------------------------------------------------End of the Text---------------------------------------------------------------------