| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 Encoding for Multi-Carrier Messages (16.2.3) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Text proposal on the format of handover messages in ASN.1 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 Encoding for Multi-Carrier Messages (16.2.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the ASN.1 encoding formats for the following messages:

- AAI_MC-REQ

- AAI_MC-RSP

- AAI_Global-Config

The original parameters for these messages have been restructured to fit into the ASN.1 coding structure. The presented ASN.1 codes have been syntax checked using the open ANS.1 compiler (asn1c).

Proposed Text

Remedy 1:

In the P802.16m/D4, page 145, line 1, replace Table 733 to the follow table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 684 — AAI_MC-REQ Parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-MC-REQ |  |  |  |
| M | multiCarrierCapability | See MultiCarrierCapability | N/A |
| MultiCarrierCapability |  |  |  |
| M | candidateCombinaitonList | List of CandidateCombination. See CandidateCombination. | N/A |
| M | supportOfDataTxOnGuardSubcarrier | 0: not support; 1: support | N/A |
| CandidateCombination |  |  |  |
| M | physicalCarrierIndexList | List of carriers AMS can simultaneously support | N/A |

-- ASN1START

-- Multicarrier request MAC control message

AAI-MC-REQ ::= SEQUENCE {

multiCarrierCapability				MultiCarrierCapability

}

MultiCarrierCapability ::= SEQUENCE {

candidateCombinaitonList			SEQUENCE (SIZE(1..maxCandidateCombinations)) OF CandidateCombinaiton,

supportOfDataTxOnGuardSubcarrier	BOOLEAN

}

CandidateCombinaiton ::= SEQUENCE {

physicalCarrierIndexList			SEQUENCE (SIZE(1..maxCandidateAssignedCarriers)) OF PhysicalCarrierIndex

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 2:

In the P802.16m/D4, page 145, line 38, replace Table 734 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 685 — AAI_MC-RSP parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-MC-RSP |  |  |  |
| M | multiCarrierCapability | See MultiCarrierCapability | N/A |

-- ASN1START

-- Multicarrier response MAC control message

AAI-MC-RSP ::= SEQUENCE {

multiCarrierCapability				MultiCarrierCapability

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 3:

In the P802.16m/D4, page 146, line 10, replace Table 735 to the following table and ASN.1 encoding format:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 735 — AAI_Global-Config parameters

| M/O | Attributes / Array of attributes | Value / Note | Conditions |
| --- | --- | --- | --- |
| AAI-Global-Config |  |  |  |
| M | carrierGroupList | List of carrier group information. See CarrierGroup. | N/A |
| M | currentPhysicalCarrierIndex | The carrier that broadcasts this message; the physical carrier index refers to AAI_Global-Config message | N/A |
| CarrierGroup |  |  |  |
| M | networkWiseMCConfigIndex | Index associated to Table 779 | N/A |
| M | startFreqAssignmentIndex | Frequency Assignment Index of the first carrier in this carrier group | N/A |
| M | carrierList | List of carrier information. See Carrier | N/A |
| Carrier |  |  |  |
| M | physicalCarrierIndex | Index of the physical carrier | N/A |
| M | duplexingMode | TDD(0) or FDD(1) | N/A |

-- ASN1START

-- Multicarrier global configuration MAC control message

AAI-Global-Config ::= SEQUENCE {

carrierGroupList					SEQUENCE (SIZE(1..maxCarrierGroups)) OF CarrierGroup,

currentPhysicalCarrierIndex			PhysicalCarrierIndex

}

CarrierGroup ::= SEQUENCE {

networkWiseMCConfigIndex			INTEGER(0..31),

startFreqAssignmentIndex			INTEGER(0..31),

carrierList							SEQUENCE (SIZE(1..maxCarriers)) OF Carrier

}

Carrier ::= SEQUENCE {

physicalCarrierIndex				PhysicalCarrierIndex,

duplexingMode						ENUMERATED {tdd(0), fdd(1)}

}

-- ASN1STOP

------------------------------------------------------  Text End  ---------------------------------------------------------------

Appendix

A complete version of ASN.1 code for the proposed messages in this contribution is provided. The code has been successfully compiled using open ASN.1 compiler (asn1c).

MacControlMsg DEFINITIONS AUTOMATIC TAGS ::= BEGIN

-- MAC Control Messages

MAC-Control-Message ::= SEQUENCE {

message MAC-Control-Msg-Type

}

MAC-Control-Msg-Type ::= CHOICE {

aaiRngReq			AAI-RNG-REQ,

aaiRngRsp			AAI-RNG-RSP,

aaiRngAck			AAI-RNG-ACK,

aaiNbrAdv			AAI-NBR-ADV,

aaiHoInd			AAI-HO-IND,

aaiHoReq			AAI-HO-REQ,

aaiHoCmd			AAI-HO-CMD,

aaiMcReq			AAI-MC-REQ,

aaiMcRsp			AAI-MC-RSP,

aaiGlobalConfig		AAI-Global-Config

}

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

AKCount	::= 		INTEGER (0..7)

CMACTuple ::= 		BIT STRING (SIZE (92))

ABSID ::=			BIT STRING (SIZE (48))

AMSIDStar ::=		BIT STRING (SIZE (48))

MACVersion ::=		BIT STRING (SIZE (8))

StationID ::=   	BIT STRING (SIZE (12))

TemporarySTID ::=	BIT STRING (SIZE (12))

PhysicalCarrierIndex ::= INTEGER (0..63)

-------------------------------------------------------------------------------

------------------------ AAI-HO-IND ------------------------------------------

-------------------------------------------------------------------------------

-- HO indication MAC control message

AAI-HO-IND ::= SEQUENCE {

hoEvent			CHOICE {

targetABSSelection		TargetABSSelection,

targetABSUnreachable	TargetABSUnreachable,

servingABSDisconnected	NULL,

hoCancel				HandoverCancel

}

}

TargetABSSelection ::= SEQUENCE {

targetABSID				INTEGER (0..7),

physicalCarrierIndex	PhysicalCarrierIndex		OPTIONAL

}

TargetABSUnreachable ::= SEQUENCE {

targetABSID				INTEGER (0..7)				OPTIONAL,

physicalCarrierIndex	PhysicalCarrierIndex		OPTIONAL

}

HandoverCancel ::= SEQUENCE {

akCount					AKCount

}

-------------------------------------------------------------------------------

------------------------ AAI-HO-REQ ------------------------------------------

-------------------------------------------------------------------------------

-- HO request MAC control message

AAI-HO-REQ ::= SEQUENCE {

reportMetrix					BIT STRING {

cinr-Mean(0),

rssi-Mean(1),

relative-Delay(2)

},

neighborReportList				SEQUENCE (SIZE(1..maxNewABSIndices)) OF NeighborReport,

carrierPreassignmentIndicator	BOOLEAN							OPTIONAL

}

NeighborReport ::= SEQUENCE {

absIndex						INTEGER(0..255),

cinrMean						INTEGER(0..255)					OPTIONAL,

rssiMean						INTEGER(0..255)					OPTIONAL,

relativeDelay					INTEGER(0..255)					OPTIONAL

}

-------------------------------------------------------------------------------

------------------------ AAI-HO-CMD ------------------------------------------

-------------------------------------------------------------------------------

-- HO command MAC control message

AAI-HO-CMD ::= SEQUENCE {

mode		CHOICE {

hoCmd 		HandoverCommand,

zsCmd		ZoneSwitchCommand,

hoReject	NULL

}

}

HandoverCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

hoReentryInterleavingInterval	INTEGER(0..255)					OPTIONAL,

hoReentryIteration				INTEGER(0..255)					OPTIONAL,

disconnectTimeOffset			INTEGER(0..255),

resourceRetainTime				INTEGER(0..255),

targetBSList					SEQUENCE (SIZE(1..maxNewABSIndices)) OF TargetBSInfo,

preallocatedBasicCID			INTEGER(0..65535)				OPTIONAL

}

ZoneSwitchCommand ::= SEQUENCE {

hoReentryMode					BOOLEAN,

resourceRetainTime				INTEGER(0..255),

actionTime						INTEGER(0..3),

preambleIndex-LZone				INTEGER(0..113),

cdmaRngFlag						BOOLEAN

}

TargetBSInfo ::= SEQUENCE {

targetBSID						ABSID,

preambleIndex					INTEGER(0..255),

faIndex							INTEGER(0..255),

s-sfhChangeCount				INTEGER(0..255),

actionTime						INTEGER(0..3),

cdmaRngFlag						BOOLEAN,

dedicatedCDMARngCode			INTEGER(0..31)					OPTIONAL,

dedicatedRngOpp					DedicatedRngOpp					OPTIONAL,

hoProcessOptimization			BIT STRING (SIZE (4)),

seamlessHO						BOOLEAN,

rngInitiationDeadline			INTEGER(0..31),

serviceLevelPrediction			ServiceLevelPrediction,

physicalCarrierIndex			INTEGER(0..63)					OPTIONAL,

preassignedCarrierInfo			PreassignedCarrierInfo			OPTIONAL

}

DedicatedRngOpp ::= SEQUENCE {

rngOppIndex						INTEGER(0..7),

subframeIndex					INTEGER(0..7)

}

ServiceLevelPrediction ::= ENUMERATED {

noServiceAvailable(0),

partiallyAvailable(1),

fullyAvailable(2),

noPrediction(3) }

PreassignedCarrierInfo ::= SEQUENCE {

carrierStatusBitmap				BIT STRING (SIZE(maxPreassignedCarriers)),

physicalCarrierIndexList		SEQUENCE (SIZE(1..maxPreassignedCarriers)) OF PhysicalCarrierIndex

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-REQ ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging request MAC control message

AAI-RNG-REQ ::= SEQUENCE {

rngPurposeIndication		BIT STRING {

reentry-ho-idle(0),

location-update-dcr-ext(1),

emergency-call-setup(2),

location-update-embs(3),

location-update-dcr(4),

reentry-dcr(5),

reentry-coverage-loss(6),

reentry-legacy(7) }				OPTIONAL,

csgIDList					SEQUENCE (SIZE (1..maxCSGIDs)) OF CSGID					OPTIONAL,

rngInfo						CHOICE {

nwReentryInfo	NWReentryInfo,

zoneSwitchInfo	ZoneSwitchInfo,

directHOInfo	DirectHOInfo

}

}

CSGID ::= 						BIT STRING (SIZE (10))

NWReentryInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48))				OPTIONAL, -- HO reentry

crID						BIT STRING (SIZE (16))				OPTIONAL, -- reentry from CR or DCR

stationID					StationID							OPTIONAL, -- HO reentry

akCount						AKCount								OPTIONAL, -- reentry, locateion update, HO

amsIDStar					AMSIDStar							OPTIONAL,

macVersion					MACVersion							OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL, -- reentry from idle

pagingCycleChange			INTEGER (0..15)						OPTIONAL,

powerDownIndicator			BOOLEAN								OPTIONAL,

amsMobilityInfo				ENUMERATED {slow, medium, fast}		OPTIONAL,

cmacTuple					CMACTuple							OPTIONAL

}

PagingInfo ::= SEQUENCE {

pagingControllerID			BIT STRING (SIZE (48)),

deregistrationID			BIT STRING (SIZE (10)),

pagingGroupID				BIT STRING (SIZE (16)),

pagingCycle					INTEGER (0..15),

pagingOffset				INTEGER (0..15)

}

ZoneSwitchInfo ::= SEQUENCE {

nonceAMS					BIT STRING (SIZE (64)),

nonceABS					BIT STRING (SIZE (64))

}

DirectHOInfo ::= SEQUENCE {

servingBSID					BIT STRING (SIZE (48)),

legacyCID					INTEGER(0..4095),

legacyCMACKeyCount			INTEGER(0..65535),

legacyCMACTuple				BIT STRING (SIZE (104))

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-RSP ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging response MAC control message

AAI-RNG-RSP ::= SEQUENCE {

rangingStatus				RangingStatus,

stID						CHOICE {

temporarySTID	TemporarySTID,

stationID		StationID

}									OPTIONAL,

amsIDStar					AMSIDStar							OPTIONAL,

locationUpdateResponse		LocationUpdateResponse				OPTIONAL,

pagingInfo					PagingInfo							OPTIONAL,

hoProcessOptimization		BIT STRING (SIZE (4))				OPTIONAL,

nbrMeasurementReport		BOOLEAN								OPTIONAL,

zoneSwitchInfo				ZoneSwitchInfo						OPTIONAL,

redirectionInfoList			SEQUENCE (SIZE(1..maxRedirectInfos)) OF ABSInfo	OPTIONAL,

unsolicitedBWIndicator		BOOLEAN								OPTIONAL,

clcInfo						CLCInfo								OPTIONAL,

crID						BIT STRING (SIZE (16))				OPTIONAL,

rangingRequestBit			BOOLEAN								OPTIONAL

}

LocationUpdateResponse ::= ENUMERATED {

success,

failed,

success-DL-traffic-pending,

dcr-mode-initiation-allowed }

ABSInfo ::= SEQUENCE {

absID						ABSID,

carrierInfoList				SEQUENCE (SIZE(1..maxCarriers)) OF CarrierInfo

}

CarrierInfo ::= SEQUENCE {

saPreambleIndex				INTEGER(0..767),

aPreambleTxPower			INTEGER(0..255),

physicalCarrierIndex		PhysicalCarrierIndex,

macProtocolVersion			INTEGER(0..255)

}

CLCInfo ::= SEQUENCE {

infoType					ENUMERATED {

clc-limit,

clc-request,

clc-response,

clc-report }

--	response					CHOICE {

--								}

}

-------------------------------------------------------------------------------

------------------------ AAI-RNG-ACK ------------------------------------------

-------------------------------------------------------------------------------

-- Ranging acknowledgement MAC control message

AAI-RNG-ACK ::= SEQUENCE {

frameIdentifier				FrameIdentifier,

rangingAck					CHOICE {

unicastRngAck	UnicastRngAck,

broadcastRngAck	BroadcastRngAck

}

}

FrameIdentifier ::= SEQUENCE {

superFrameNumber			INTEGER (0..3),

frameIndex					INTEGER (0..3)

}

UnicastRngAck ::= SEQUENCE {

rangingStatus				RangingStatus,

adjustmentParameters		AdjustmentParameters		OPTIONAL

}

RangingStatus ::= ENUMERATED {	continue(0), abort(1), success(2) }

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment		INTEGER (0..1023)			OPTIONAL,

powerLevelAdjustment		INTEGER (0..7)				OPTIONAL,

frequencyOffsetAdjustment	INTEGER (0..255)			OPTIONAL

}

BroadcastRngAck ::=	SEQUENCE {

rngAckBitmap				BIT STRING,

detectedSlotsList			SEQUENCE OF DetectedSlot	OPTIONAL

}

DetectedSlot ::= SEQUENCE {

receivedCodesList			SEQUENCE (SIZE(1..maxReceivedCodes)) OF ReceivedCode

}

ReceivedCode ::= SEQUENCE {

codeIndex					INTEGER (0..127),

unicastRngAck				UnicastRngAck

}

-------------------------------------------------------------------------------

------------------------ AAI-NBR-ADV ------------------------------------------

-------------------------------------------------------------------------------

-- Neighbor advertisement MAC control message

AAI-NBR-ADV ::= SEQUENCE {

changeCount					INTEGER (0..7),

cellType					ENUMERATED {macro, micro, macro-hotzone, femto, relay},

totalNumberOfSegments		INTEGER (0..15),

segmentIndex				INTEGER (0..15),

startingABSIndex			INTEGER (0..255),

neighborInfoList			SEQUENCE (SIZE(1..maxNeighborBSs)) OF NeighborInfo

}

NeighborInfo ::= SEQUENCE {

absInfo						ABSInfo,

sfhEncodingFormat			CHOICE {

fullSubpktInfo	FullSubpktInfo,

deltaEncoding	DeltaEncoding,

noSFHIncluded	NULL }

}

FullSubpktInfo ::= SEQUENCE {

sfh-subpkt1					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt2					BIT STRING (SIZE (88))				OPTIONAL,

sfh-subpkt3					BIT STRING (SIZE (88))				OPTIONAL

}

DeltaEncoding ::= SEQUENCE {

deltaInfo					BIT STRING

}

-------------------------------------------------------------------------------

------------------------ AAI-MC-REQ -------------------------------------------

-------------------------------------------------------------------------------

-- Multicarrier request MAC control message

AAI-MC-REQ ::= SEQUENCE {

multiCarrierCapability				MultiCarrierCapability

}

MultiCarrierCapability ::= SEQUENCE {

candidateCombinaitonList			SEQUENCE (SIZE(1..maxCandidateCombinations)) OF CandidateCombinaiton,

supportOfDataTxOnGuardSubcarrier	BOOLEAN

}

CandidateCombinaiton ::= SEQUENCE {

physicalCarrierIndexList			SEQUENCE (SIZE(1..maxCandidateAssignedCarriers)) OF PhysicalCarrierIndex

}

-------------------------------------------------------------------------------

------------------------ AAI-MC-RSP -------------------------------------------

-------------------------------------------------------------------------------

-- Multicarrier response MAC control message

AAI-MC-RSP ::= SEQUENCE {

multiCarrierCapability				MultiCarrierCapability

}

-------------------------------------------------------------------------------

------------------------ AAI-Global-Config-------------------------------------

-------------------------------------------------------------------------------

-- Multicarrier global configuration MAC control message

AAI-Global-Config ::= SEQUENCE {

carrierGroupList					SEQUENCE (SIZE(1..maxCarrierGroups)) OF CarrierGroup,

currentPhysicalCarrierIndex			PhysicalCarrierIndex

}

CarrierGroup ::= SEQUENCE {

networkWiseMCConfigIndex			INTEGER(0..31),

startFreqAssignmentIndex			INTEGER(0..31),

carrierList							SEQUENCE (SIZE(1..maxCarriers)) OF Carrier

}

Carrier ::= SEQUENCE {

physicalCarrierIndex				PhysicalCarrierIndex,

duplexingMode						ENUMERATED {tdd(0), fdd(1)}

}

-------------------------------------------------------------------------------

--------------------  Multiplicity and type constraint definitions ------------

-------------------------------------------------------------------------------

maxNewABSIndices				INTEGER ::= 64

maxPreassignedCarriers			INTEGER ::= 64

maxCSGIDs						INTEGER ::= 64

maxRedirectInfos				INTEGER ::= 64

maxReceivedCodes				INTEGER ::= 32

maxNeighborBSs					INTEGER ::= 256

maxCandidateCombinations		INTEGER ::= 16

maxCandidateAssignedCarriers	INTEGER ::= 8

maxCarrierGroups				INTEGER ::= 16

maxCarriers						INTEGER ::= 32

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

END