| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on zero target ABS handling in HO (16.2.6) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Xiangying Yang Intel Kelvin Chou MediaTek Yung-han Chen, ITRI Inuk Jung LGE Jaehyuk Jang Samsung Stavros Tzavidas, Shahab Sayeedi Motorolla | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.6” |  |
| Abstract | This contribution proposes text for more accurate snooping operation |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on zero target ABS handling in HO (16.2.6)

Xiangying Yang 
Intel

Kelvin Chou

MediaTek

Yung-han Chen,

ITRI

Inuk Jung

LGE

Jaehyuk Jang

Samsung

Stavros Tzavidas, Shahab Sayeedi

Motorolla

- Introduction

There are two HO cases handling in D3

- all TBS are unreachable

- there is 0 target ABS in HO-CMD

Due to some text change accepted in D3, the two cases seem to be redundant by one sentence, and the first case was removed in D4. We suggest go back to previous D3 agreement with minor text change, instead of fixing all the problems currently in D4 incomplete text.

- Proposed Test

Modify section 16.2.6 Page 208 Line 64 as follows

--------------------------------text start-----------------------------------------

16.2.6.3.4 HO Execution

If all the target ABSs that are included in the AAI_HO-CMD message are unreachable, and if the AMS has a preferred target ABS not included in the AAI_HO-CMD message, the AMS informs the serving ABS of its preferred target ABS by sending AAI_HO-IND message with code 0b01 before the expiration of Disconnect Time, and the AMS performs network reentry at the new target ABS as indicated in the AAI_HO-IND message. The AMS shall also indicate the BSID of its old serving ABS and previous used STID to the new target ABS during network entry at the new target ABS. If the AMS has no preferred target ABS, it may perform HO cancellation as described in section 16.2.6.3.6.

If the AAI_HO-CMD message includes no target ABS, the AMS shall inform the serving ABS of it's preferred target ABS by sending the AAI_HO-IND message with HO Event Code 0b01 before expiration of Disconnect Time. If a serving ABS receives the AAI_HO-IND message with HO Event Code 0b01, it shall respond to the AMS by sending AAI_HO-CMD with a target ABS which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait until receiving the AAI_HO-CMD message before expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS as defined above. If the AAI_HO-CMD message includes no target ABS, the Disconnect Time defines the deadline by which the AMS shall respond by sending AAI_HO-IND to the serving ABS. If the AAI_HO-CMD message includes no target ABS and the AMS does not have preferred target ABS, AMS may perform HO cancellation as described in Section 16.2.6.3.6

If the AAI_HO-CMD message includes no target ABS, or if all the target ABSs that are included in the AAI_HO-CMD message are unreachable as defined above, the AMS shall inform the serving ABS of it's preferred target ABS by sending the AAI_HO-IND message with HO Event Code 0b01 before expiration of Disconnect Time. If a serving ABS receives the AAI_HO-IND message with HO Event Code 0b01, it shall respond to the AMS by sending AAI_HO-CMD with a target ABS which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait until receiving the AAI_HO-CMD message before expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS as defined above. If the AMS has no preferred target ABS, it may perform HO cancellation as described in section 16.2.6.3.6. If the AAI_HO-CMD message includes no target ABS, or all the target ABSs that are included in the AAI_HO-CMD message are unreachable, the Disconnect Time defines the deadline by which the AMS shall respond by sending AAI_HO-IND to the serving ABS.

--------------------------------text end-----------------------------------------