| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on zero target ABS handling in HO (16.2.6) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Xiangying Yang Intel Kelvin Chou MediaTek Yung-han Chen, ITRI Inuk Jung LGE Jaehyuk Jang Samsung Stavros Tzavidas, Shahab Sayeedi Motorola Chunmei Liu, Massoud Olfat Clearwire Daniel Cohn, Vladimir Yanover Alvarion | E-mail: xiangying.yang@intel.com shahab.sayeedi@motorola.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.6” |  |
| Abstract | This contribution proposes clarification and cleanup of handover rejection support. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on zero target ABS handling in HO (16.2.6)

Xiangying Yang 
Intel

Kelvin Chou

MediaTek

Yung-han Chen,

ITRI

Inuk Jung

LGE

Jaehyuk Jang

Samsung

Stavros Tzavidas, Shahab Sayeedi

Motorola

- Introduction

Remove duplicate references to Disconnect time

Limit handover negotiation to two handover rejects from AMS if AMS has a preferred target ABS

- Proposed Test

Modify section 16.2.6 Page 208 Line 64 as follows

--------------------------------text start-----------------------------------------

16.2.6.3.4 HO Execution

<:::>

If the AAI_HO-CMD message includes no target ABS, or if all the target ABSs that are included in the AAI_HO-CMD message are unreachable as defined above, the AMS shall inform the serving ABS of it's preferred target ABS by sending the AAI_HO-IND message with HO Event Code 0b01 before expiration of Disconnect Time. If a serving ABS receives the AAI_HO-IND message with HO Event Code 0b01, it shall respond to the AMS by sending AAI_HO-CMD with a target ABS which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait until receiving the AAI_HO-CMD message before expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS as defined above. If the AMS has no preferred target ABS, it may perform HO cancellation as described in section 16.2.6.3.6. If the AAI_HO-CMD message includes no target ABS, or all the target ABSs that are included in the AAI_HO-CMD message are unreachable, the Disconnect Time defines the deadline by which the AMS shall respond by sending AAI_HO-IND to the serving ABS.

If all target ABSs included in the AAI_HO-CMD message are unreachable (as defined in this section) or if the AAI_HO-CMD message includes no target ABS, and if the AMS has a preferred target ABS it shall inform the serving ABS of its preferred target ABS by sending the AAI_HO-IND message with HO Event Code 0b01 prior to expiration of Disconnect Time.

If a serving ABS receives the AAI_HO-IND message with HO Event Code 0b01, it shall respond to the AMS by sending the AAI_HO-CMD message with target ABSs which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait to receive the AAI_HO-CMD message until expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS (as defined in this section). If the AMS cannot maintain communication with the serving ABS, it may proceed to its preferred target ABS after sending the AAI_HO-IND message with its preferred target ABS to the serving ABS. If after receiving a second AAI_HO-CMD message none of the target ABSs proposed by the serving BS are reachable by the AMS, the AMS may proceed to its preferred target ABS. If the AMS has no preferred target ABS to include in the AAI_HO-IND message, it may perform HO cancellation as described in section 16.2.6.3.6.