| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed remedy for the comment #941(16.2.3.28) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Alper Yegin,Youngkyo Baek, Hyunjeong Kang, Samsung Electronics Phillip Barber Huawei | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.3.28” |  |
| Abstract | This contribution provides proposed remedy for L2 transfer message bypass firewalls problem |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed remedy for the comment #941(16.2.3.28)

Alper Yegin, Youngkyo Baek, Hyunjeong Kang

Samsung Electronics

Phillip Barber

Huawei

# Introduction

L2 transfer messages are likely to by pass L3 or above layer firewalls and gives some access control risk or security vulnerability. So L2 transfer messages should be for specifically identified purpose and, in case of vender specific usages, precaution to minimize those risks is required.

We suggest adding those precautions or warning description as the proposed text.

# Proposed Text

Modify the sentences, line 11, page 114 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

This container is also used for 16m messages that are not processed by the ABS or ARS, but are rather processed by network entities beyond the ABS or ARS.

AAI_L2_XFER messages are very likely to bypass any firewalls or other security protection that may be implemented on terminal devices incorporating the AMS, firewalls which mostly, if not exclusively, monitor protocols on L3&L4 and above layers. For this reason, use of the AAI_L2_XFER message poses a substantial security and access control risk to the terminal a) unless the AAI_L2_XFER message is strictly used for very-specific, identified purposes (e.g., SMS, etc.), by the designated applications only, and b) unless the vendor implementation takes specific precaution to mitigate and minimize these risks. Vendor implementation and mitigation techniques are outside the scope of this specification. Any other use of the AAI_L2_XFER message may have unintended security and access control risks not contemplated in this specification.

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# References

- IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.