| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonization Text Proposal on BR-ACK A-MAP IE |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Kelvin Chou MediaTek Inc. Heejeong Cho LG Electronics | Kelvin.Chou@mediatek.com heejeong.cho@lge.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Text proposal on the format of handover messages in ASN.1 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonization Text Proposal on BR-ACK A-MAP IE

Kelvin Chou

MediaTek Inc.

Heejeong Cho

LG Electronics

Proposed Text

Remedy 1:

In the P802.16m/D4, page 522, line 15, modify text and Table 838 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.3.6.5.2.4.12 BR ACK A-MAP IE

The BR Acknowledgement A-MAP IE indicates the decoding status of the BR opportunities in the previous UL frame. All the successfully received preamble sequences, if present, shall be included in an ascending order. BR opportunities are encoded in ascending order based on the number of the uplink subframe in which they are contained. In addition, the BR-ACK A-MAP IE also includes the allocation information for the fixed sized BR header (refer to 16.2.2.1.3.1). The UL resource and HFA shall be allocated to the preamble sequences whose grant indicator is ‘1’. The allocations shall be ordered based on the index of preamble sequences arranged in an ascending order. If UL resource for the BR header is not allocated through the BR-ACK A-MAP IE, CDMA Allocation A-MAP IE is used for the UL resource allocation. The maximum number of the HARQ retransmissions is set to the default value defined in 16.2.14.2.

The ABS shall not transmit in one sub-frame more than 1 BR ACK A-MAP IE.

| Table 838 — BR-ACK A-MAP IE* |  |  |
| --- | --- | --- |
| Syntax | Size (bits) | Notes |
| BR-ACK_A-MAP_ IE() { | — | — |
| A-MAP IE Type | 4 | BR-ACK A-MAP IE |
| BR-ACK Bitmap | N_BR_Opportunities | Each bit indicates the decoding status of BR preamble sequence in the corresponding BR opportunity. The bitmap size is the number of BR opportunities in a frame. 0b0: No BR preamble sequence is detected, 0b1: At least one preamble sequence is detected |
| MSB of resource start offset | 2 | 0b00, 0b01, 0b10: 2bit-MSB of the start offset of the resource allocation (LRU) 0b11: No grant exist in this BR ACK A-MAP IE. |
| if (MSB of resource start offset != 0b11) { | — | — |
| LSB of resource start offset | 5 | This field is the LSB of the start offset of the Resource allocation (LRU) for BR Header |
| HFA start offset | 63 | This field is start offset of HARQ Feedback Allocation. |
| Allocation size | 1 | Resource size in a subframe which is allocated to a BR preamble sequence index for each BR header 0b0: 1 LRU, 0b1: 2 LRUs |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource for BR header. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D, is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| } | — | — |
| for ( i = 0; i < N_BR_Opportunities ; i++) { | — | The i-th entry in this for-loop indicates the received preamble sequences in the BR opportunity indicates by the i-th 1-bit in BR-ACK Bitmap |
| if (BR-ACK Bitmap[i] == 1) { | — | — |
| Number of received preamble sequences (L) | 62 | The number of BR preamble sequence indices included in this ACK A-MAP IE. |
| for (j = 0; j < L; j++) { | — | — |
| Preamble sequence index | 5 | Preamble sequence index received in the BR opportunity |
| MSG decoding indicator | 1 | To indicate the decoding status of quick access message 0b0: MSG not decoded 0b1: MSG decoded relevant to Preamble sequence index |
| If (MSB of resource start offset != 0b11) && (MSG decoding indicator == 0b0) { | — | — |
| Grant indicator | 1 | To indicate whether grant of BR Header for the BR preamble sequence index is included or not If this bit is set, the UL resource is allocated with fixed size and MCS. |
| } | — | — |
| } | — | — |
| } | — | — |
| } | — | — |
| Reserved | variable | For byte alignment |
| } | — | — |

*A 16 bit CRC is generated based on the contents of the BR-ACK A-MAP IE. The CRC is masked by the reserved Station ID for BR-ACK A-MAP