| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Starting Position Determination for Downlink HARQ |  |
| Date Submitted | 2010-04-26 |  |
| Source(s) | Toru Oizumi Panasonic Corporation | E-mail: oizumi.toru@jp.panasonic.com |
| Re: | IEEE P802.16m/D5, section 16.3.11.4.1 |  |
| Abstract | This contribution proposes changes in the section 16.3.11.4.1 IR HARQ of IEEE P802.16m/D5 |  |
| Purpose | To be discussed and adopted for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Starting Position Determination for Downlink HARQ

Toru Oizumi

Panasonic Corporation

# 1. Introduction

This contribution proposes the changes of the starting position determination for downlink HARQ.

# 2. Current starting position scheme in IEEE P802.16m/D5

The starting position for downlink HARQ in IEEE P802.16m/D5[1] is defined below.

Table 964 - Starting position determination for downlink HARQ

| SPID | Starting position Pi,k |
| --- | --- |
| 0 | 0 |
| 1 | −NCTC,kmodNFBBuffer,k |
| 2 | NFBBuffer,k/2−NCTC,k/2modNFBBuffer,k |
| 3 | NFBBuffer,k−NCTC,k/2modNFBBuffer,k |

where NCTC,k is the number of coded bits that shall be transmitted for the kth FEC block, and NFB_Buffer,k is the buffer size for the kth FEC block

The starting position is reasonable in the following cases for example.

(a)  NCTC,k = NFB_Buffer,k/3, Code rate=1

(b)  NCTC,k = NFB_Buffer,k/2, Code rate=2/3

(c)  NCTC,k = NFB_Buffer,k/1.5, Code rate=1/2

Figure 1 - Current starting position scheme

However, if NCTC,k is reduced after 1st transmission, some part of data in Circular Buffer may not be transmitted in the current starting position scheme.

Figure 2 - Problem of current starting position scheme

# 2. Proposal of starting position scheme

Generally, retransmission of the different data from other (re)transmission(s) has better PER performance than that of the same data.  Therefore the number of overlapped and non-transmitted (gapped) data in Circular Buffer should be lower to get better PER performance.

After 1st transmission,  NCTC,k can be reduced, or the code rate of retransmission can be higher.,[1].

Now we assume the code rate higher than 1/2 in 1st transmission is rare case, since all of the systematic bits and some of the parity bits need to be transmitted to get reasonable PER performance under the real network environments.

Considering the background and our assumption, we propose the starting position for SPID=2 be changed to “NFB_Buffer,k/2” to avoid non-transmitted data when NCTC,k is reduced after 1st transmission.

Figure 3 - Proposal of starting position scheme

============================ Start of Proposed Text ==================================

Table 964 - Starting position determination for downlink HARQ

| SPID | Starting position Pi,k |
| --- | --- |
| 0 | 0 |
| 1 | −NCTC,kmodNFBBuffer,k |
| 2 | NFBBuffer,k/2modNFBBuffer,k |
| 3 | NFBBuffer,k−NCTC,k/2modNFBBuffer,k |

============================== End of Proposed Text ================================

Reference

[1] IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, April 2010.