| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Restore Deleted Language Describing AMS Actions During Random Access Bandwidth Requests (Section 16.2.11.1.1) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | Stavros Tzavidas Motorola Prateek Kapadia, Abhay Karandikar TICET, IIT Bombay | stavros.tzavidas@motorola.com |
| Re: | P802.16m, Letter Ballot 31a |  |
| Abstract | Changes that were made from D4 to D5 removed essential functionality from the description of the bandwidth request procedure. Specifically when an AMS has data to send that belongs to an access class that is less than the MIN access class that the ABS is advertising the AMS shall wait. But in D5 the text that describes what the AMS shall do once its access class is allowed has been unnecessarily removed. This causes unnecessary confusion and ambiguity. We propose to bring back the D4 text that described the proper actions that the AMS shall take once its access class is allowed. |  |
| Purpose | Adoption of proposed text by P802.16m TG. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Restore Deleted Language Describing AMS Actions During Random Access Bandwidth Requests (Section 16.2.11.1.1)

Stavros Tzavidas, Prateek Kapadia, Abhay Karandikar

Motorola, TICET (IIT Bombay)

# Introduction

Changes that were made from D4 to D5 removed essential functionality from the description of the bandwidth request procedure. Specifically when an AMS has data to send that belongs to an access class that is less than the MIN access class that the ABS is advertising the AMS shall wait. But in D5 the text that describes what the AMS shall do once its access class is allowed has been unnecessarily removed. This causes unnecessary confusion and ambiguity. We propose to bring back the D4 text that described the proper actions that the AMS shall take once its access class is allowed.

# Proposed Solution

We propose to restore the D4 text, which was precise, clear and was unnecessarily deleted.

# Proposed Text

[Change the text in section 16.2.11.1.1 « Contention-based random accesss bandwidth request », on page 321, line53, as indicated:]

16.2.11.1.1 Contention-based random access bandwidth request

The ABS may advertise a sequence of minimum access classes in the BR Channel Configuration MIN Access Class elements within the AAI_SCD for each frame in a superframe. This sequence of minimum access classes is maintained until another advertisement with the AAI_SCD. Based on the sequence of minimum access classes, the AMS can select the frame used for the contention-based random access in order to minimize collision. If no minimum access classes are advertised in the AAI_SCD that means that all access classes are allowed. An access class is assigned to a service flow via DSx MAC control messages during service flow establishment / modification. When an AMS has information to send and decides to use the contention-based random access bandwidth request, the AMS shall check if the information the AMS has to send is for an access class with priority higher than or equal to the minimum access class advertised by BR channel configuration in the AAI_SCD. If it is not (the minimum access classes are not sufficiently low such that the AMS access class is allowed), then the AMS shall wait until the BR channel configuration in the AAI_SCD advertises a sequence of minimum access classes, one of which is less than or equal to the access class of the data and the AMS. When the AMS access class is allowed, the AMS shall randomly select a backoff value within the backoff window specified by the connection priority. This random backoff value indicates the number of BR opportunities that the AMS shall defer before transmitting  a bandwidth request.